3GPP TSG-RAN WG4 Meeting #90	R4-1904486

# Xi'an, China, 8th – 12th April, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6430 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS36.133 for UE behaviour after measurement gap (Section 8.1.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - UE behavior after MG doesn’t considering MGTA |  |  |  |  |  |  |  |  |
| Summary of change: |  | - When MGTA is applied, it is up to UE implementation whether or not the UE is able to conduct transmission in slot occurring immediately after the subframe partially overlapped with measurement gap. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - UE behavior after MG doesn’t considering MGTA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS36.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------- Beginning of Change ------------------------------------------------------------

### 8.1.2	Requirements

#### 8.1.2.1	UE measurement capability

If the UE requires measurement gaps to identify and measure inter-frequency and/or inter-RAT cells and the UE does not support perServingCellMeasurementGap-r14 or is not configured with per serving cell measurement gaps, in order for the requirements in the following subsections to apply the E-UTRAN must provide a single measurement gap pattern with constant gap duration for concurrent monitoring of all frequency layers and RATs. If the UE requires measurement gaps to identify and measure inter-frequency and/or inter-RAT cells and the UE supports perServingCellMeasurementGap-r14 and is configured with per serving cell measurement gaps, in order for the requirements in the following subsections to apply the E-UTRAN must provide gap pattern(s) on at least each serving component carrier (per-CC) where the UE has indicated in the perCC-ListGapIndication IE that gaps are required. No gap pattern is required to be provided on the serving component carrier where UE has indicated in the the perCC-ListGapIndication IE that gaps are not required. The requirements apply if the gap on each serving cell is at least that which the UE has indicated with gapIndication in the perCC-ListGapIndication IE, and if the gapOffset, MGRP and MGL are the same for each serving component carrier. During the measurement gaps the UE:

During the measurement gaps the UE:

-	shall not transmit any data

-	is not expected to tune its receiver on any of the E-UTRAN carrier frequencies of PCell and any SCell.

-	is not expected to tune its receiver on any of the E-UTRAN carrier frequencies of PCell, PSCell, and SCell.

If the UE supporting dual connectivity is configured with PSCell, during the total interruption time as shown in Figure 8.1.2.1-1, the UE shall not transmit and receive any data in SCG.

In addition, for UE supporting E-UTRA-NR dual connectivity, if MG timing advance of 0.5ms is applied, the UE:

-	shall not transmit any data

-	is not expected to tune its receiver on any of the E-UTRAN carrier frequencies of PCell and any SCell.

-	is not expected to tune its receiver on any of the E-UTRAN carrier frequencies of PCell, PSCell, and SCell.

in subframes fully or partially overlapping with the measurement gaps on E-UTRAN serving cells. The total interruption time on E-UTRAN serving cells is (MGL+1) subframes.

When MG timing advance of 0.5ms is not applied, in the uplink subframe occurring immediately after the measurement gap,

-	if the following conditions are met then it is up to UE implementation whether or not the UE can transmit data:

-	all the serving cells belong to E-UTRAN TDD;

-	the measurement objects do not include any NR carrier frequency;

-	if the subframe occurring immediately before the measurement gap is an uplink subframe.

-	Otherwise the UE shall not transmit any data.

When MG timing advance of 0.5ms is applied, in the uplink subframe occurring immediately after the subframe partially overlapped with measurement gap,

-	it is up to UE implementation whether or not the UE can transmit data

In determining the above UE behaviour in the uplink subframe occurring immediately after the measurement gap or after the subframe partially overlapped with measurement gap, the UE shall treat a special subframe as an uplink subframe if the special subframe occurs immediately before the measurement gap.

----------------------------------------------------------- End of Change ----------------------------------------------------------------