3GPP TSG-RAN WG4 Meeting #90bis               	R4-1904504

Xi’an, 8-12 April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR section 8.1.2.2, 8.1.3.2 Scaling factor N for SSB and CSI-RS RLM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 1.4.2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NR RLM requirements are not complete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the discussion in our company paper R4-1903937, update N=1 factor CSI-RS based RLM similarly as proposed for CSI-RS based BFD: - N=1 with the following exceptions: - UE is not required to measure CSI-RS RLM measurements on a CSI-RS resource if the CSI-RS resource and a search space set associated with a CORESET in the same OFDM symbol(s) and the CSI-RS and DM-RS of PDCCH are not QCL type-D, or - UE is not required to measure CSI-RS RLM measurements on a CSI-RS resource if the UE is configured with a CSI-RS in PRBs overlapping those of the CORESET in the OFDM symbols occupied by the search space set(s) For SSB based RLM: - N=8 always Correct the parameter names in tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.2, 8.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### << Start of change 1>>

#### 8.1.2.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB [ms] period becomes worse than the threshold Qout_SSB within TEvaluate_out_SSB [ms] evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB [ms] period becomes better than the threshold Qin_SSB within TEvaluate_in_SSB [ms] evaluation period.

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 8.1.2.2-1 for FR1.

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 8.1.2.2-2 for FR2 with N=8.

For FR1,

-	P=1/(1 – TSSB/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

For FR2,

-	P=1/(1 – TSSB/TSMTCperiod), when RLM-RS is not overlapped with measurement gap and RLM-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is 3, when RLM-RS is not overlapped with measurement gap and RLM-RS is fully overlapped with SMTC period (TSSB = TSMTCperiod).

-	P is 1/(1- TSSB/MGRP - TSSB/TSMTCperiod), when RLM-RS is partially overlapped with measurement gap and RLM-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod

-	P is 1/(1- TSSB /MGRP)*3, when RLM-RS is partially overlapped with measurement gap and RLM-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP  and TSSB = 0.5*TSMTCperiod

-	P is 1/{1- TSSB /min (TSMTCperiod ,MGRP)}, when RLM-RS is partially overlapped with measurement gap and RLM-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap

-	P is 1/(1- TSSB /MGRP)*3, when RLM-RS is partially overlapped with measurement gap and RLM-RS is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

Longer evaluation period would be expected if the combination of RLM-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

Table 8.1.2.2-1: Evaluation period TEvaluate_out and TEvaluate_in for FR1

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | max(200,ceil(10*P)*TSSB) | max(100,ceil(5*P)*TSSB) |
| DRX cycle≤320 | max(200,ceil(15*P)*max(TDRX,TSSB)) | max(100,ceil(7.5*P)*max(TDRX,TSSB)) |
| DRX cycle>320 | ceil(10*P)*TDRX | ceil(5*P)*TDRX |
| NOTE: TSSB = ssb-periodicityServingCell is the periodicity of SSB configured for RLM. |  |  |

Table 8.1.2.2-2: Evaluation period TEvaluate_out and TEvaluate_in for FR2

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | max(200,ceil(10*P*N)*TSSB) | max(100,ceil(5*P*N)*TSSB) |
| DRX cycle≤320 | max(200,ceil(15*P*N)*max(TDRX,TSSB)) | max(100,ceil(7.5*P*N)*max(TDRX,TSSB)) |
| DRX cycle>320 | ceil(10*P*N)*TDRX | ceil(5*P*N)*TDRX |
| NOTE: TSSB = ssb-periodicityServingCell is the periodicity of SSB configured for RLM. |  |  |

#### << End of change 1>>

#### << Start of change 2>>

#### 8.1.3.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_CSI-RS [ms] period becomes worse than the threshold Qout_CSI-RS within TEvaluate_out_CSI-RS [ms] evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_CSI-RS [ms] period becomes better than the threshold Qin_CSI-RS within TEvaluate_in_CSI-RS [ms] evaluation period.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 8.1.3.2-1 for FR1.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 8.1.3.2-2 for FR2, where

-	N=1, with the following exceptions:

  - UE is not required to measure CSI-RS RLM measurements on a CSI-RS resource if the CSI-RS resource and a search space set associated with a CORESET in the same OFDM symbol(s) and the CSI-RS and DM-RS of PDCCH are not QCL type-D, or

  - UE is not required to measure CSI-RS RLM measurements on a CSI-RS resource if the UE is configured with a CSI-RS in PRBs overlapping those of the CORESET in the OFDM symbols occupied by the search space set(s)

For FR1,

-	P=1/(1 – TCSI-RS/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

For FR2,

-	P=1, when RLM-RS is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=1/(1 – TCSI-RS/MGRP) , when RLM-RS is partially overlapped with measurement gap and RLM-RS is not overlapped with SMTC occasion (TCSI-RS < MGRP)

-	P=1/(1 – TCSI-RS /TSMTCperiod), when RLM-RS is not overlapped with measurement gap and RLM-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P is 3, when RLM-RS is not overlapped with measurement gap and RLM-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P is 1/(1- TCSI-RS /MGRP - TCSI-RS /TSMTCperiod), when RLM-RS is partially overlapped with measurement gap and RLM-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TCSI-RS < 0.5*TSMTCperiod

-	P is 1/(1- TCSI-RS /MGRP)* 3, when RLM-RS is partially overlapped with measurement gap and RLM-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP  and TCSI-RS = 0.5*TSMTCperiod

-	P is 1/{1- TCSI-RS /min (TSMTCperiod ,MGRP)}, when RLM-RS is partially overlapped with measurement gap and RLM-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap

-	P is 1/(1- TCSI-RS /MGRP)* 3, when RLM-RS is partially overlapped with measurement gap and RLM-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

Note:	The overlap between CSI-RS RLM and SMTC means that CSI-RS based RLM is within the SMTC window duration.Longer evaluation period would be expected if the combination of RLM-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

The values of Mout and Min used in Table 8.1.3.2-1 and Table 8.1.3.2-2 are defined as:

-	Mout = 20 and Min = 10, if the CSI-RS resource configured for RLM is transmitted with Density =3.

Table 8.1.3.2-1: Evaluation period TEvaluate_out and TEvaluate_in for FR1

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | max(200, ceil(Mout×P)×TCSI-RS) | max(100, ceil(Min×P) × TCSI-RS) |
| DRX ≤ 320ms | max(200, ceil(1.5×Mout×P)× max(TDRX, TCSI-RS)) | max(100, ceil(1.5×Min×P)× max(TDRX, TCSI-RS)) |
| DRX > 320ms | ceil(Mout×P) × TDRX | ceil(Min×P) × TDRX |
| NOTE: TCSI-RS is the periodicity of CSI-RS resource configured for RLM. TDRX is the DRX cycle length. |  |  |

Table 8.1.3.2-2: Evaluation period TEvaluate_out and TEvaluate_in for FR2

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | max(200, ceil(Mout×P×N)×TCSI-RS) | max(100, ceil(Min×P×N) × TCSI-RS) |
| DRX ≤ 320ms | max(200, ceil(1.5×Mout×P×N)× max(TDRX, TCSI-RS)) | max(100, ceil(1.5×Min×P×N)× max(TDRX, TCSI-RS)) |
| DRX > 320ms | ceil(Mout×P×N) × TDRX | ceil(Min×P×N) × TDRX |
| NOTE: TCSI-RS is the periodicity of CSI-RS resource configured for RLM. TDRX is the DRX cycle length. |  |  |

#### << End of change 2>>