3GPP TSG-RAN WG4 Meeting #90 bis	R4-1904896

Xian, China, 8 April – 12 April 2019

Source:	Ericsson

Title:	TP for TR 37.716-11-11: DC_40A_n78A and DC_40C_n78A

Agenda item:			9.3.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-11-11 to include DC_40A_n78A and DC_40C_n78A as defined in WID [1],

# 2. Text Proposal

# ---Start of changes---

### 6.1.x	DC_40_n78

#### 6.1.x.1	Operating bands for DC

Table 6.1.x.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_40_n78 | 40 | n78 | No |

#### 6.1.x.2	Configuration for DC

Table 6.x.2.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_40A_n78A | DC_40A_n78A | 40A | n78A |
| DC_40C_n78A | DC_40C_n78A | CA_40C | n78A |

#### 6.1.x.3	Maximum output power for DC

Table 6.1.x.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_40A_n78A | 23 | +2/-3 |

#### 6.1.x.4	Spurious emission band UE co-existence for DC

Table 6.1.x.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_40_n78 | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

#### 6.1.x.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.x.5-1

Table 6.1.x.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2300 | 2400 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 4600 | 4800 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 6900 | 7200 | 9900 | 11400 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 9200 | 9600 | 13200 | 15200 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 11500 | 12000 | 16500 | 19000 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 13800 | 14400 | 19800 | 22800 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 16100 | 16800 | 23100 | 26600 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1500 | 900 | 5600 | 6200 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800 | 1500 | 4200 | 5300 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7900 | 8600 | 8900 | 10000 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3000 | 1800 | 11200 | 12400 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3100 | 3900 | 7500 | 9100 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10200 | 11000 | 12200 | 13800 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12900 | 10800 | 6300 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15500 | 17600 | 12500 | 13400 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6800 | 5100 | 600 | 700 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14500 | 16200 | 13500 | 14800 |

Based on Table 6.X.5-1, it can be seen that

- 2nd order harmonic may fall into Rx frequencies of band n79.

- 2nd order IMD products may fall into Rx frequencies of bands 8, 11, 21, 32, 45, 46, 47, 50, 51, 74, 75, and 76.

- 3rd order IMD products may fall into Rx frequencies of bands 5, 6, 8, 11, 18, 19, 20, 21, 26, 27, 28, 32, 44, 45, 46, 50, 51, 74, 75, 76, n77 and n79.

- 4th order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 7, 9, 10, 22, 23, 25, 30, 33, 34, 35, 36, 37, 38, 39, 41, 42, 43, 48, 49, 65, 66, 69, 70 and n77. Own Rx frequencies of band 40 and n78 do not need to be considered since these are two TDD bands.

- 5th order IMD products may fall into Rx frequencies of bands 46, 47 and 71.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.X.3-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

6.1.x.5-2: Band 40 and Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 4th IMD |
|  | 2400 | - | 2494 | Yes | Asia | 4th IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd, 3rd, 5th IMD |
|  | 5150 | - | 5350 | Yes | Europe | 3rd, 5th IMD |
|  | 5470 | - | 5725 | Yes |  | 2nd, 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 2nd, 3rd, 5th IMD |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

### 6.1.x.6	∆TIB and ∆RIB values

For DC_40_n78, the TIB,c and RIB,c values are reused from the LTE combinations CA_40-42 and CA_40-43, and are given in the tables below.

# Table 6.1.x.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_40_n78 | 40 | 04 |
|  | n78 | 0.54 |
| NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

# Table 6.1.x.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_40_n78 | 40 | 0.44 |
|  | n78 | 0.54 |
| NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 6.1.x.7	MSD

3rd order harmonic mixing at band 40 Rx would be caused by the leakage of 2nd harmonics of n78 Tx (which will happen when 2 x n78 Tx equals 3 x band 40 Rx). Therefore, MSD values are needed. The tentative values are reused from DC_41_n77 and are defined in the tables below.

Table 7.3B.2.3.2-1: Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 40 | n787 |  | 8.3 | 8.0 | 6.9 |  | 3.9 | 3 | 2.3 | 1.2 |  | 0.4 |
| n78 | 408 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  | N/A |
| NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| 40 | n78 | 15 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |
| n78 | 40 | 30 | N/A | 50 | 50 | 50 |  |  |  |  |  |  |  |

---End of changes---

# Reference

[1] 		RP-190705, “Revised WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band + 1 NR band)”, NTT DOCOMO, INC.