3GPP TSG RAN WG4 Meeting #90bis	R4-1904905

Xi'an, China, 8 - 12 April 2019

Agenda item:	9.4.2

Source:	SoftBank Corp.

Title:	TP for TR 37.716-21-11: EN-DC_1-11_n77

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_1-11_n77 was approved in RAN#83[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-11_n77 | CA_1-11 | n77 | DC_1_n77 |

Table 2: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-11A_n77A | DC_1A_n77A DC_11A_n77A | CA_1A-11A | n77A |

3.  Co-existence

When Uplink EN-DC configuration is DC_1A_n77A, (1) IMD2 of (B1 - n77) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n77A, (2) IMD2 of (B11 - n77) will fall into Rx band of Band 1.

Thus, it is proposed to evaluate (1) and (2).

[Proposal-1] It is proposed to evaluate (1) IMD2 of (B1 - n77) to B11 Rx and (2) IMD2 of (B11 - n77) to B1 Rx.

4.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows. EN-DC values are picked from [2].

Table 3: ΔTIB of constituent 2 band combo.

| ΔTIB | B1+B11 | B1+n77 | B11+n77 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0.3 | 0.6 |  | 0.6 |
| B11 | 0.3 |  | 0.4 | 0.4 |
| n77 |  | 0.8 | 0.8 | 0.8 |

Table 4: ΔRIB of constituent 2 band combo.

| ΔRIB | B1+B11 | B1+n77 | B11+n77 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0 | 0.2 |  | 0.2 |
| B11 | 0 |  | 0 | 0 |
| n77 |  | 0.5 | 0.5 | 0.5 |

Then the proposal for this 2LTE-1NR EN-DC is:

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n77 | 1 | 0.6 |
|  | 11 | 0.4 |
|  | n77 | 0.8 |

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n77 | 1 | 0.2 |
|  | 11 | 0 |
|  | n77 | 0.5 |

5.  REFSENS

As mentioned in section 3, REFSENS exceptions due to IMD2 are expected. Based on the evaluation in [3], the following values are proposed.

Table 5: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-11A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | FDD | N/A |  |
|  | n77 | 3441 | 10 | 50 | 3441 | N/A | TDD | N/A |  |
|  | 11 | 1438 | 5 | 25 | 1486 | 31.4 | FDD | IMD2 |  |
| DC_1A-11A_n77A | 11 | 1438 | 5 | 25 | 1486 | N/A | FDD | N/A |  |
|  | n77 | 3578 | 10 | 50 | 3578 | N/A | TDD | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 30.8 | FDD | IMD2 |  |

6.  Conclusion

This paper proposes study results of EN-DC 1-11_n77. As in proposa-1, REFSENS exception results for IMD2 are needed to complete the combo.

TP is shown in Annex.

7.  Reference

[1]  RP-190379  WID revision: EN-DC for 3 bands DL with 2 bands UL (2 LTE bands + 1 NR band); rapporteur: Huawei, Huawei, HiSilicon

[2]  TR37.863-01-01

[3]  R4-190xxxx	TP on MSD analysis results for LTE(2DL/1UL) + NR(1DL/1UL) DC UE, LG Electronics, Softbank

Annex.   Proposed text for TR37.716-21-11(v0.3.0): LTE 2B -NR 1B DC

[Unchanged Parts Skipped]

5.1.x	DC_1-11_n77

### 5.1.x.1	Operating bands for DC_1-11_n77

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-11_n77 | CA_1-11 | n77 | DC_1_n77 |

### 5.1.x.2	Configurations for DC_1-11_n77

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-11A_n77A | DC_1A_n77A DC_11A_n77A | CA_1A-11A | n77A |

### 5.1.x.3	Co-existence studies

### When Uplink EN-DC configuration is DC_1A_n77A, (1) IMD2 of (B1 - n77) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n77A, (2) IMD2 of (B11 - n77) will fall into Rx band of Band 1.

### 5.1.x.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.x.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n77 | 1 | 0.6 |
|  | 11 | 0.4 |
|  | n77 | 0.8 |

Table 5.1.x.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n77 | 1 | 0.2 |
|  | 11 | 0 |
|  | n77 | 0.5 |

### 5.1.x.5	REFSENS requirements

As mentioned above, IMD2 of B1 and n77 to Band 11 Rx and IMD2 of B11 and n77 to Band 1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-11A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | FDD | N/A |  |
|  | n77 | 3441 | 10 | 50 | 3441 | N/A | TDD | N/A |  |
|  | 11 | 1438 | 5 | 25 | 1486 | 31.4 | FDD | IMD2 |  |
| DC_1A-11A_n77A | 11 | 1438 | 5 | 25 | 1486 | N/A | FDD | N/A |  |
|  | n77 | 3578 | 10 | 50 | 3578 | N/A | TDD | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 30.8 | FDD | IMD2 |  |

[Unchanged Parts Skipped]