3GPP TSG RAN WG4 Meeting #91	R4-1905306

Reno, USA, May 13th – 17th, 2019

3GPP TSG RAN WG1 #96bis			R1-1905842

Xi’an, China, April 8th – April 12th, 2019

Title:	LS on clarification of OTA timing alignment for IAB

Release:	Rel-16

Work Item:	NR_IAB_Core

Source:	RAN WG1

To:	RAN WG4

Cc:	-

Contact Person:

#### Name:	Xinghua Song

E-mail Address:	songxinghua@huawei.com

- Overall Description:

In RAN1 #96, RAN1 sent an LS to RAN4 on OTA timing alignment for IAB in R1-1903810. RAN1 would like to have some further clarifications since there were some inconsistency between the agreement made in RAN1 #Ad Hoc1901 and R1-1903810.

Based on agreement in RAN1 #Ad Hoc 1901, if the OTA timing alignment applied, an IAB node should set its DL TX timing ahead of its DL Rx timing by TA/2 + T_delta, where the TA is the timing gap between UL Tx timing and DL Rx timing at the IAB node. Thus TA = (NTA + NTA_offset)xTc as defined in [38.133]. T_delta is signalled from the parent node and is intended to enable the IAB node MT to estimate the propagation delay (Tp) between itself and the parent. With this understanding, Tp = TA/2 + T_delta and T_delta = -Tg/2, where Tg is the switching gap between UL Rx and DL Tx at the parent node. However, in R1-1903810, it was assumed that TA = NTAxTc. In this case, Tp = TA/2 + T_delta and T_delta = -(Tg – NTA_offsetxTc )/2.

RAN1 would like to inform RAN4 that it is RAN1’s understanding that RAN1 agreement in #Ad Hoc 1901 holds hence TA = (NTA + NTA_offset)xTc .

2. Actions:

To RAN 4: RAN1 would like to inform RAN4 of the above clarification when the LS in R1-1903810 is discussed in RAN4.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-WG1 Meeting #97    13th – 17th May 2019	Reno, USA

TSG-WG1 Meeting #98    26th – 30th Aug 2019	Prague, Czech

Appendix

Figure 1:  Alignment of DL TX and UL RX timing at the parent node