3GPP TSG-RAN WG4 Meeting #91	R4-1905336

Reno, USA, 13th – 17th May 2019

Source: 	Nokia, Nokia Shanghai Bell

Title: 					n38 40 MHz A-MPR

Agenda Item: 			9.25.1

Document for:	Discussion

# 1	Introduction

In this contribution presents A-MPR results for n38 40 MHz channel bandwidth with additional spurious emission limit for band 7 protection as described in [1].

# 2	Discussion

## 2.1	Simulation scenario and assumptions

The simulation scenario

Additional spurious emission limits for protecting band 7 are described in [1] as -15.5 dBm per 5 MHz measurement bandwidth (MBW) in between 2620 MHz and 2645 MHz and -40 dBm per 1 MHz MBW in between 2645 MHz and 2690 MHz.

Simulation assumptions were as follows

IQ-Image and LO leakage = 28 dBc

CIM3 = 60 dBc

PA calibration point was 20 MHz, 15 kHz, QPSK, DFT-S-OFMA, 100 RB at lower channel edge with 0.5 dB MPR

SEM, ACLR, spurious emission and additional spurious emission limits described in [1] for A-MPR.

The simulated A-MPR is assumed applied as max(A-MPR, MPR).

40 MHz channel bandwidth located at the worst-case position (2600 MHz center frequency)

## 2.2	Simulation Results

Figure 3: A-MPR triangles for SCS = 15 kHz and QPSK modulation

Table I: Summary of A-MPR results

|  |  | A-MPR [dB] |  |
| --- | --- | --- | --- |
|  |  | Regrowth + IM | CIM |
| SC-FDMA | pi/2-BPSK | 8.5 | 1 |
|  | QPSK | 8.5 | 2 |
|  | 16QAM | 8.5 | 4 |
|  | 64QAM | 8.5 | 5 |
|  | 256QAM | 7 | 3.5 |
| OFDMA | QPSK | 9 | 7.5 |
|  | 16QAM | 9 | 7.5 |
|  | 64QAM | 9 | 8 |
|  | 256QAM | 8 | 7.5 |
| NOTE 1: Regrowth + IM region applies when LCRB ≥ -2*RRend + 384/(SCS/15 kHz) |  |  |  |
| NOTE 2: CIM region applies when LCRB ≤ 6/(SCS/15 kHz), RBend ≤ 24/(SCS/15 kHz) |  |  |  |

# 3	Conclusion

In this contribution we presented A-MPR results for n38 40 MHz channel bandwidth with additional spurious emission limit for band 7 protection as described in [1].

# 4	Reference

[1]	R4-193856 , “UE RF requirements on adding wider channel bandwidths in Band n38”, Huawei, HiSilicon.

# Appendix

A-MPR triangles