3GPP TSG-RAN WG4 Meeting #91	R4-1905337

Reno, USA, 13th – 17th May 2019

Source: 	Nokia

Title: 					n7 new channel BW REFSENS

Agenda Item: 			9.16.1

Document for:			Approval

# 1	Introduction

This contribution is a proposal how to define n7 REFSENS for the new wider bandwidths.

# 2	Discussion

In [1] it was recommended that n7 REFSENS definition is based on RB restriction or MSD option. In this contribution we propose to use mix of both methods.

- For channel bandwidths 25 – 40 MHz uplink allocation is placed on worst case position but allocation size is reduced from what is used for 20 MHz channel in order not to have to define MSD.

- For channel bandwidth of 50 MHz MSD is specified by using same uplink allocation RB amount as for 40 MHz channel.

With method described above we have some understanding on how REFSENS behaves when channel bandwidths get larger. In case uplink allocation is shifted as was proposed in one of the contributions there is no guidance on the impact.

Based on the results presented in [2] we propose to define the n7 REFSENS as follows.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) |  | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) |  | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.8 |  | -90.7 | -89.9 | -88.6 | -77.6 |  |  |  |  |  |
|  | 30 |  | -95.1 | -93.1 | -92.0 |  | -90.8 | -90.0 | -88.7 | -77.7 |  |  |  |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 |  | -91.0 | -90.1 | -88.9 | -77.8 |  |  |  |  |  |

Table 7.3.2-3: Uplink configuration for reference sensitivity (from 38.101-1)

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n7 | 15 | 25 | 501 | 751 | 751 | 721 | 641 | 451 | 451 |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 361 | 361 | 321 | 201 | 201 |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 181 | 181 | 161 | 101 | 101 |  |  |  |  |  |
| NOTE 1: 1 Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 3	Conclusion

In this contribution we have proposed how to define n7 REFSENS.

# 4	References

[1] R4-1903525, n7 new channel bandwidths REFSENS definition, RAN4#90bis, Nokia

[2] R4-1900067, n7 new channel bandwidths emissions study, RAN4#90, Nokia