3GPP TSG-RAN WG4 Meeting #91	R4-1905345

Reno, USA, 13th – 17th May 2019

Source: 	Nokia

Title: 					PC2 operation on bands 31 and 72: Emission simulations

Agenda Item: 			7.11.1

Document for:	Approval

# 1	Introduction

This contribution presents simulation results for PC2 operation on bands 31 and 72.

# 2	Discussions

As was mentioned in [1] UE to UE co-exitance and DTV band protection needs special attention and emission studies.

Figure 1: Frequency arrangement for 450 MHz bands and DTT band

UE to UE co-exitance

Band 31 uplink only needs to protect band 31 downlink and not band 72 downlink where are band 72 uplink needs to protect both band 31 and 72 downlinks. This means that the problematic case 31 UL--> 72 DL from duplex-filter point of view is not defined as a requirement.

Table 1: UE to UE co-ex

| 31 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 40, 42, 43, 65, 67, 68, 69 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 72 | E-UTRA Band 1, 7, 20, 22, 28, 31, 32, 33, 34, 38, 42, 43, 47, 52, 65, 68, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 8, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |

Hence we can just study the maximum emissions from 5 MHz distance from uplink band edge to determine the UE to UE co-ex requirement. From figure 1 we can see that emissions are -27.3 dBm at max when the channel is fully allocated which is the worst case. When we apply the minimum Tx to antenna attenuation of roughly 40 dB [2] we can see that – 50 dBm protection level is easily achieved.

Figure 1: Emissions at 5 MHz offset.

As a conclusion

Proposal 1: brackets can be removed from existing 36.101 relating to band 72 downlink protection from band 72 uplink and current PC3 UE to UE co-ex requirement can be applied to PC2.

DTV protection:

Digital television protection starts from 4701 MHz upwards and is -42dBm/8MHz. From Figure 2 we can see that emissions are at most – 45 dBm/1MHz which equals -36 dBm/8MHz. When we take into account filter rejection it is obvious that the requirements is met.

Observation: DTV protection is inherently met

Figure 2: Emissions at 470 MHz.

# 3	Conclusion

In this paper we have presented simulation results for PC2 operation for bands 31 and 72. We have made folliweing proposal

Proposal 1: brackets can be removed from existing 36.101 relating to band 72 downlink protection from band 72 uplink and current PC3 UE to UE co-ex requirement can be applied to PC2.

and one observation

Observation: DTV protection is inherently met

# 4	References

[1] R4-1903528, UE aspects for New WID: Power Class 2 UE for LTE bands 31 and 72, Nokia, RAN4#90bis

[2] https://en.rf360jv.com/inf/40/ds/is/B1220.pdf