3GPP TSG-RAN WG4 Meeting #91	R4-1905347

Reno, USA, 13th  – 17th May 2019

Source:	Nokia, AMX

Title:	TP for TR 36.716-04-01 for CA_3-5-7-28 and CA_3-3-5-7-28

Agenda item:	7.4.2

Document for:	Approval

# Background

This contribution provides text proposal for CA_3-5-7-28 and CA_3-3-5-7-28

as defined in the revised WID [1].

# Reference

[1]	RP-190138, “Revised WID on Rel-16 LTE inter-band CA for x bands DL (x=4, 5) with 1 band UL”, Nokia

# Text Proposal

************************* Start of changes *******************************************

## 5.X	CA_3-5-7-28 and CA_3-3-5-7-28

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.5A-2b: Inter-band CA operating bands (four bands)

| E-UTRA CA Band | E-UTRA Band (Table 5.5.1) |
| --- | --- |
| CA_3-5-7-28 | 3, 5, 7, 28 |
| CA_3-3-5-7-28 | 3, 5, 7, 28 |
| NOTE: The frequency range in band 28 is restricted for this CA band combination to 718-748 MHz for the UL and 773-803 MHz for the DL |  |

Table 5.x.1-1: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-5A-7A-28A | - | 3 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-5A-7C-28A | - | 3 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-3A-5A-7A-28A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 90 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.X.2		ΔTIB,c and ΔRIB,c values

ΔRIB,c values for CA_3-5-7-28 are derived from CA_3-5-7, CA_3-5-28, CA_3-7-28 and CA_5-7-28.

Table 5.x.2-1: ΔTIB,c (four bands)

| E-UTRA operating band combination | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3-5-7-28 CA_3-3-5-7-28 | 3 | 0.5 |
|  | 5 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.5 |

Table 5.x.2-2: ΔRIB,c (four bands)

| E-UTRA operating band combination | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3-5-7-28 CA_3-3-5-7-28 | 3 | 0 |
|  | 5 | 0.1 |
|  | 7 | 0 |
|  | 28 | 0.1 |

### 5.X.3	REFSENS requirements

REFSENS exceptions are not needed for CA_3-5-7-28 or CA_3-3-5-7-28.

************************* End of changes *******************************************