3GPP TSG-RAN WG4 Meeting #91	R4-1905387

Reno, USA, 13th – 17th May 2019

Title: 	Discussion on NR V2X UE RF requirements

Source: 	CATT

Agenda item:	8.4.3

Document for:	Discussion

- Introduction

In the last RAN4#90bis meeting, WI on 5G V2X with NR sidelink is established in [1]. A work plan of the WI was presented in [2] and a work plan for RF aspects was also agreed. It is expected that the principle of introducing NR V2X RF requirements can be studied and agreed.

In this contribution, we discuss related issues of introducing NR V2X RF requirements and give our understandings.

- Discussion

The RF requirements for E-UTRA V2X UE have been specified in TS36.101, the structure of which can be reused in TS38.101. new sub-clause with suffix of capital letter for V2X UE RF requirements will be added in current specification. In TS36.101, ‘A’ is used for CA, ‘B’ is used for UL-MIMO, ‘C’ is used for DC, ‘D’ is used for D2D, ‘E’ is used for MTC, ‘F’ is used for NB-IoT, and ‘G’ is used for V2X. In TS38.101-1, ‘A’ is used for CA, ‘B’ is used for DC, ‘C’ is used for SUL, and ‘D’ is used for UL-MIMO, not aligning with TS36.101. When it comes to TS38.101,the suffix ‘E’ can be used for 5G V2X correspondingly.

Proposal 1: New sub-clause with suffix ‘E’ can be used for RF requirements of5G V2X in TS38.101-1/2.

The RF requirements of 5G V2X will be defined and some considerations arelisted as shown in the following table:

| Sub-clause | Title | Considerations |
| --- | --- | --- |
| 5.2E | Operating bands for V2X Communication | The operating bands will be added based on the proposals of 5GAA and operators. |
| 5.3E | Channel bandwidth for V2X Communication | The channel bandwidth for V2X communication will be added based on the proposal of 5GAA and operators. |
| 5.4E | Channel arrangement for V2X Communication | The sub-clause may not be needed, since there is no corresponding part in TS36.101.General contents can beused for V2X. |
| 5.5E | Configurations for V2X Communication | Multi-carrier or CA operation can be introduced in this sub-clause if required. |
| 6.2E | UE maximum output power for V2X Communication | The requirement for 23dBm and 33dBm will be introduced for 5G V2X UE. The requirements of LTE V2X 23dBm can be used as a reference for 5G V2X UE, especially on the 5.9GHz frequency band. This sub-clause can be divided into several aspects as follows： 6.2E.1 UE maximum output power 6.2E.2 UE maximum output power reduction 6.2E.3 Configured transmitted power |
| 6.3E | Output power dynamics for V2X Communication | The requirement of LTE V2X 23dBm can be used as a reference for 5G V2X UE. This sub-clause canbe divided as follows： 6.3E.1 UE Minimum output power 6.3E.2 Transmit OFF power 6.3E.3 ON/OFF time mask 6.3E.4 Power Control |
| 6.4E | Transmit signal quality for V2X Communication | This sub-clause canbe divided as follows： 6.4E.1 Frequency error 6.3E.2 Transmit modulation quality The frequency error and EVM Requirements ofLTE V2X may be reused for 5G V2X on FR1. Whether more tightened requirements of IBE are needed or not should be further discussed. |
| 6.5E | Output RF spectrum emissions for V2X Communication | This sub-clause can be divided as follows： 6.5E.1 Occupied bandwidth 6.5E.2 Out of band emission 6.5E.3 Spurious emission 6.5E.4 Transmit intermodulation The definition of occupied bandwidth for LTE V2X can be reused on FR1, even the bandwidth of 5G V2X may not be same as that of LTE V2X. Out of band emission requirements canbe defined based on the results in co-existencestudies. .New SEM and ACLR requirements will be specified. Spurious emission requirements should be reused for LTE V2X UE. Transmit intermodulation requirement may be reused on FR1. |
| 7.3E | Reference sensitivity for V2X Communication | The reference sensitivity and reference measurement channel canbe defined based on new bandwidth and physical channel designed for 5G V2X UE. |
| 7.4E | Maximum input level for V2X Communication | The requirement of LTE V2X UE may be reused for 5G V2X UE on FR1. |
| 7.5E | Adjacent Channel Selectivity (ACS) for V2X Communication | The ACS requirement of 5GV2X UE canbe defined based on the results in co-existence studies. |
| 7.6E | Blocking characteristics for V2X Communication | In TS36.101, thers is no corresponding sub-clause for LTE V2X , and general requirements are applied. The sameway may be used for 5G V2X UE. |
| 7.7E | Spurious response for V2X Communication | The requirements of LTE V2X can be used as a reference for that of 5G V2X UE. |
| 7.8E | Intermodulation characteristics for V2X Communication | Same requirements of LTE V2X may be reused for 5G V2X UE on FR1. |
| 7.9E | Spurious emissions for V2X Communication | This sub-clause may not be needed. General requirements should be reused. |
| 7.10E | Receiver image for V2X Communication | In TS38.101, there are no receiver image requirement for 5G V2X UE, while the requirement of LTE V2X UE exists in TS36.101. The requirement of 5G V2X UE should be discussed further. |

- Conclusion

The document discusses the specification structure and presents our initial considerations of introducing RF requirements for 5G V2X UE. Based on the analysis in this contribution, we give the following proposal:

Proposal 1: New sub-clause with suffix ‘E’ can be used for RF requirements of 5G V2X in TS38.101-1/2.

- Reference

[1]	RP-190766, New WID on 5G V2X with NR sidelink, LG Electronics, Huawei

[2]	R4-1905084, Work plan for NR V2X services in Rel-16, LG Electronics