3GPP TSG-RAN WG4 Meeting #91                              R4-1905404

# Reno, US, 13th– 17th May, 2019

Title:            Proposal on PC2 inter-band EN-DC (TDD+TDD) UE behaviour

Source: 	CMCC

Agenda Item:	9.20.3

Document for:	Approval

1    Introduction

In RAN4#90bis meeting, RAN4 discussed the SAR requirements for PC2 EN-DC (1 LTE TDD band + 1 NR TDD band), and A WF was agreed. In this contribution, we propose to specify HPUE behaviour for EN-DC (TDD+TDD) based on the conclusion of WF on PC2 EN-DC (1 LTE TDD band +1 NR TDD band) [1].

2    Discussion

As stated in WF on PC2 EN-DC (1 LTE TDD band +1 NR TDD band), the following agreements for PC2 EN-DC combination (LTE TDD PC3+NR TDD PC3) was reached at the last RAN4 meeting:

- As network based solution, at least the following is proposed.

  - Introducing a new signaling which indicates that the NR maxUplinkDutyCycle capability corresponds to LTE UL/DL configuration and it is a per band combination capability

  - The NR maxUplinkDutyCycle capability for PC2 inter-band EN-DC is optional.

    - The UE behaviour is FFS if NR maxUplinkDutyCycle capability for PC2 inter-band EN-DC is absent

  - It is assumed that the UE follows the UL grant regardless of duty-cycle

  - The values range of NR maxUplinkDutyCycle for PC2 inter-band EN-DC are from [20]% to 100% and the default value for NR uplink duty cycle can be specified according to different TDD configurations as following

- UE based solution is FFS. Also solution to combine network and UE based solution will be FFS

Based on the agreements, the NR maxUplinkDutyCycle capability for PC2 inter-band EN-DC is optional and the UE behaviour is FFS if NR maxUplinkDutyCycle capability for PC2 inter-band EN-DC is absent. It is necessary for RAN4 to choose the appropriate UE behaviour to reduce the risk of exceeding SAR limits when UE does not support this new signaling for PC2 inter-band EN-DC.

Proposal 1: It is proposed to specify UE behaviour for PC2 inter-band EN-DC (LTE TDD PC3+ NR TDD PC3) as follows:

If UE supports a different power class than the default UE power class for EN-DC band combination, and the supported power class enables higher maximum output power than that of the default power class:

- if the field of UE capability maxUplinkDutyCycle-EN-DC is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than default value is defined in Table 1 (The exact evaluation period is no less than one radio frame); or

- if the field of UE capability maxUplinkDutyCycle-EN-DC is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-EN-DC as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

-  if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower:

-  shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in sub-clause 6.2B.4;

-  else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-EN-DC as defined in TS 38.331; or

-  if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to default value is defined in Table 1 when maxUplinkDutyCycle-EN-DC is absent. (The exact evaluation period is no less than one radio frame):

-  shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

Table 1 Default value for maxUplinkDutyCycle-EN-DC

| LTE UL/DL Configuration | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Default value for maxUplinkDutyCycle-EN-DC | [40]% | [60]% | [80]% | [70]% | [80]% | [90]% | [50]% |

According to WID [2], release independent issue is to be considered for PC2 EN-DC (LTE TDD PC3+ NRTDD PC3). Considering the following reasons, it is proposed that PC2 inter-band EN-DC (LTE TDD PC3 + NR TDD PC3) can be supported from Rel-15 in release independence manner based on following reasons:

- The maxUplinkDutyCycle-EN-DC capability for PC2 inter-band EN-DC (TDD+TDD) is optional.

- For UEs that do not support maxUplinkDutyCycle-EN-DC capability, the PC2 inter-band EN-DC (TDD+TDD) can be release independent to Rel-15 by meeting the default value for maxUplinkDutyCycle-EN-DC corresponds to LTE UL/DL configuration.

- The UE behaviour can be defined as proposal 1 if NR maxUplinkDutyCycle-EN-DC capability for PC2 inter-band EN-DC is absent.

Proposal 2:  PC2 inter-band EN-DC (LTE TDD PC3 + NR TDD PC3) can be supported from Rel-15 in release independence manner.

3   Conclusions

In this contribution, we get the following proposals:

Proposal 1: It is proposed to specify UE behaviour for PC2 inter-band EN-DC (LTE TDD PC3+ NR TDD PC3) as follows:

If UE supports a different power class than the default UE power class for EN-DC band combination, and the supported power class enables higher maximum output power than that of the default power class:

- if the field of UE capability maxUplinkDutyCycle-EN-DC is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than default value is defined in Table 1 (The exact evaluation period is no less than one radio frame); or

- if the field of UE capability maxUplinkDutyCycle-EN-DC is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-EN-DC as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

-  if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower:

-  shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in sub-clause 6.2B.4;

-  else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-EN-DC as defined in TS 38.331; or

-  if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to default value is defined in Table 1 when maxUplinkDutyCycle-EN-DC is absent. (The exact evaluation period is no less than one radio frame):

-  shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

Table 1 Default value for maxUplinkDutyCycle-EN-DC

| LTE UL/DL Configuration | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Default value for maxUplinkDutyCycle-EN-DC | [40]% | [60]% | [80]% | [70]% | [80]% | [90]% | [50]% |

Proposal 2:  PC2 inter-band EN-DC (LTE TDD PC3 + NR TDD PC3) can be supported from Rel-15 in release independence manner.

4   References

- [1]  R4-1905001 WF on PC2 EN-DC (1 LTE TDD band + 1 NR TDD band). CMCC, vivo, OPPO, CATT

- [2]  RP-190315  Revised WID on Power Class 2 UE for EN-DC (1 LTE band +1 NR band).CMCC