3GPP TSG-RAN WG4 Meeting #91                                                                R4-1905413

Reno, US, 13th-17th May, 2019

Source:	Samsung, Verizon

Title:	TP for TR 38.716-01-01: CA_n48B and CA_n48C

Agenda item:			9.1.2

Document for:	Approval

# 1. Introduction

In RAN#83 meeting, the revised WID on Rel-16 NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y)  was approved [1], in which the latest NR intra band Carrier Aggregation was attached.  This contribution is a text proposal for TR 38.716-01-01 to include CA_n48B and CA_n48C.

2. Reference

- RP-190689, Revised WID on Rel-16 NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y), Ericsson .

# 3. Text Proposal

<Start of Text Proposal>

1	Scope

The present document is a technical report for NR Intra-band Carrier Aggregation Rel-16 for xDL/yUL including contiguous and non-contiguous spectrum under Rel-16 time frame. The purpose is to gather the relevant background information and studies in order to address NR Intra-band Carrier Aggregation requirements for the Rel-16 band combinations in Table 1-1, Table 1-2, Table 1-3 and Table 1-4.

Table 1-1: Release 15 NR Intra-band carrier contiguous aggregation combinations FR1

| CA combination | REL-indep. from |
| --- | --- |
| DL_n78C_UL_n78C | Rel-15 |
| DL_n79C_UL_n79C | Rel-15 |
| DL_n66B_UL_n66A | Rel-15 |
| DL_n41C_UL_n41A | Rel-15 |
| DL_n41C_UL_n41C | Rel-15 |
| DL_n71B | Rel-15 |
| DL_n77C_UL_n77C | Rel-15 |
| DL_n48B_UL_n48A |  |
| DL_n48B_UL_n48B |  |
| DL_n48C _UL_n48A |  |
| DL_n48C _UL_n48C |  |

<Unchanged text omitted>

5.x	CA_2DL_n48B

5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Supported bandwidth combinations for CA_2DL_n48B

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_n48B | CA_n48B | 51 | 15 |  |  |  | 20 | 0 |
|  |  | 10 | 10 |  |  |  |  |  |
|  |  | 15 | 51 |  |  |  |  |  |
|  |  | 51 | 20 |  |  |  | 25 |  |
|  |  | 10 | 15 |  |  |  |  |  |
|  |  | 15 | 10 |  |  |  |  |  |
|  |  | 20 | 51 |  |  |  |  |  |
|  |  | 10 | 20 |  |  |  | 30 |  |
|  |  | 15 | 15 |  |  |  |  |  |
|  |  | 20 | 10 |  |  |  |  |  |
|  |  | 15 | 20 |  |  |  | 35 |  |
|  |  | 20 | 15 |  |  |  |  |  |
|  |  | 20 | 20 |  |  |  | 40 |  |
|  |  | 51 | 40 |  |  |  | 45 |  |
|  |  | 40 | 51 |  |  |  |  |  |
|  |  | 10 | 40 |  |  |  | 50 |  |
|  |  | 40 | 10 |  |  |  |  |  |
| CA_n48C | CA_n48C | 10 | 90 |  |  |  | 100 | 0 |
|  |  | 20 | 80 |  |  |  |  |  |
|  |  | 40 | 60 |  |  |  |  |  |
|  |  | 50 | 50 |  |  |  |  |  |
|  |  | 60 | 40 |  |  |  |  |  |
|  |  | 80 | 20 |  |  |  |  |  |
|  |  | 90 | 10 |  |  |  |  |  |
|  |  | 15 | 90 |  |  |  | 105 |  |
|  |  | 90 | 15 |  |  |  |  |  |
|  |  | 10 | 100 |  |  |  | 110 |  |
|  |  | 20 | 90 |  |  |  |  |  |
|  |  | 50 | 60 |  |  |  |  |  |
|  |  | 60 | 50 |  |  |  |  |  |
|  |  | 90 | 20 |  |  |  |  |  |
|  |  | 100 | 10 |  |  |  |  |  |
|  |  | 15 | 100 |  |  |  | 115 |  |
|  |  | 100 | 15 |  |  |  |  |  |
|  |  | 20 | 100 |  |  |  | 120 |  |
|  |  | 40 | 80 |  |  |  |  |  |
|  |  | 60 | 60 |  |  |  |  |  |
|  |  | 80 | 40 |  |  |  |  |  |
|  |  | 100 | 20 |  |  |  |  |  |
|  |  | 40 | 90 |  |  |  | 130 |  |
|  |  | 50 | 80 |  |  |  |  |  |
|  |  | 80 | 50 |  |  |  |  |  |
|  |  | 90 | 40 |  |  |  |  |  |
|  |  | 40 | 100 |  |  |  | 140 |  |
|  |  | 50 | 90 |  |  |  |  |  |
|  |  | 60 | 80 |  |  |  |  |  |
|  |  | 80 | 60 |  |  |  |  |  |
|  |  | 90 | 50 |  |  |  |  |  |
|  |  | 100 | 40 |  |  |  |  |  |
|  |  | 50 | 100 |  |  |  | 150 |  |
|  |  | 60 | 90 |  |  |  |  |  |
|  |  | 90 | 60 |  |  |  |  |  |
|  |  | 100 | 50 |  |  |  |  |  |
| NOTE 1: 51 MHz is not applicable for 30/60kHz SCS |  |  |  |  |  |  |  |  |

5.x.2	Co-existence studies

There are no co-existence issues for this combination.

5.x.3	REFSENS

There are no REFSENS exceptions for this combination. The uplink CA configuration for REFSENS defined as Table 5.x.3-1:

Table 5.x.3-1: Intra-band contiguous CA uplink configuration for reference sensitivity

| CA configuration / CC combination / NRB_agg / Duplex mode |  |  |  |
| --- | --- | --- | --- |
| Uplink CA configuration | All combinations of channel bandwidths defined in Table 5.x.1-1 |  | Duplex Mode |
|  | UL PCC allocation | UL SCC allocation |  |
| CA_48B | NOTE1 | NOTE1 | TDD |
| CA_48C | NOTE1 | NOTE1 | TDD |
| NOTE 1: In case of uplink sub-block is TDD intra-band contiguous CA then the uplink PCC and SCC allocations are the same as the UL configuration defined in Table 7.3.2-3 of TS 38.101-1. |  |  |  |

<End of Text Proposal>