3GPP TSG-RAN WG4 Meeting #91    	R4-1905433

Reno, US, 13th – 17th May, 2019

Agenda item:	6.12.2.3.1

Source:	Samsung

Title:	Updated simulation results for NR PUCCH

Document for:	Discussion

# 1	Introduction.

In this contribution, the updated ideal and impairment simulation results are provided.

# 2	Simulation Results

In this subsection, both ideal simulation results and impairment results are provided in this contribution, as shown in table 1 to table 4, respectively.

Table 1: Ideal and impairment simulation result for NR PUCCH with 1T2R in FR1

| Format | SCS& BW | Number of bit | Numb of symbols | Number of RBs | Test metric | Ideal result | Impairment result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 15Khz, 10MHz | 1 | 1 | 1 | ACK miss | 7.9 | 9.9 |
|  |  | 1 | 2 | 1 |  | 2.7 | 4.7 |
|  | 30KHz, 40MHz | 1 | 1 | 1 |  | 8.6 | 10.6 |
|  |  | 1 | 2 | 1 |  | 2.4 | 4.4 |
| Format 1 | 15Khz, 10MHz | 2 | 14 | 1 | ACK miss | -5.6 | -3.6 |
|  |  |  |  |  | NACK to ACK | -5.3 | -3.3 |
|  | 30KHz, 40MHz |  |  |  | ACK miss | -5 | -3 |
|  |  |  |  |  | NACK to ACK | -4.4 | -2.4 |
| Format2 | 15Khz, 10MHz | 4 | 1 | 4 | ACK miss | 3.4 | 5.4 |
|  |  | 22 | 2 | 9 | BLER@1% | -0.5 | 1.5 |
|  | 30KHz, 40MHz | 4 | 1 | 4 | ACK miss | 2.2 | 4.2 |
|  |  | 22 | 2 | 9 | BLER@1% |  |  |
| Format3 | 15Khz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -1.4 | 1.1 |
|  |  |  |  |  | With additional DMRS | -1.5 | 1 |
|  |  | 16 | 4 | 3 | Without additional DMRS | 0.5 | 2.5 |
|  | 30KHz, 40MHz | 16 | 14 | 1 | Without additional DMRS | -0.8 | 1.7 |
|  |  |  |  |  | With additional DMRS | -0.8 | 1.7 |
|  |  | 16 | 4 | 3 | Without additional DMRS |  |  |
| Format4 | 15Khz, 10MHz | 22 | 14 | 1 | Without additional DMRS | 0.5 | 2.5 |
|  |  |  |  |  | With additional DMRS | 0.5 | 2.5 |
|  | 30KHz, 40MHz |  |  |  | Without additional DMRS | 1.3 | 3.3 |
|  |  |  |  |  | With additional DMRS | 1.4 | 3.4 |
| Format0 | 15Khz, 5MHz | 1 | 1 | 1 | ACK miss | 8.00 | 10.00 |
|  |  | 1 | 2 | 1 |  | 1.70 | 3.70 |
|  | 15Khz, 20MHz | 1 | 1 | 1 |  | 7.80 | 9.80 |
|  |  | 1 | 2 | 1 |  | 1.80 | 3.80 |
|  | 30KHz, 10MHz | 1 | 1 | 1 |  | 8.50 | 10.50 |
|  |  | 1 | 2 | 1 |  | 3.20 | 5.20 |
|  | 30KHz, 20MHz | 1 | 1 | 1 |  | 8.70 | 10.70 |
|  |  | 1 | 2 | 1 |  | 2.10 | 4.10 |
|  | 30KHz, 100MHz | 1 | 1 | 1 |  |  |  |
|  |  | 1 | 2 | 1 |  |  |  |
| Format1 | 15Khz, 5MHz | 2 | 14 | 1 | ACK miss | -6.40 | -4.40 |
|  |  |  |  |  | NACK to ACK | -5.90 | -3.90 |
|  | 15Khz, 20MHz |  |  |  | ACK miss | -6.40 | -4.40 |
|  |  |  |  |  | NACK to ACK | -5.50 | -3.50 |
|  | 30KHz, 10MHz |  |  |  | ACK miss | -5.30 | -3.30 |
|  |  |  |  |  | NACK to ACK | -4.60 | -2.60 |
|  | 30KHz, 20MHz |  |  |  | ACK miss | -6.10 | -4.10 |
|  |  |  |  |  | NACK to ACK | -5.60 | -3.60 |
|  | 30KHz, 100MHz |  |  |  | ACK miss |  |  |
|  |  |  |  |  | NACK to ACK |  |  |
| Format2 | 15Khz, 5MHz | 4 | 1 | 4 | ACK miss | 3.50 | 5.50 |
|  |  | 22 | 2 | 9 | BLER@1% |  |  |
|  | 15Khz, 20MHz | 4 | 1 | 4 | ACK miss | 3.30 | 5.30 |
|  |  | 22 | 2 | 9 | BLER@1% |  |  |
|  | 30KHz, 10MHz | 4 | 1 | 4 | ACK miss | 3.5 | 5.5 |
|  |  | 22 | 2 | 9 | BLER@1% | -1.3 | 0.7 |
|  | 30KHz, 20MHz | 4 | 1 | 4 | ACK miss | 3.2 | 5.2 |
|  |  | 22 | 2 | 9 | BLER@1% | -0.5 | 1.5 |
|  | 30KHz, 100MHz | 4 | 1 | 4 | ACK miss |  |  |
|  |  | 22 | 2 | 9 | BLER@1% |  |  |
| Format3 | 15Khz, 5MHz | 16 | 14 | 1 | Without additional DMRS | -2.30 | 0.20 |
|  |  |  |  |  | With additional DMRS | -2.30 | 0.20 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0.9 | 1.1 |
|  | 15Khz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -2.20 | 0.30 |
|  |  |  |  |  | With additional DMRS | -2.40 | 0.10 |
|  |  | 16 | 4 | 3 | Without additional DMRS | 0.5 | 2.5 |
|  | 30KHz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -0.90 | 1.60 |
|  |  |  |  |  | With additional DMRS | -1.10 | 1.40 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0.8 | 1.2 |
|  | 30KHz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -1.80 | 0.70 |
|  |  |  |  |  | With additional DMRS | -2.00 | 0.50 |
|  |  | 16 | 4 | 3 | Without additional DMRS | 0.3 | 2.3 |
|  | 30KHz, 100MHz | 16 | 14 | 1 | Without additional DMRS |  |  |
|  |  |  |  |  | With additional DMRS |  |  |
|  |  | 16 | 4 | 3 | Without additional DMRS |  |  |
| Format4 | 15Khz, 5MHz | 22 | 14 | 1 | Without additional DMRS | -0.30 | 2.20 |
|  |  |  |  |  | With additional DMRS | -0.20 | 2.30 |
|  | 15Khz, 20MHz |  |  |  | Without additional DMRS | -0.20 | 2.30 |
|  |  |  |  |  | With additional DMRS | -0.30 | 2.20 |
|  | 30KHz, 10MHz |  |  |  | Without additional DMRS | 1.00 | 3.50 |
|  |  |  |  |  | With additional DMRS | 1.10 | 3.60 |
|  | 30KHz, 20MHz |  |  |  | Without additional DMRS | 0.10 | 2.60 |
|  |  |  |  |  | With additional DMRS | 0.10 | 2.60 |
|  | 30KHz, 100MHz |  |  |  | Without additional DMRS |  |  |
|  |  |  |  |  | With additional DMRS |  |  |

Table 2: Ideal and impairment simulation result for NR PUCCH with 1T4R in FR1

| Format | SCS& BW | Number of bit | Numb of symbols | Number of RBs | Test metric | Ideal result | Impairment result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 15Khz, 10MHz | 1 | 1 | 1 | ACK miss | 1.8 | 3.8 |
|  |  | 1 | 2 | 1 |  | -1.8 | 0.2 |
|  | 30KHz, 40MHz | 1 | 1 | 1 |  | 2.1 | 4.1 |
|  |  | 1 | 2 | 1 |  | -2.0 | 0.0 |
| Format 1 | 15Khz, 10MHz | 2 | 14 | 1 | ACK miss | -9.9 | -7.9 |
|  |  |  |  |  | NACK to ACK | -9.6 | -7.6 |
|  | 30KHz, 40MHz |  |  |  | ACK miss | -9.5 | -7.5 |
|  |  |  |  |  | NACK to ACK | -8.7 | -6.7 |
| Format2 | 15Khz, 10MHz | 4 | 1 | 4 | ACK miss | -2.4 | 0.4 |
|  |  | 22 | 2 | 9 | BLER@1% | -4.9 | -2.9 |
|  | 30KHz, 40MHz | 4 | 1 | 4 | ACK miss | -2.2 | -0.2 |
|  |  | 22 | 2 | 9 | BLER@1% |  |  |
| Format3 | 15Khz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -5.7 | -3.7 |
|  |  |  |  |  | With additional DMRS | -6.1 | -4.1 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -4.6 | -2.6 |
|  | 30KHz, 40MHz | 16 | 14 | 1 | Without additional DMRS | -5.2 | -3.2 |
|  |  |  |  |  | With additional DMRS | -5.5 | -3.5 |
|  |  | 16 | 14 | 1 | Without additional DMRS |  |  |
| Format4 | 15Khz, 10MHz | 22 | 14 | 1 | Without additional DMRS | -4.1 | -2.1 |
|  |  |  |  |  | With additional DMRS | -4.4 | -2.4 |
|  | 30KHz, 40MHz |  |  |  | Without additional DMRS | -3.6 | -1.6 |
|  |  |  |  |  | With additional DMRS | -3.7 | -1.7 |
| Format0 | 15Khz, 5MHz | 1 | 1 | 1 | ACK miss | 1.6 | 3.6 |
|  |  | 1 | 2 | 1 |  | -2.4 | -0.4 |
|  | 15Khz, 20MHz | 1 | 1 | 1 |  | 1.7 | 3.7 |
|  |  | 1 | 2 | 1 |  | -2.5 | -0.5 |
|  | 30KHz, 10MHz | 1 | 1 | 1 |  | 2.1 | 4.1 |
|  |  | 1 | 2 | 1 |  | -1.5 | 0.5 |
|  | 30KHz, 20MHz | 1 | 1 | 1 |  | 1.9 | 3.9 |
|  |  | 1 | 2 | 1 |  | -2.2 | -0.2 |
|  | 30KHz, 100MHz | 1 | 1 | 1 |  |  |  |
|  |  | 1 | 2 | 1 |  |  |  |
| Format1 | 15Khz, 5MHz | 2 | 14 | 1 | ACK miss | -10.4 | -8.4 |
|  |  |  |  |  | NACK to ACK | -9.9 | -7.9 |
|  | 15Khz, 20MHz |  |  |  | ACK miss | -10.5 | -8.5 |
|  |  |  |  |  | NACK to ACK | -9.9 | -7.9 |
|  | 30KHz, 10MHz |  |  |  | ACK miss | -9.6 | -7.6 |
|  |  |  |  |  | NACK to ACK | -9.3 | -7.3 |
|  | 30KHz, 20MHz |  |  |  | ACK miss | -10.3 | -8.3 |
|  |  |  |  |  | NACK to ACK | -9.8 | -7.8 |
|  | 30KHz, 100MHz |  |  |  | ACK miss |  |  |
|  |  |  |  |  | NACK to ACK |  |  |
| Format2 | 15Khz, 5MHz | 4 | 1 | 4 | ACK miss | -2.3 | -0.3 |
|  |  | 22 | 2 | 9 | BLER@1% | -5.7 | -3.7 |
|  | 15Khz, 20MHz | 4 | 1 | 4 | ACK miss | -2.4 | -0.4 |
|  |  | 22 | 2 | 9 | BLER@1% | -4.9 | -2.9 |
|  | 30KHz, 10MHz | 4 | 1 | 4 | ACK miss | -2.4 | -0.4 |
|  |  | 22 | 2 | 9 | BLER@1% | -5.6 | -3.6 |
|  | 30KHz, 20MHz | 4 | 1 | 4 | ACK miss | -2.5 | -0.5 |
|  |  | 22 | 2 | 9 | BLER@1% | -4.8 | -2.8 |
|  | 30KHz, 100MHz | 4 | 1 | 4 | ACK miss |  |  |
|  |  | 22 | 2 | 9 | BLER@1% |  |  |
| Format3 | 15Khz, 5MHz | 16 | 14 | 1 | Without additional DMRS | -6.2 | -3.7 |
|  |  |  |  |  | With additional DMRS | -6.6 | -4.1 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -5.5 | -3.5 |
|  | 15Khz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -6.3 | -3.8 |
|  |  |  |  |  | With additional DMRS | -6.7 | -4.2 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -4.6 | -2.6 |
|  | 30KHz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -5.4 | -2.9 |
|  |  |  |  |  | With additional DMRS | -5.7 | -3.2 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -5.7 | -3.7 |
|  | 30KHz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -6 | -3.5 |
|  |  |  |  |  | With additional DMRS | -6.4 | -3.9 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -4.8 | -2.8 |
|  | 30KHz, 100MHz | 16 | 14 | 1 | Without additional DMRS |  |  |
|  |  |  |  |  | With additional DMRS |  |  |
|  |  | 16 | 4 | 3 | Without additional DMRS |  |  |
| Format4 | 15Khz, 5MHz | 22 | 14 | 1 | Without additional DMRS | -4.6 | -2.1 |
|  |  |  |  |  | With additional DMRS | -4.8 | -2.3 |
|  | 15Khz, 20MHz |  |  |  | Without additional DMRS | -4.7 | -2.2 |
|  |  |  |  |  | With additional DMRS | -5 | -2.5 |
|  | 30KHz, 10MHz |  |  |  | Without additional DMRS | -3.7 | -1.2 |
|  |  |  |  |  | With additional DMRS | -3.8 | -1.3 |
|  | 30KHz, 20MHz |  |  |  | Without additional DMRS | -4.5 | -2 |
|  |  |  |  |  | With additional DMRS | -4.7 | -2.2 |
|  | 30KHz, 100MHz |  |  |  | Without additional DMRS |  |  |
|  |  |  |  |  | With additional DMRS |  |  |

Table 3: Ideal and impairment simulation result for NR PUCCH with 1T8R in FR1

| Format | SCS& BW | Number of bit | Numb of symbols | Number of RBs | Test metric | Ideal result | Impairment result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 15Khz, 10MHz | 1 | 1 | 1 | ACK miss | -2.5 | -0.5 |
|  |  | 1 | 2 | 1 |  | -5.2 | -3.2 |
|  | 30KHz, 40MHz | 1 | 1 | 1 |  | -2.2 | -0.2 |
|  |  | 1 | 2 | 1 |  | -5.3 | -3.3 |
| Format 1 | 15Khz, 10MHz | 2 | 14 | 1 | ACK miss | -13.2 | -11.2 |
|  |  |  |  |  | NACK to ACK | -12.9 | -10.9 |
|  | 30KHz, 40MHz |  |  |  | ACK miss | -13 | -11 |
|  |  |  |  |  | NACK to ACK | -12.7 | -10.7 |
| Format2 | 15Khz, 10MHz | 4 | 1 | 4 | ACK miss | -6.3 | -4.3 |
|  |  | 22 | 2 | 9 | BLER@1% | -8.4 | -6.4 |
|  | 30KHz, 40MHz | 4 | 1 | 4 | ACK miss | -5.4 | -3.4 |
|  |  | 22 | 2 | 9 | BLER@1% |  |  |
| Format3 | 15Khz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -9.1 | -6.6 |
|  |  |  |  |  | With additional DMRS | -9.6 | -7.1 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -8.5 | -6.5 |
|  | 30KHz, 40MHz | 16 | 14 | 1 | Without additional DMRS | -8.8 | -6.3 |
|  |  |  |  |  | With additional DMRS | -9.2 | -6.7 |
|  |  | 16 | 14 | 1 | Without additional DMRS |  |  |
| Format4 | 15Khz, 10MHz | 22 | 14 | 1 | Without additional DMRS | -7.7 | -5.2 |
|  |  |  |  |  | With additional DMRS | -8.1 | -5.6 |
|  | 30KHz, 40MHz |  |  |  | Without additional DMRS | -7.4 | -4.9 |
|  |  |  |  |  | With additional DMRS | -7.7 | -5.2 |
| Format0 | 15Khz, 5MHz | 1 | 1 | 1 | ACK miss | -2.5 | -0.5 |
|  |  | 1 | 2 | 1 |  | -5.5 | -0.5 |
|  | 15Khz, 20MHz | 1 | 1 | 1 |  | -2.5 | -0.5 |
|  |  | 1 | 2 | 1 |  | -5.6 | -3.6 |
|  | 30KHz, 10MHz | 1 | 1 | 1 |  | -2.4 | -0.4 |
|  |  | 1 | 2 | 1 |  | -5.1 | -3.1 |
|  | 30KHz, 20MHz | 1 | 1 | 1 |  | -2.2 | -0.2 |
|  |  | 1 | 2 | 1 |  | -5.3 | -3.3 |
|  | 30KHz, 100MHz | 1 | 1 | 1 |  |  |  |
|  |  | 1 | 2 | 1 |  |  |  |
| Format1 | 15Khz, 5MHz | 2 | 14 | 1 | ACK miss | -13.5 | -11.5 |
|  |  |  |  |  | NACK to ACK | -13.3 | -11.3 |
|  | 15Khz, 20MHz |  |  |  | ACK miss | -13.5 | -11.5 |
|  |  |  |  |  | NACK to ACK | -13.2 | -11.2 |
|  | 30KHz, 10MHz |  |  |  | ACK miss | -13.1 | -11.1 |
|  |  |  |  |  | NACK to ACK | -12.7 | -10.7 |
|  | 30KHz, 20MHz |  |  |  | ACK miss | -13.3 | -11.3 |
|  |  |  |  |  | NACK to ACK | -13.1 | -11.1 |
|  | 30KHz, 100MHz |  |  |  | ACK miss |  |  |
|  |  |  |  |  | NACK to ACK |  |  |
| Format2 | 15Khz, 5MHz | 4 | 1 | 4 | ACK miss | -6.3 | -4.3 |
|  |  | 22 | 2 | 9 | BLER@1% | -8.8 | -6.8 |
|  | 15Khz, 20MHz | 4 | 1 | 4 | ACK miss | -6.3 | -4.3 |
|  |  | 22 | 2 | 9 | BLER@1% | -8.3 | -6.3 |
|  | 30KHz, 10MHz | 4 | 1 | 4 | ACK miss | -6.3 | -4.3 |
|  |  | 22 | 2 | 9 | BLER@1% | -8,8 | -6.8 |
|  | 30KHz, 20MHz | 4 | 1 | 4 | ACK miss | -6.5 | -4.5 |
|  |  | 22 | 2 | 9 | BLER@1% | -8.3 | -6.3 |
|  | 30KHz, 100MHz | 4 | 1 | 4 | ACK miss |  |  |
|  |  | 22 | 2 | 9 | BLER@1% |  |  |
| Format3 | 15Khz, 5MHz | 16 | 14 | 1 | Without additional DMRS | -9.4 | -6.9 |
|  |  |  |  |  | With additional DMRS | -9.9 | -7.4 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -9 | -7 |
|  | 15Khz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -9.1 | -6.9 |
|  |  |  |  |  | With additional DMRS | -9.6 | -7.4 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -8.5 | -6.5 |
|  | 30KHz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -8.8 | -6.3 |
|  |  |  |  |  | With additional DMRS | -9.4 | -6.9 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -9.2 | -7.2 |
|  | 30KHz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -9.2 | -6.7 |
|  |  |  |  |  | With additional DMRS | -9.7 | -7.2 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -8.7 | -6.7 |
|  | 30KHz, 100MHz | 16 | 14 | 1 | Without additional DMRS |  |  |
|  |  |  |  |  | With additional DMRS |  |  |
|  |  | 16 | 4 | 3 | Without additional DMRS |  |  |
| Format4 | 15Khz, 5MHz | 22 | 14 | 1 | Without additional DMRS | -8 | -5.5 |
|  |  |  |  |  | With additional DMRS | -8.3 | -5.8 |
|  | 15Khz, 20MHz |  |  |  | Without additional DMRS | -8 | -5.5 |
|  |  |  |  |  | With additional DMRS | -8.3 | -5.8 |
|  | 30KHz, 10MHz |  |  |  | Without additional DMRS | -7.5 | -5 |
|  |  |  |  |  | With additional DMRS | -7.9 | -5.4 |
|  | 30KHz, 20MHz |  |  |  | Without additional DMRS | -7.8 | -5.3 |
|  |  |  |  |  | With additional DMRS | -8.1 | -5.6 |
|  | 30KHz, 100MHz |  |  |  | Without additional DMRS |  |  |
|  |  |  |  |  | With additional DMRS |  |  |

Table 4: Ideal and impairment simulation result for NR PUCCH with 1T2R in FR2

| Format | SCS& BW | Number of bit | Numb of symbols | Number of RBs | Test metric | Ideal result | Impairment result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 60Khz, 100MHz | 1 | 1 | 1 | ACK miss | 7.15 | 9.65 |
|  |  | 1 | 2 | 1 |  | 1.90 | 4.40 |
|  | 120KHz, 100MHz | 1 | 1 | 1 |  | 7.26 | 9.76 |
|  |  | 1 | 2 | 1 |  | 1.92 | 4.42 |
| Format 1 | 60Khz, 100MHz | 2 | 14 | 1 | ACK miss | -7.04 | -4.54 |
|  |  |  |  |  | NACK to ACK | -4.77 | -2.27 |
|  | 120KHz, 100MHz |  |  |  | ACK miss | -7.05 | -4.55 |
|  |  |  |  |  | NACK to ACK | -4.78 | -2.28 |
| Format2 | 60Khz, 100MHz | 4 | 1 | 4 | ACK miss | 2.36 | 4.86 |
|  |  | 22 | 2 | 9 | BLER@1% | -0.73 | 1.77 |
|  | 120KHz, 100MHz | 4 | 1 | 4 | ACK miss | 2.63 | 5.13 |
|  |  | 22 | 2 | 9 | BLER@1% | -0.80 | 1.70 |
| Format3 | 60Khz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -1.34 | 1.16 |
|  |  |  |  |  | With additional DMRS | -2.10 | 0.40 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0.32 | 2.18 |
|  | 120KHz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -1.18 | 1.32 |
|  |  |  |  |  | With additional DMRS | -1.85 | 0.65 |
|  |  | 16 | 14 | 3 | Without additional DMRS | -0.03 | 2.47 |
| Format4 | 60Khz, 100MHz | 22 | 14 | 1 | Without additional DMRS | 2.53 | 5.03 |
|  |  |  |  |  | With additional DMRS | 1.88 | 4.38 |
|  | 120KHz, 100MHz |  |  |  | Without additional DMRS | 2.72 | 5.22 |
|  |  |  |  |  | With additional DMRS | 1.93 | 4.43 |
| Format0 | 60Khz, 50MHz | 1 | 1 | 1 | ACK miss | 7.12 | 9.62 |
|  |  | 1 | 2 | 1 |  | 2.40 | 4.90 |
|  | 120Khz, 50MHz | 1 | 1 | 1 |  | 7.35 | 9.85 |
|  |  | 1 | 2 | 1 |  | 2.33 | 4.83 |
|  | 120Khz, 200MHz | 1 | 1 | 1 |  | 7.21 | 9.71 |
|  |  | 1 | 2 | 1 |  | 1.29 | 3.79 |
| Format1 | 60Khz, 50MHz | 2 | 14 | 1 | ACK miss | -6.64 | -4.14 |
|  |  |  |  |  | NACK to ACK | -3.19 | -0.69 |
|  | 120Khz, 50MHz |  |  |  | ACK miss | -6.62 | -4.12 |
|  |  |  |  |  | NACK to ACK | -4.14 | -1.64 |
|  | 120KHz, 200MHz |  |  |  | ACK miss | -7.55 | -5.05 |
|  |  |  |  |  | NACK to ACK | -5.16 | -2.66 |
| Format2 | 60Khz, 50MHz | 4 | 1 | 4 | ACK miss | 2.15 | 4.65 |
|  |  | 22 | 2 | 9 | BLER@1% | 0.13 | 2.63 |
|  | 120Khz, 50MHz | 4 | 1 | 4 | ACK miss | 2.61 | 5.11 |
|  |  | 22 | 2 | 9 | BLER@1% | 0.6 | 3.1 |
|  | 120KHz, 200MHz | 4 | 1 | 4 | ACK miss | 2.8 | 5.3 |
|  |  | 22 | 2 | 9 | BLER@1% | -0.63 | 1.87 |
| Format3 | 60Khz, 50MHz | 16 | 14 | 1 | Without additional DMRS | -0.75 | 1.75 |
|  |  |  |  |  | With additional DMRS | -1.48 | 1.02 |
|  |  | 16 | 4 | 3 | Without additional DMRS | 0.27 | 2.77 |
|  | 120Khz, 50MHz | 16 | 14 | 1 | Without additional DMRS | -0.81 | 1.69 |
|  |  |  |  |  | With additional DMRS | -1.44 | 1.06 |
|  |  | 16 | 4 | 3 | Without additional DMRS | 0.53 | 3.03 |
|  | 120KHz, 200MHz | 16 | 14 | 1 | Without additional DMRS | -1.79 | 0.71 |
|  |  |  |  |  | With additional DMRS | -2.51 | -0.01 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0.46 | 2.04 |
| Format4 | 60Khz, 50MHz | 22 | 14 | 1 | Without additional DMRS | 2.94 | 5.44 |
|  |  |  |  |  | With additional DMRS | 2.34 | 4.84 |
|  | 120Khz, 50MHz |  |  |  | Without additional DMRS | 3.07 | 5.57 |
|  |  |  |  |  | With additional DMRS | 2.29 | 4.79 |
|  | 120KHz, 200MHz |  |  |  | Without additional DMRS | 2.2 | 4.7 |
|  |  |  |  |  | With additional DMRS | 1.49 | 3.99 |

# 3	Conclusion

In this contribution, the updated ideal and impairment simulation results are provided.

# References

R4-1904718, WF on NR PUCCH demodulation requirements in Rel-15, Huawei