3GPP TSG-RAN WG4 Meeting #91	R4-1905438

# Reno, US, 13th- 17th May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | - | rev | 0 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on UCI on PUSCH performance requirement for TS 38.104 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UCI multiplexed on PUSCH performance requirements have been introduced for NR Rel-15. There is no section for requirements with UCI multiplexed on PUSCH |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new section in Chapter 8 and 11 Add the UCI multipexed on PUSCH performance requirements in Chapters 8 and 11 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UCI multiplexed on PUSCH performance cannot be verified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3, 11.2.1.3 and 11.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.141-1/TS38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<Start of change 1 >>

### 8.2.2	Requirements for PUSCH with transform precoding enabled

### 8.2.3	Performance requirements for UCI multiplexed on PUSCH

#### 8.2.3.1	General

In the test of UCI multiplexed on PUSCH, the UCI information only contains CSI part1 and CSI part2 information, there is no HACK/ACK information transmitted.

The CSI part1 block error probability (BLER) is defined as the probability of incorrectly decoding the CSI part1 information when the CSI part1 information is sent as follow:

ProbCSI part1 incorrectly=#(false CSI part1)#(CSI part1)

Where:

-	#(false CSI part1) denotes the number of incorrectly decoding CSI part1 information transmission occasions

-	#(CSI part1) denotes the number of CSI part1information transmission occsasions

The CSI part2 block error probability (BLER) is defined as the probability of incorrectly decoding the CSI part2 information when the CSI part1 and CSI part2 information are sent.

ProbCSI part2 incorrectly=#(false CSI part2)#(CSI part2)

Where:

-	#(false CSI part2) denotes the number of incorrectly decoding CSI part2 information transmission occasion

-	#(CSI part2) denotes the number of CSI part2 information transmission occsasions

The number of UCI information bit payload per slot is defined for two cases as follows:

●	7 bits:  5 bits for CSI part1, 2 bits for CSI part2

●	40 bits: 20 bits for CSI part1, 20 bits for CSI part2

The UCI payload content with 7 bits case is assumed as [c0, c1, c2, c3, c4] = [0 1 0 1 0] for CSI part1 information, where c0 is mapping to the RI information, and [c0, c1] = [1 0] for CSI part2 information.

The UCI payload content with 40bits case is assumed random codeword selection.

In both of tests, PUSCH data, UCI with CSI part1 and CSI part2 are transmitted simultaneously.

Table: 8.2.3.1-1 Test parameters for testing UCI on PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD |  | 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | Maximum number of OFDM symbols for front loaded DM-RS | 1 |
|  | Number of additional DM-RS symbols | 1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | EPRE ratio of PUSCH to DM-RS | -3 dB |
|  | DM-RS port | {0, 1} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | A, B |
|  | PUSCH starting symbol index | 0 |
|  | PUSCH symbol length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| UCI | PUSCH data transmission | Enabled |
|  | Number of CSI part1 and CSI part2 information bit payload | {5,2}, {20, 20} |
|  | scaling | 1 |
|  | betaOffsetACK-Index1 | 11 |
|  | betaOffsetCSI-Part1-Index1 and betaOffsetCSI-Part1-Index2 | 13 |
|  | betaOffsetCSI-Part2-Index1 and betaOffsetCSI-Part2-Index2 | 13 |
|  | UCI partition for frequency hopping | Disabled |

#### 8.2.3.2	Minimum requirements

The CSI part1 block error probability shall not exceed 0.1% at the SNR given in table 8.2.3.2-1 and table 8.2.3.2-2. The CSI part2 block error probability shall not exceed 1% at the SNR given in table 8.2.3.2-3 and table 8.2.3.2-4.

Table 8.2.3.2-1: Minimum requirements for UCI multiplexed on PUSCH, type A, CSI part1, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | UCI bits (CSI part1, CSI part 2) | DM-RS configuration | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 7 (5, 2) | 1+1 | G-FR1-A4-11 | TBD |
|  | 2 | Normal | TDLC300-100 Low | 40 (20,20) | 1+1 | G-FR1-A4-11 | TBD |

Table 8.2.3.2-2: Minimum requirements for UCI multiplexed on PUSCH, type B, CSI part1, 10MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | UCI bits (CSI part1, CSI part 2) | DM-RS configuration | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 7 (5, 2) | 1+1 | G-FR1-A4-11 | TBD |
|  | 2 | Normal | TDLC300-100 Low | 40 (20,20) | 1+1 | G-FR1-A4-11 | TBD |

Table 8.2.3.2-3: Minimum requirements for UCI multiplexed on PUSCH, type A, CSI part2, 10MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | UCI bits (CSI part1, CSI part 2) | DM-RS configuration | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 7 (5, 2) | 1+1 | G-FR1-A4-11 | TBD |
| 1 | 2 | Normal | TDLC300-100 Low | 40 (20,20) | 1+1 | G-FR1-A4-11 | TBD |

Table 8.2.3.2-4: Minimum requirements for UCI multiplexed on PUSCH, type B, CSI part2, 10MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | UCI bits (CSI part1, CSI part 2) | DM-RS configuration | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 7 (5, 2) | 1+1 | G-FR1-A4-11 | TBD |
| 1 | 2 | Normal | TDLC300-100 Low | 40 (20,20) | 1+1 | G-FR1-A4-11 | TBD |

<<end of change 1 >>

<<Start of change 2 >>

## 11.2	Performance requirements for PUSCH

### 11.2.1	Requirements for BS type 1-O

#### 11.2.1.1	Requirements for PUSCH with transform precoding disabled

Apply the requirements defined in subclause 8.2.1 for 2Rx.

#### 11.2.1.2	Requirements for PUSCH with transform precoding enabled

Apply the requirements defined in subclause 8.2.2 for 2Rx.

#### 11.2.1.3	Requirements for UCI multiplexd on PUSCH

Apply the requirements defined in subclause 8.2.3 for 2Rx.

<<end of change 2 >>

<<Start of change 3 >>

### 11.2.2	Requirements for BS type 2-O

#### 11.2.2.1	Requirements for PUSCH with transform precoding disabled

#### 11.2.2.2	Requirements for PUSCH with transform precoding enabled

#### 11.2.2.3	Requirements for UCI multiplexed on PUSCH

##### 11.2.2.3.1	General

In the test of UCI multiplexed on PUSCH, the UCI information only contains CSI part1 and CSI part2 information, there is no HACK/ACK information transmitted.

The CSI part1 block error probability (BLER) is defined as the probability of incorrectly decoding the CSI part1 information when the CSI part1 information is sent as follow:

ProbCSI part1 incorrectly=#(false CSI part1)#(CSI part1)

Where:

-	#(false CSI part1) denotes the number of incorrectly decoding CSI part1 information ransmission occasions

-	#(CSI part1) denotes the number of CSI part1 information transmission occsasions

The CSI part2 block error probability (BLER) is defined as the probability of incorrectly decoding the CSI part2 information when the CSI part1 and CSI part2 information are sent.

ProbCSI part2 incorrectly=#(false CSI part2)#(CSI part2)

Where:

-	#(false CSI part2) denotes the number of incorrectly decoding CSI part2 information ransmission occasions.

-	#(CSI part2) denotes the number of CSI part2 information transmission occsasions

The number of UCI information bit payload per slot is defined for two cases as follows:

●	7 bits:  5 bits for CSI part1, 2 bits for CSI part2

●	40 bits: 20 bits for CSI part1, 20 bits for CSI part2

The UCI payload content with 7 bits case is assumed as [c0, c1, c2, c3, c4] = [0 1 0 1 0] for CSI part1information, where c0 is mapping to the RI information, and [c0, c1] = [1 0] for CSI part2 information.

The UCI payload content with 40bits case is assumed random codeword selection.

In both of tests, PUSCH data, UCI with CSI part1 and CSI part2 are transmitted simultaneously.

Table: 11.2.2.3.1-1 Test parameters for testing UCI multiplexed on PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD |  | 120 kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | Maximum number of OFDM symbols for front loaded DM-RS | 1 |
|  | Number of additional DM-RS symbols | 0 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | EPRE ratio of PUSCH to DM-RS | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | PUSCH starting symbol index | 0 |
|  | PUSCH symbol length | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| UCI | PUSCH data transmission | Enabled |
|  | Number of CSI part1 and CSI part2 information bit payload | {5, 2}, {20,20} |
|  | scaling | 1 |
|  | betaOffsetACK-Index1 | 11 |
|  | betaOffsetCSI-Part1-Index1 and betaOffsetCSI-Part1-Index2 | 13 |
|  | betaOffsetCSI-Part2-Index1 and betaOffsetCSI-Part2-Index2 | 13 |
|  | UCI partition for frequency hopping | Disabled |

##### 11.2.2.3.2	Minimum requirements

The CSI part1 block error probability shall not exceed 0.1% at the SNR given in table 11.2.2.3.2-1. The CSI part2 block error probability shall not exceed 1% at the SNR given in table 11.2.2.3.2-2.

Table 11.2.2.3.2-1: Minimum requirements for UCI multiplexed on PUSCH, type B, CSI part1, 50MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | UCI bits (CSI part1, CSI part 2) | DM-RS configuration | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 7 (5, 2) | 1 | G-FR2-A4-3 | TBD |
|  | 2 | Normal | TDLA30-300 Low | 40 (20,20) | 1 | G-FR2-A4-3 | TBD |

Table 11.2.2.3.2-2: Minimum requirements for UCI multiplexed on PUSCH, type B, CSI part2, 50MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | UCI bits (CSI part1, CSI part 2) | DM-RS configuration | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 7 (5, 2) | 1 | G-FR2-A4-3 | TBD |
|  | 2 | Normal | TDLA30-300 Low | 40 (20,20) | 1 | G-FR2-A4-3 | TBD |

<<end  of change 3 >>