3GPP TSG-RAN WG4 Meeting #91	R4-1905440

# Reno, US, 13th- 17th May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | - | rev |  | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on UCI on PUSCH radicated performance requirement for TS 38.141-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UCI multiplexed on PUSCH performance requirements have been introduced for NR Rel-15. There is no section for requirements with UCI multiplexed on PUSCH |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new section 8.2.3 in Chapter 8 Add the UC multipIexed on PUSCH performance requirements in Chapter 8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR UCI multiplexed on PUSCH performance can’t be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<Start of change 1 >>

### 8.2.2	Performance requirements for PUSCH with transform precoding enabled

### 8.2.3	Performance requirements for UCI multiplexed on PUSCH

#### 8.2.3.1	Definition and applicability

The performance requirement of UCI multiplexed on PUSCH is determined by two paramters: block error probabitiy of CSI part1 and block error probability of CSI part2. The performance is measured by the required SNR at block error probability of CSI part1 not exceeding 0.1%, and the required SNR at block error probability not exceeding 1%.

The CSI part 1 block error probability (BLER) is defined as the probability of incorrectly decoding the CSI part1 information when the CSI part1 information

The CSI part 2 block error probability (BLER) is defined as the probability of incorrectly decoding the CSI part2 information when the CSI part1 and CSI part2 information are sent.

In the test of UCI multiplexed on PUSCH, the UCI information only contains CSI part1 and CSI part2 information, there is no HACK/ACK information transmitted.

The Number of UCI information bit payload per slot is defined for two cases as follows:

●	7 bits:  5 bits for CSI part1, 2 bits for CSI part2

●	40 bits: 20 bits for CSI part1, 20 bits for CSI part2

The UCI payload content with 7 bits case is assumed as [c0, c1, c2, c3, c4] = [0 1 0 1 0] for CSI part1information, where c0 is mapping to the RI information, and [c0, c1] = [1 0] for CSI part2 information.

The UCI payload content with 40bits case is assumed random codeword selection.

In both of tests, PUSCH data, UCI with CSI part1 and CSI part2 are transmitted simultaneously.

Which specifc test(s) is applicable to BS is based on the test applicability rule defined in subclause 8.1.2.

#### 8.2.3.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2] subclause 11.2.1.3.

For BS type 2-O, the minimum requirement is in TS 38.104 [2] subclause 11.2.2.3.

#### 8.2.3.3	Test Purpose

The test shall verify the receiver’s ability to detect UCI CSI part1 and CSI part2 bits multiplexed on PUSCH under multipath fading propagation conditions for a given SNR.

#### 8.2.3.4	Method of test

##### 8.2.3.4.1	Initial Conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see sub-clause 4.9.1

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (D.31).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

##### 8.2.3.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters lised in table 8.2.3.4.2-1. The UCI information bit payload per slot is equal to 7bits with CSI part1 5bits, CSI part2 2bits, and the UCI information bit payload per slot is equal to 40 bits with CSI part1 20bits, CSI part2 20 bits.

Table: 8.2.3.4.2-1 Test parameters for testing UCI multiplexed on PUSCH

| Parameter |  | BS type 1-O | BS type 2-O |
| --- | --- | --- | --- |
| Transform precoding |  | Disabled |  |
| Uplink-downlink allocation for TDD |  | 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 120 kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 1 |  |
|  | RV sequence | 0 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | Maximum number of OFDM symbols for front loaded DM-RS | 1 |  |
|  | Number of additional DM-RS symbols | 1 | 0 |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | EPRE ratio of PUSCH to DM-RS | -3 dB |  |
|  | DM-RS port | {0,1} | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |  |
| Time domain resource | PUSCH mapping type | A,B | B |
|  | PUSCH starting symbol index | 0 |  |
|  | PUSCH symbol length | 14 | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| UCI | PUSCH data transmission | Enabled |  |
|  | Number of CSI part1 and CSI part2 information bit payload | {5,2}, {20, 20} |  |
|  | scaling | 1 |  |
|  | betaOffsetACK-Index1 | 11 |  |
|  | betaOffsetCSI-Part1-Index1 and betaOffsetCSI-Part1-Index2 | 13 |  |
|  | betaOffsetCSI-Part2-Index1 and betaOffsetCSI-Part2-Index2 | 13 |  |
|  | UCI partition for frequency hopping | Disabled |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.2.3.5.1 and 8.2.3.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in Table 8.2.3.4.2-2.

Table 8.2.3.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 30 | 10 | [-80.7] - ΔOTAREFSENS dBm / 8.64MHz |
| BS type 2-O | 120 | 50 | TBD |

8)	The signal genentors sends a test partten where UCI with CSI part1 and CSI part2 information can be multiplexed on PUSCH. The following statistics are kept: the number of incorrectly decoded CSI part1 information transmission, the number of incorrectly decoded CSI part2 information transmission during UCI mupltiplexed on PUSCH transmission.

#### 8.2.3.5	Test Requirement

##### 8.2.3.5.1	Test requirement for BS type 1-O

The fraction of incorrectly decoded UCI CSI part1 according to subclause 8.2.3.4.2 shall be less than 0.1% for the SNR lised in table 8.2.3.5.1-1 and table 8.2.3.5.1-2. The fraction of incorrectly decoded UCI CSI part2 according to subclause 8.2.3.4.2 shall be less than 1% for the SNR lised in table 8.2.3.5.1-3 and table 8.2.3.5.1-4.

Table 8.2.3.5.1-1: Test requirements for UCI multiplexed on PUSCH, type A, CSI part1, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | UCI bits (CSI part1, CSI part 2) | DM-RS configuration | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 7 (5, 2) | 1+1 | G-FR1-A4-11 | TBD |
|  | 2 | Normal | TDLC300-100 Low | 40 (20,20) | 1+1 | G-FR1-A4-11 | TBD |

Table 8.2.3.5.1-2: Test requirements for UCI multiplexed on PUSCH, type B, CSI part1, 10MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | UCI bits (CSI part1, CSI part 2) | DM-RS configuration | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 7 (5, 2) | 1+1 | G-FR1-A4-11 | TBD |
|  | 2 | Normal | TDLC300-100 Low | 40 (20,20) | 1+1 | G-FR1-A4-11 | TBD |

Table 8.2.3.5.1-3: Test requirements for UCI multiplexed on PUSCH, type A, CSI part2, 10MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | UCI bits (CSI part1, CSI part 2) | DM-RS configuration | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 7 (5, 2) | 1+1 | G-FR1-A4-11 | TBD |
|  | 2 | Normal | TDLC300-100 Low | 40 (20,20) | 1+1 | G-FR1-A4-11 | TBD |

Table 8.2.3.5.1-4: Test requirements for UCI multiplexed on PUSCH, type B, CSI part2, 10MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | UCI bits (CSI part1, CSI part 2) | DM-RS configuration | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 7 (5, 2) | 1+1 | G-FR1-A4-11 | TBD |
|  | 2 | Normal | TDLC300-100 Low | 40 (20,20) | 1+1 | G-FR1-A4-11 | TBD |

##### 8.2.3.5.2	Test requirement for BS type 2-O

The fraction of incorrectly decoded UCI CSI part1 measured according to subclause 8.2.3.4.2 shall be less than 0.1% for the SNR lised in table 8.2.3.5.2-1. The fraction of incorrectly decoded UCI CSI part2 measured according to subclause 8.2.3.4.2 shall be less than 1% for the SNR lised in table 8.2.3.5.2-2.

Table 8.2.3.5.2-1: Test requirements for UCI multiplexed on PUSCH, type B, CSI part1, 50 MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation braches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | UCI bits (CSI part1, CSI part 2) | DM-RS configuration | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 7 (5, 2) | 1 | G-FR2-A4-4 | TBD |
|  | 2 | Normal | TDLA30-300 Low | 40 (20,20) | 1 | G-FR2-A4-4 | TBD |

Table 8.2.3.5.2-2: Test requirements for UCI multiplexed on PUSCH, type B, CSI part1, 50MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation braches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | UCI bits (CSI part1, CSI part 2) | DM-RS configuration | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 7 (5, 2) | 1 | G-FR2-A4-4 | TBD |
|  | 2 | Normal | TDLA30-300 Low | 40 (20,20) | 1 | G-FR2-A4-4 | TBD |

<<end of change 1 >>