3GPP TSG-RAN WG4 Meeting #91	R4-1905457

# Reno, US, 13th– 17th May, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.810 | CR | <CR#> | rev | <Rev#> | Current version: | 16.2.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR38.810 on beam correspondence |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_test_methods |  |  |  |  |  | Date: |  |  | 2018-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The test procedures of beam correspondence requirements are not informative enough for the detailed configuration of DUT and test equipments. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This draft CR changes the following items: - specify detailed requirements on downlink reference signals for beam correspondence; - specify the detailed requirements on spatialRelationInfo IE configuration; - clarify that DUT transmits PUSCH for EIRP measurement; - clarify the detailed TX beam refinement procedures for beam correspondence capability [bit-0] DUT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.3.7, 5.2.1.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes 1>>

##### 5.2.1.3.7	TX Beam Peak direction search and EIRP Spherical Coverage

The beam peak search and spherical coverage test procedure apply to DUTs with different beam correspondence capability, as defined in TS38.306 [19]. For the purpose of DUT refining TX beam with beam correspondence capability, the System Simulator shall assure the presensce of downlink reference signals including both SSB and CSI-RS and Type D QCL shall be maintained between SSB and CSI-RS.

The TX beam peak direction is found with a 3D EIRP scan (separately for each orthogonal downlink polarization). The TX beam peak direction search grid points for this single grid approach are defined in Annex G.2. Alternatively, a coarse and fine grid approach could be used according to the definition in Annex G.2.4.

The measurement procedure includes the following steps for each of the points in the grid:

1)	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= to form the TX beam towards the measurement antenna.

2)	DUT refines its TX beam toward that direction depending on DUT’s beam correspondence capability which shall match OEM declaration: if  DUT’s beam correspondence capability is [bit-1], then DUT chooses autonomously the corresponding TX beam for PUSCH transmission using downlink reference signals to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping; if DUT’s beam correspondence capability is [bit-0], then DUT chooses the TX beam for PUSCH transmission using downlink reference signals which is based on beam correspondence with relying on both DL measurements and uplink beam sweeping by its network-assisted uplink beam management capability, as defined in TS38.306 [19].

3)	Lock the beam.

4)	Measure the mean power Pmeas(PolMeas=PolLink=) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator).

5)	Calculate EIRP (PolMeas=PolLink=) by adding the composite loss of the entire transmission path for utilized signal path, LEIRP,θ, and frequency to the measured power Pmeas(PolMeas=PolLink=)

6)	Measure the mean power Pmeas (PolMeas=PolLink=) of the modulated signal arriving at the power measurement equipment.

7)	Calculate EIRP (PolMeas=PolLink=) by adding the composite losses of the entire transmission path for utilized signal path, LEIRP,ϕ, and frequency to the measured power Pmeas (PolMeas=PolLink=)

8)	Calculate total EIRP(PolLink=) = EIRP(PolMeas=PolLink=) + EIRP(PolMeas=PolLink=)

9)	Unlock the beam.

10)	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink=polarization to form the TX beam towards the measurement antenna.

11)	Repeat steps 2) to 9).

For beam correspondence capability [bit-0] DUT, the above step 2) can be further clarified as following sub-steps:

2.1) DUT uses downlink reference signals to select proper RX beam and uses autonomous beam correspondence to select the TX beam.

2.2) SS configures M=8 SRS resources to UE, with the field of spatialRelationInfo to be not present.

2.3) Based on the DUT autonomous chosen TX beam, DUT chooses M=8 beams to transmit M=8 SRS-resources.

2.4) Based on measurement of the received SRS, SS chooses the best SRS within the SRS Resource Indicator (SRI) included in the scheduling grant for PUSCH.

2.5) DUT transmits PUSCH corresponding to the SRS resource selected by the SRI.

The TX beam peak direction is where the maximum total component of EIRP(PolLink=) or EIRP(PolLink=) is found.

The EIRP results from the TX beam peak search using the minimum number of grid points as described in Annex G.2 can be re-used for EIRP spherical coverage. In case a coarse beam peak grid is used for TX beam peak search, using the minimum number of grid points defined in Annex G.3.3.2.3, the EIRP results can be re-used for EIRP spherical coverage.

In case a separate test is performed for EIRP spherical coverage, the procedure above shall be followed using the minimum number of grid points defined in Annex G.3.3.2.3 for spherical coverage.

The EIRPtarget-CDF is then obtained from the Cumulative Distribution Function (CDF) computed using maximum(EIRP(PolLink=), EIRP(PolLink=)) for all grid points. When using constant step size measurement grids, a theta-dependent correction shall be applied, i.e., the PDF probability contribution for each measurement point is scaled by sin(θ).

<< End of Changes 1>>

<< Start of Changes 2>>

##### 5.2.1.3.9	Beam Correspondence Tolerance

The beam correspondence requirement is fulfilled if the DUT satisfies one of the following conditions, depending on the beam correspondence capability, as defined in TS38.306 [19]:

-	If [bit-1], the DUT shall meet the minimum peak EIRP requirement and spherical coverage requirement with its autonomously chosen UL beams and without uplink beam sweeping. Such a DUT is considered to have met the beam correspondence tolerance requirement.

-	If [bit-0], the DUT shall meet the minimum peak EIRP requirement and spherical coverage requirement with uplink beam sweeping. Such a DUT shall meet the beam correspondence tolerance requirement and shall support uplink beam management, as defined in TS38.306 [19].

Thus, the beam correspondence tolerance test is only applicable to the DUT that has beam correspondence capability as [bit-0] (which shall match OEM declaration), such that DUT relies on uplink beam sweeping to fulfil the minimum peak EIRP and spherical coverage requirements.

ΔEIRPBC is introduced for beam correspondence tolerance based on two EIRP measurements (EIRP1 and EIRP2). EIRP1 is the measured total EIRP based on the beam which DUT chooses autonomously (corresponding beam) to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping. EIRP2 is the measured total EIRP based on the beam yielding highest EIRP in a given direction, which is based on beam correspondence with relying on UL beam sweeping. ΔEIRPBC shall be calculated over the link angles spanning a subset of the spherical coverage grid points which are corresponding to the top 50% of the EIRP2 measurement.

For each of the points in the grid:

1)	Follow the test procedures specified in subclause 5.2.1.3.7 with uplink beam sweeping disabled, obtain total EIRP1(PolLink=) and total EIRP1(PolLink=EIRP1 is calculated by EIRP1 = maximum(EIRP1(PolLink=), EIRP1(PolLink=).

2)	Follow the test procedures specified in subclause 5.2.1.3.7, with uplink beam sweeping enabled (SS does not configure the spatialRelationInfo to DUT) during DUT TX beam refinement, obtain total EIRP2(PolLink=) and total EIRP2(PolLink=EIRP2 is calculated by EIRP2 = maximum(EIRP2(PolLink=), EIRP2(PolLink=).

3)	Calculate the ΔEIRPBC = EIRP2 – EIRP1.

The ΔEIRPtarget-CDF is then obtained from the Cumulative Distribution Function (CDF) computed using ΔEIRPBC for each of all top 50% of the EIRP2 measurement points in the grid. When using constant step size measurement grids, a theta-dependent correction shall be applied, i.e., the PDF probability contribution for each measurement point is scaled by sin(θ).

Editor’s Note: The side conditions for downlink reference signals SSB and CSI-RS in beam correspondence tolerance test are FFS.

<< End of Changes 2>>