3GPP TSG-RAN WG4 Meeting RAN4#91                                              R4-1905459

Reno ,US, 13 -17 May, 2019

Agenda Item:   6.12.1.2.1

Source:        Samsung

Title:          Simulation results for HST test cases

Document for:  Discussion

# Introduction

The NR PDSCH demodulation alignment simulation assumptions were approved in the RAN4 #90bis meeting [1].

Companies were encouraged to provide performance evaluations and analysis of the PDSCH simulation assumptions. In this contribution we provide simulation results and our views for the HST scenarios.

# Discussion

## Simulation assumptions

In the previous meeting several options were identified as following:

| Parameters | Configurations |
| --- | --- |
| CBW/SCS | o Case1: FR1 FDD 10MHz CBW 15kHz SCS o Case2: FR1 TDD 40MHz CBW 30kHz |
| HARQ RV sequence | o {0, 2, 3,1} for all the modulation orders |
| PDSCH scheduling | o Skip scheduling PDSCH for PBCH slot for FR1 i.e. slot 0 per 20ms o PDSCH mapping type A o PRB bundling: 2 o Full PRB allocation |
| Control symbols assumption | o Symbols #0 and 1 in each slot o Do not schedule PDSCH in in control symbols duration |
| DMRS | o Type 1 single symbol front loaded o Number of additional DMRS:  2 additional DMRSs for HST scenarios and fading channel i.e. for full DL slots OFDM symbols with DMRS: #2, #7 and #11 o FDM is applied between DMRS and data for test cases with 1 Layer and 2 Layers |
| TRS | o Pattern: 2 slots o Periodicity 10 ms for HST tests o Offset 1 slot for HST tests and 10ms from SSB for other tests o OFDM symbols with TRS:  #6 and #10 o Bandwidth: “52 PRB” for HST tests o Start PRB coincides with PRB #0 of DL BWP o 0 dB boosting. |
| CSIRS | NZP CSI-RS; ZP CSI-RS. RRC configuration; Configuration 1; Configuration 1. periodicityAndOffset; 20 ms, 0 slots; 20 ms, 0. nrofPorts; 2 or 4; 4. frequencyDomainAllocation; ‘000001’ Row 3 for 2 ports Row 5 for 4 ports; Row5, ‘000100’. firstOFDMSymbolInTimeDomain; 12; 12. cdm-Type; 2; 2. density; 1; 1. |
| TDD Configuration | - FR1 30kHz SCS - Configuration 1: 7D1S2U, S = 6D:4G:4U |
| Overhead for TBS determination | 0 |
| Channel models | - HST single tap channel model with the following parameters: Parameter; Value. Ds; 300 m. Dmin; 2 m. v; 300 km/h. fd; 750 Hz for 15 kHz SCS test and 1000 Hz for 30 kHz SCS test. |
| Receiver assumption | - MMSE-IRC - TRS based tracking for HST tests |
| MIMO configuration | - 1x2 - 1x4 |
| PDSCH type and MCS | - Type A, 16QAM, MCS 13 |
| Number of layer | - 1 |
| Number of PDSCH PRBs | - FDD:52 - TDD:106 |
| Channel Model | - Single tap HST |
| Max number of HARQ process | - FDD: 4 - TDD: 8 |

## Simulation results

In the following, we present our simulation results for case 15 with HST channel model including FDD 1x2, FDD 1x4, TDD 1x2 and TDD 1x4 scenarios.

- FDD 1x2

Figure 1: Absolute throughput for FR1 FDD 1x2

- FDD 1x4

Figure 2: Absolute throughput for FR1 FDD 1x4

- TDD 1x2

Figure 3: Absolute throughput for FR1 TDD 1x2

- TDD 1x4

Figure4: Absolute throughput for FR1 TDD 1x4

Table 1:   SNR point of case 15

|  | configuration | FDD 1x2 | FDD 1x4 | TDD 1x2 | TDD 1x4 |
| --- | --- | --- | --- | --- | --- |
| SNR at 30% relative throughput (dB) | No compensation | --- | --- | --- | --- |
|  | Compensation with ideal Fd | -1.4 | -4.5 | -1.4 | -4.2 |
|  | Compensation with estimated Fd | -1.4 | -4.2 | -1.4 | -4.0 |
| SNR at 60% relative throughput (dB) | No compensation | --- | --- | --- | --- |
|  | Compensation with ideal Fd | 3.3 | 0.2 | 3.3 | 0.3 |
|  | Compensation with estimated Fd | 3.3 | 0.2 | 3.3 | 0.3 |
| SNR at 70% relative throughput (dB) | No compensation | --- | --- | --- | --- |
|  | Compensation with ideal Fd | 3.5 | 0.4 | 3.5 | 0.5 |
|  | Compensation with estimated Fd | 3.6 | 0.4 | 3.5 | 0.5 |
| SNR at 80% relative throughput (dB) | No compensation | --- | --- | --- | --- |
|  | Compensation with ideal Fd | 3.7 | 0.6 | 3.7 | 0.7 |
|  | Compensation with estimated Fd | 3.7 | 0.6 | 3.7 | 0.7 |

Observation:

- There is no significant throughput difference when the maximum Doppler shift varied from 875Hz (FDD 875Hz) to 1000Hz (TDD 1000Hz) for MCS13

- With agreed test set-up (TRS configurations, Doppler shift), UE is able to track the Doppler shift without performance degradation in single tap HST channel model

- There is significant performance degradation without frequency shift tracking and compensation.

- For single tap HST channel, throughput ramp up drastically from 50%~100% within 1dB SNR range.

Proposal 1: In order to achieve relative stable test points, we propose to use 30% relative TP as the test point to define SNR requirements.

# Conclusion

In this contribution, we provided preliminary simulation results for HST test with above observations.

The main proposal is as follows:

Proposal 1: In order to achieve relative stable test points, we propose to use 30% relative TP as the test point to define SNR requirements.

# Reference

[1] RAN4 email reflector, “Simulation assumptions for NR PDSCH demodulation performance requirements”, Intel Corporation