3GPP TSG-RAN WG4 Meeting RAN4#91                                              R4-1905460

Reno ,US, 13 -17 May, 2019

Agenda Item:   6.12.1.2.1

Source:        Samsung

Title:          Simulation results for additional TDD patterns

Document for:  Discussion

# Introduction

The NR PDSCH demodulation alignment simulation assumptions were approved in the RAN4 #90bis meeting [1].

Companies were encouraged to provide performance evaluations and analysis of the PDSCH simulation assumptions. In this contribution we provide simulation results and our views for the additional TDD patterns scenarios.

# Discussion

## Simulation assumptions

In the previous meeting the case 16 and case 17 were required to be evaluated, the options were identified as following:

- Case16 Case Number; CHBW/ SCS; MIMO; PDSCH type and MCS; Number of layer; Number of PDSCH PRBs; Channel Model; TDD Configuration; Max number of HARQ process. 16; 40MHz/30kHz; 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low; Type A, QPSK MCS 4; 1 Layer; 106; TDL-B 100ns, 400 Hz; Configuration 4; [10]. - Case17 Case Number; CHBW/ SCS; MIMO; PDSCH type and MCS; Number of layer; Number of PDSCH PRBs; Channel Model; TDD Configuration; Max number of HARQ process. 17; 40MHz/30kHz; 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low; Type A, QPSK MCS 4; 1 Layer; 106; TDL-B 100ns, 400 Hz; Configuration 5; [10].

## Simulation results

In the following, we present our simulation results for case 16, and case 17 separately.

- Case 16

For case 16, the TDD configuration is as following:

Configuration 4: DSSU, S1=10D:2G:2U, S2=12D:2G

Figure1: Absolute throughput for case16

- Case 17

For case 17, the TDD configuration is:

Configuration 5: DSUU, S=12D:2G

Figure 2: Absolute throughput for case 17

Observation:

There is no significant difference between case 16 and case 17. We choose 70% relative throughput as reference test case point:

|  | Alignment results | Impairment results |
| --- | --- | --- |
| Test case 16 2Rx | -3.3 | -1.3 |
| Test case 16 4Rx | -5.6 | -3.6 |
| Test case 17 2Rx | -3.2 | -1.2 |
| Test case 17 4Rx | -5.6 | -3.6 |

# Conclusion

In this contribution, we provided preliminary PDSCH simulation results for the additional TDD pattern cases with below observations and proposals.

# Reference

[1] RAN4 email reflector, “Simulation assumptions for NR PDSCH demodulation performance requirements”, Intel Corporation