3GPP TSG-RAN WG4 Meeting #91	R4-1905507

# Reno, US,  13th  - 17th May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of RefSens exceptions due to UL harmonic interference for NR CA in 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-4-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the proposals on discussion paper R4-1905506. Proposal 2: Unless stated otherwise, UL resource blocks shall be centred within the transmission bandwidth configuration for the channel bandwidth. (Note: This configuration is defined for combinations with direct overlap. For combinations with no direct overlap, the uplink configuration and allocation would be defined case by case.) Proposal 3: UL RB restriction defined for single carrier case should not be used to further restrict the aggressor UL allocation in refsense exception requirements due to UL harmonics. Proposal 4: For 2nd order harmonics, for the lower MSD (MSD B) region where channel bandwidth is larger than 20MHz, either change the current 0dB requirements to N/A, or change the offset equation to accommodate channel bandwidth larger than 20MHz. For the case of smaller or equal to 20MHz, the current location is kept. There exists a ∆FHD which is explained in R4-147285 that should be included. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For 2nd order harmonics, for the lower MSD (MSD B) region where channel bandwidth is larger than 20MHz, change the current 0dB requirements to N/A. Add a “Note 3: Unless stated otherwise, UL resource blocks shall be centred within the transmission bandwidth configuration for the channel bandwidth.” This could be served as a general guideline is defined for combinations with direct overlap. For combinations with no direct overlap, the uplink configuration and allocation would be defined case by case. Add ∆FHD for Note 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 2nd order harmonics is problematic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change >

7.3A.4	Reference sensitivity exceptions due to UL harmonic interference for CA

Sensitivity degradation is allowed for a band in frequency range 1 if it is impacted by UL harmonic interference from another band in frequency range 1 of the same CA configuration. Reference sensitivity exceptions are specified in Table 7.3A.4-1 with uplink configuration specified in Table 7.3A.4-2.

Table 7.3A.4-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n3 | n771,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.9 | 16.1 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  | N/A | N/A | N/A | N/A | N/A | N/A |
| n3 | n781,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.9 | 16.1 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  | N/A | N/A | N/A | N/A | N/A | N/A |
| n8 | n784,5 |  | 10.8 | 9.1 | 8.0 |  |  | 5.1 | 4.2 | 3.5 | 2.3 | 2.1 | 1.4 |
| n8 | n796,7 |  |  |  |  |  |  | [6.8] | 6.2 | [5.6] | 4.9 |  | 4.4 |
| n28 | n751,2 | 28.1 | 25.3 | 24.0 | 22.8 |  |  |  |  |  |  |  |  |
|  | n786,7 |  | [10.4] | [8.9] | [7.8] |  |  | [4.7] | [3.7] | [3] | [1.7] | [1.2] | [0.7] |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n3-n77, CA_n3-n78. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 5: The requirements should be verified for UL NR-ARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 7: The requirements should be verified for UL NR-ARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n3 | n77 |  | 25 | 36 | 50 |  |  | 100 | 128 | 160 | 160 | 160 | 160 |
| n3 | n78 |  | 25 | 36 | 50 |  |  | 100 | 128 | 160 | 160 | 160 | 160 |
| n8 | n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |
| n8 | n79 |  |  |  |  |  |  | 25 | 25 | 25 | 25 |  | 25 |
| n28 | n75 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |  |
| n28 | n78 |  | 10 | 15 | 20 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |
| NOTE 1: 15 kHz SCS is assumed for UL band. NOTE 2: The UL configuration applies regardless of the channel bandwidth of the low band. NOTE 3: Unless stated otherwise, UL resource blocks shall be centred within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of change >