3GPP TSG-RAN WG4 Meeting #91                                                                 R4-1905515

Reno, USA, 13 May - 17 May 2019

Source:	Huawei, HiSilicon

Title:	TP to TR 37.716.21-21 for DC_1_n79-n258

Agenda item:	9.8.3

Document for:	Approval

Background

This contribution provides text proposal on the EN-DC configuration DC_1A_n79A-n258A as defined in Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) RP-190151 [1].

Text Proposal

---Start of changes---

6.x	DC_1_n79-n258

6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n79-n258 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| EN-DC configuration | Uplink configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n79A-n258A | DC_1A_n79A DC_1A_n258A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |

6.x.3	Co-existence studies

Co-existence studies of DC_1A_n79A are captured in 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_1A_n258A there is no requirements specified.

6.x.4	∆TIB and ∆RIB values

For DC_1_n79-n258, the TIB,c and RIB values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n79-n258 | 1 | 0 |
|  | n79 | 0 |
|  | n258 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n79-n258 | 1 | 0 |
|  | n79 | 0 |
|  | n258 | 0 |

6.x.5	MSD

There is no additional MSD issue for DC_1_n79-n258.

---End of changes---

Reference

[1]	RP-190151, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics