3GPP TSG-RAN WG4 Meeting #91	R4-1905545

Reno, US, 13th– 17th May, 2019

Source: 	ZTE

Title:	Further discussion on the frequency range for EMC RE test

Agenda Item:	6.9.2.1

Document for:	Approval

Introduction

The contribution [1] in RAN4#90bis gave a suggestion to update the upper frequency for NR BS RE test from 12.75GHz to 5th harmonic of the DL operating band for BS type 1-C and BS type 1-H. In this contribution, we further analysis the necessity of the higher frequency range in NR BS RE test.

Discussion

In current TS38.113[2], the frequency range for NR BS RE test are from 30 MHz to FDL,low - ΔfOBUE and from FDL,high + ΔfOBUE up to 12750 MHz.

The frequency range for unwanted emissions measurement are described in [3] shown in the following table.

Table 1. Frequency range for unwanted emission measurements

| Fundamental frequency range | Frequency range for measurements |  |
| --- | --- | --- |
|  | Lower limit | Upper limit (The test should include the entire harmonic band and not be truncated at the precise upper frequency limit stated) |
| 9 kHz-100 MHz | 9 kHz | 1 GHz |
| 100 MHz-300 MHz | 9 kHz | 10th harmonic |
| 300 MHz-600 MHz | 30 MHz | 3 GHz |
| 600 MHz-5.2 GHz | 30 MHz | 5th harmonic |
| 5.2 GHz-13 GHz | 30 MHz | 26 GHz |
| 13 GHz-150 GHz | 30 MHz | 2nd harmonic |
| 150 GHz-300 GHz | 30 MHz | 300 GHz |

It can be see from the table, for FR1 NR BS, the 5th harmonic of the upper frequency of the DL operating band is covered by 12.75GHz if the upper frequency of the DL operating bands are below 2.55GHz.

The upper frequency limit for NR FR1 BS operating bands are all below 6GHz so far, which means the upper frequency up to 12.75GHz for NR FR1 BS RE test are not suitable for the operating bands upper frequency limit up to 6GHz. For some operating bands, the upper limit is higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in [3].

Proposal: Updated the testing frequency range for NR RE testing in the following table.

| Frequency Range | Minimum requirement (e.r.p.) / Reference bandwidth | NOTE |
| --- | --- | --- |
| 30 MHz ≤ f < 1000 MHz | -36 dBm/ 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 dBm/ 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the DL operating band in GHz | -30 dBm/ 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm/ 1 MHz | 2 |
| FDL,low - ΔfOBUE < f < FDL,high +ΔfOBUE | Not defined | 3,4 |
| NOTE 1: Applies for Band that the upper frequency edge of the DL Band which is between 2.55GHz and 5.2GHz. NOTE 2: Applies for Band that the upper frequency edge of the DL Band more than 5.2 GHz. NOTE 3: For BS capable of multi-band operation, the frequency ranges relating to the RF bandwidths of all supported operating bands apply. NOTE 4: ΔfOBUE is defined in subclause 6.6.1 of TS 38.104 [2]. |  |  |

Conclusion

In this contribution, we give the discussion of RE frequency range and the proposal are:

Proposal: Updated the testing frequency range for NR RE testing in the following table.

| Frequency Range | Minimum requirement (e.r.p.) / Reference bandwidth | NOTE |
| --- | --- | --- |
| 30 MHz ≤ f < 1000 MHz | -36 dBm/ 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 dBm/ 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the DL operating band in GHz | -30 dBm/ 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm/ 1 MHz | 2 |
| FDL,low - ΔfOBUE < f < FDL,high +ΔfOBUE | Not defined | 3,4 |
| NOTE 1: Applies for Band that the upper frequency edge of the DL Band is between 2.55 GHz and 5.2GHz. NOTE 2: Applies for Band that the upper frequency edge of the DL Band more than 5.2 GHz. NOTE 3: For BS capable of multi-band operation, the frequency ranges relating to the RF bandwidths of all supported operating bands apply. NOTE 4: ΔfOBUE is defined in subclause 6.6.1 of TS 38.104 [2]. |  |  |

- References

R4-1903561, discussion on frequency range for EMC RE test, ZTE

- TS 38.113, 3rd Generation Partnership Project;Technical Specification Group Radio Access Network;NR;Base Station (BS) ElectroMagnetic Compatibility (EMC)

ITU-R  SM.329-12, Unwanted emissions in the spurious domain