3GPP TSG-RAN4 Meeting #91	R4-1905605

# Reno, USA, 13th – 17th May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.133 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance draftCR on RLM scheduling restriction test cases in FR2 (A.5.5.1.9) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Test case needs maintenance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Maintenance work is done on the RLM scheduling restriction test case for FR2, - Correct AoA setup according to agreement - Correct RMC for PDCCH scheduling - Introduce Noc and other values - Correct editorial errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.5.5.1.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

A.5.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2

A.5.5.1.9.1	Test Purpose and Environment

The purpose is to verify that the NR UE correctly follows the RLM scheduling restrictions requirements defined in clause 8.1.7. This test verifies that the UE correctly receive the PDCCH scheduled on the symbols right before the RLM SSB symbols without overlap so that it sends ACK/NACK correctly, under the condition that the SSB is with different numerology as the PDCCH/PDSCH.

Two cells are deployed in the test, which are E-UTRAN PCell (Cell 1) and NR FR2 PSCell (Cell 2). The test parameters for NR PSCell are given in table A.5.5.1.9.1-1, table A.5.5.1.9.1-2 and table A.5.5.1.9.1-3 below and the parameters and applicability for the E-UTRAN cell are defined in A.3.7.2. The UE is required during time period T1 to transmit ACK/NACK correctly upon scheduling of PDSCH.

Table A.5.5.1.9.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FDD LTE, 120 kHz SSB SCS, 120 kHz RMC SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | TDD LTE, 120 kHz SSB SCS, 120 kHz RMC SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.5.5.1.9.1-2: General test parameters for RLM scheduling restriction test case in FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | 1 and 2 | 1 for NR PSCell and 2 for LTE PCell |
| SSB configuration |  | 1, 2 | SSB.1 FR2 |  |
| SMTC configuration |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length | s | 1, 2 | OFF |  |
| T1 | s | 1, 2 | 5 | During T1 the UE is required to correctly transmit ACK/NACK |

Table A.5.5.1.9.1-3: Cell specific test parameters for RLM scheduling restriction test case in FR2

| Parameter | Unit | Test configuration | Cell 2 |
| --- | --- | --- | --- |
|  |  |  | T1 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.3.1 |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.3.2 |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |
| PDCCH/PDSCH TCI state |  | 1, 2 | TCI.State.2 |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |
| RLM-RS |  | 1, 2 | SSB |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |
|  | dB | 1, 2 | 5 |
| Note2 | dBm/SCS | 1, 2 | -98 |
| Note2 | dBm/15 kHz | 1, 2 | -98 |
|  | dB | 1, 2 | 5 |
| SS-RSRP Note3 | dBm/SCS | 1, 2 | -93 |
| Io | dBm/95.04 MHz | 1, 2 | -62.82 |
| Propagation Condition |  | 1, 2 | AWGN |

A.5.5.1.9.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.1.7.3.

<End of Changes>