3GPP TSG-RAN4 WG4 Meeting #91	R4-1905663

# Reno, Nevada, USA, 13 - 17 May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | - | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR: Updates to FR1 conformance testing for PF3 and PF4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TBD still exist for some tests |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the conformance requirements for some tests Removed the square brackets for AWGN power level setting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will still be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.4, 8.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

8.3.4	Performance requirements for PUCCH format 3

8.3.4.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

The transient period as specified in TS 38.101-1 [21] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.2.

A test with or without additional DMRS configured is only applicable if the BS support it.

8.3.4.2	Minimum requirement

The minimum requirement is in TS 38.104 [2] subclause 8.3.5.

8.3.4.3	Test purpose

The test shall verify the receiver’s ability to detect UCI under multipath fading propagation conditions for a given SNR.

8.3.4.4	Method of test

8.3.4.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see subclause 4.9.1

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and BS type 1-H respectively.

8.3.4.4.2	Procedure

1)	Adjust the AWGN generator, according to the subcarrier spacing and channel bandwidth defined in table 8.3.4.4.2-1.

Table 8.3.4.4.2-1: AWGN power level at the BS input

| Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 | 5 | -83.5 dBm / 4.5MHz |
|  | 10 | -80.3 dBm / 9.36MHz |
|  | 20 | -77.2 dBm / 19.08MHz |
| 30 | 10 | -80.7 dBm / 8.64MHz |
|  | 20 | -77.4 dBm / 18.36MHz |
|  | 40 | -74.2 dBm / 38.16MHz |
|  | 100 | -70.1 dBm / 98.28MHz |

2)	The characteristics of the wanted signal shall be configured according to TS 38.211 [17]. The specific test parameters are configured as below:

Table 8.3.4.4.2-2: Test parameters

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation | QPSK |  |
| startingPRB | 0 |  |
| intraSlotFrequencyHopping | enabled |  |
| secondHopPRB | The largest PRB index - nrofPRBs |  |
| pucch-GroupHopping | neither |  |
| hoppingId | 0 |  |
| nrofPRBs | 1 | 3 |
| nrofSymbols | 14 | 4 |
| the number of UCI bits | 16 | 16 |
| startingSymbolIndex | 0 | 0 |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

4)	Adjust the equipment so that the SNR specified in table 8.3.4.5-1 or table 8.3.4.5-2 is achieved at the BS input during the UCI transmissions.

5)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.4.4.2-1. The following statistics are kept: the number of incorrectly decoded UCI.

Figure 8.3.4.4.2-1: Test signal pattern for PUCCH format 3 demodulation tests

8.3.4.5	Test requirement

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.4.5-1 and table 8.3.4.5-2.

Table 8.3.4.5-1: Required SNR for PUCCH format 3 with 15kHz SCS

| Test Number | Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (annex G) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [0.7] | [1.7] | [0.9] |
|  |  |  |  |  | Additional DM-RS | [0.4] | [1.1] | [0.4] |
|  |  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | [-3.2] | [-2.8] | [-3.2] |
|  |  |  |  |  | Additional DM-RS | [-3.9] | [-3.5] | [-3.9] |
|  |  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | [-6.4] | [-6.1] | [-6.2] |
|  |  |  |  |  | Additional DM-RS | [-7.2] | [-6.9] | [-7.1] |
| 2 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | [2.6] | [2.4] |
|  |  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | [-2.6] | [-1.9] | [-2.2] |
|  |  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | [-5.9] | [-5.4] | [-5.6] |

Table 8.3.4.5-2: Required SNR for PUCCH format 3 with 30kHz SCS

| Test Number | Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (annex G) | Additional DMRS configuration | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [1.6] | [1.3] | [1.2] | TBD |
|  |  |  |  |  | Additional DM-RS | [0.9] | [0.8] | [0.6] | [0.5] |
|  |  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | [-2.5] | [-2.8] | [-2.7] | [-2.9] |
|  |  |  |  |  | Additional DM-RS | [-3.2] | [-3.6] | [-3.5] | [-3.9] |
|  |  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | [-5.8] | [-6.0] | [-6.2] | [-6.1] |
|  |  |  |  |  | Additional DM-RS | [-6.8] | [-6.9] | [-7.0] | [-7.1] |
| 2 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [2.3] | [2.4] | [2.6] | TBD |
|  |  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | [-1.6] | TBD |
|  |  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | [-5.8] | TBD |

8.3.5	Performance requirements for PUCCH format 4

8.3.5.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

The transient period as specified in TS 38.101-1 [21] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.2.

A test with or without additional DMRS configured is only applicable if the BS support it.

8.3.5.2	Minimum requirement

The minimum requirement is in TS 38.104 [2] subclause 8.3.6.

8.3.5.3	Test purpose

The test shall verify the receiver’s ability to detect UCI under multipath fading propagation conditions for a given SNR.

8.3.5.4	Method of test

8.3.5.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see subclause 4.9.1

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and BS type 1-H respectively.

8.3.5.4.2	Procedure

1)	Adjust the AWGN generator, according to the subcarrier spacing and channel bandwidth defined in table 8.3.5.4.2-1.

Table 8.3.5.4.2-1: AWGN power level at the BS input

| Subcarrier spacing(SCS) (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 kHz | 5 | -83.5 dBm / 4.5MHz |
|  | 10 | -80.3 dBm / 9.36MHz |
|  | 20 | -77.2 dBm / 19.08MHz |
| 30 kHz | 10 | -80.7 dBm / 8.64MHz |
|  | 20 | -77.4 dBm / 18.36MHz |
|  | 40 | -74.2 dBm / 38.16MHz |
|  | 100 | -70.1 dBm / 98.28MHz |

2)	The characteristics of the wanted signal shall be configured according to TS 38.211 [17]. The test parameters are configured as below:

Table 8.3.5.4.2-2: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation | QPSK |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| pucch-GroupHopping | neither |
| hoppingId | 0 |
| nrofSymbols | 14 |
| the number of UCI bits | 22 |
| startingSymbolIndex | 0 |
| occ-Length | n2 |
| occ-Index | n0 |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

4)	Adjust the equipment so that the SNR specified in table 8.3.5.5-1 or table 8.3.5.5-2 is achieved at the BS input during the UCI transmissions.

5)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.5.4.2-1. The following statistics are kept: the number of incorrectly decoded UCI.

Figure 8.3.5.4.2-1: Test signal pattern for PUCCH format 4 demodulation tests

8.3.5.5	Test requirement

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.5.5-1 and table 8.3.5.5-2.

Table 8.3.5.5-1: Required SNR for PUCCH format 4 with 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (annex G) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [2.4] | [3.1] | [2.8] |
|  |  |  |  | Additional DM-RS | [2.2] | [2.9] | [2.4] |
|  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | [-1.9] | [-1.4] | [-1.7] |
|  |  |  |  | Additional DM-RS | [-2.4] | [-2.0] | [-2.2] |
|  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | [-5.1] | [-5.1] | [-5.0] |
|  |  |  |  | Additional DM-RS | [-5.9] | [-5.8] | [-5.7] |

Table 8.3.5.5-2: Required SNR for PUCCH format 4 with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (annex G) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [3.6] | [3.2] | [3.6] | [2.9] |
|  |  |  |  | Additional DM-RS | [3.3] | [2.9] | [3.6] | [2.4] |
|  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | [-1.0] | [-1.4] | [-1.2] | [-1.6] |
|  |  |  |  | Additional DM-RS | [-1.6] | [-1.9] | [-2.0] | [-2.0] |
|  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | [-4.8] | [-4.8] | [-4.9] | [-4.9] |
|  |  |  |  | Additional DM-RS | [-5.4] | [-5.5] | [-5.8] | [-5.8] |