3GPP TSG-RAN4 WG4 Meeting # 91	R4-1905670

# Reno, Nevada, USA, 13 - 17 May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5454 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: cleanup for LTE 8Rx DL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_8Rx_AP_DL-Perf |  |  |  |  |  | Date: |  |  | 2019-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The square brackets for SNR requirements exist at least two meeting cycles; The square brackets for DL UE categories that support 8 MIMO layers were removed from core spec TS 36.306 in R2-1904607. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the square brackets for SNR requirements and DL UE category that support 8 MIMO layers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for 8Rx DL will still be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15, 9.13, Table A.3.4.3.5-1, Table A.3.9.2-1A, Table A.3.9.2-3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

8.15	Demodulation (8 receiver antenna ports)

8.15.1	PDSCH

8.15.1.1	Void

8.15.1.2	TDD (Fixed Reference Channel)

The parameters specified in Table 8.15.1.2-1 are valid for all TDD tests unless otherwise stated.

Table 8.15.1.2-1: Common Test Parameters (TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Uplink downlink configuration (Note 1) |  | 1 |
| Special subframe configuration (Note 2) |  | 4 |
| Cyclic prefix |  | Normal |
| Cell ID |  | 0 |
| Inter-TTI Distance |  | 1 |
| Number of HARQ processes per component carrier | Processes | 7 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM and 256QAM |
| Number of OFDM symbols for PDCCH | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths unless otherwise stated |
| Cross carrier scheduling |  | Not configured |
| Precoder update granularity |  | Frequency domain: 1 PRG for Transmission modes 9 and 10 unless stated otherwise Time domain: 1 ms |
| ACK/NACK feedback mode |  | Multiplexing |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |

8.15.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.15.1.2.1-2, with the addition of the parameters in Table 8.15.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmitter antennas.

Table 8.15.1.2.1-1: Test Parameters for Transmit diversity Performance (FRC) with 8Rx Antenna Ports

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 2 |
| Note 1: |  |  |  |

Table 8.15.1.2.1-2: Minimum performance Transmit Diversity (FRC) with 8Rx Antenna Ports

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.11 TDD | OP.1 TDD | EVA5 | 2x8 Medium correlation B, ULA | 70 | 1.4 | ≥2 |

8.15.1.2.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.15.1.2.2-2, with the addition of the parameters in Table 8.15.1.2.2-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.15.1.2.2-1: Test Parameters for Large Delay CDD (FRC) with 8Rx Antenna Ports

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  | Bundling |
| PDSCH transmission mode |  |  | 3 |
| Note 1: |  |  |  |

Table 8.15.1.2.2-2: Minimum performance Large Delay CDD (FRC) with 8Rx Antenna Ports

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.11-1 TDD | OP.1 TDD | EVA70 | 2x8 Low | 70 | 4.1 | ≥2 |

8.15.1.2.3	8 Layer Spatial Multiplexing (User-Specific Reference Symbols)

The requirements are specified in Table 8.15.1.2.3-2, with the addition of the parameters in Table 8.15.1.2.3-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.15.1.2.3-1: Minimum performance for 8 Layer Spatial Multiplexing (User-Specific Reference Symbols)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Beamforming model |  |  | 8 layer precoding based on WB PMI feedback |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,22 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Precoding granularity |  |  | 50 |
| PMI delay |  |  | 10 or 11 |
| Reporting interval |  |  | 1 or 4 |
| Reporting mode |  |  | PUSCH 3-1 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | False |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 0010 0000 0000 0000 |
| Note 1: . Note 2: 50 resource blocks are allocated in sub-frames 4, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0, 1 and 6. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 4: Number of OFDM symbols for PDCCH: 2 |  |  |  |

Table 8.15.1.2.3-2: Minimum performance for for 8 Layer Spatial Multiplexing (User-Specific Reference Symbols)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category | DL UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |  |
| 1 | 10 MHz 16QAM | R.50-4 TDD | OP.1 TDD | EPA5 | 8x8 Low | 70 | 18.5 | 8 | 14, 17,18,19,20,22,23,24,25,26 |

8.15.2	CA

8.15.2.1	Void

8.15.2.2	TDD

The parameters specified in Table 8.15.2.2-1 are valid for all TDD CA and DC tests unless otherwise stated.

Table 8.15.2.2-1: Common Test Parameters (TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Uplink downlink configuration (Note 1) |  | 1 |
| Special subframe configuration (Note 2) |  | 4 |
| Cyclic prefix |  | Normal |
| Cell ID |  | 0 |
| Inter-TTI Distance |  | 1 |
| Number of HARQ processes per component carrier | Processes | 7 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM and 256QAM |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths unless otherwise stated |
| Cross carrier scheduling |  | Not configured |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |

8.15.2.2.1	Eight Layer Spatial Multiplexing (User-Specific Reference Symbols)

8.15.2.2.1.1	Minimum Requirement Eight-Layer Spatial Multiplexing 8 Tx Antenna Port

The purpose of these tests is to verify the closed loop rank-eight performance with frequency selective precoding with 8Tx and 8Rx under CA.

For CA with 2 DL CCs, the requirements are specified in Table 8.15.2.2.1.1-3, based on single carrier requirement specified in Table 8.15.2.2.1.1-2, with the addition of the parameters in Table 8.15.2.2.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.15.2.2.1.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| ownlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,22 |
| Beamforming model |  |  | Annex B.4.3(Note 4, 5) |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Precoding granularity |  |  | 50 |
| PMI delay |  |  | 8 |
| Reporting interval |  |  | 5 |
| Reporting mode |  |  | PUSCH 3-1 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | False |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 0010 0000 0000 0000 |
| Note 1: . Note 2: 50 resource blocks are allocated in sub-frames 1,2,3,4,6,7,8,9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |

Table 8.15.2.2.1.1-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.50-3 TDD | OP.1 TDD | EPA5 | 8x8 Low | 70 | 17.6 |
| 10MHz | R.50-4 TDD | OP.1 TDD | EPA5 | 8x8 Low | 70 | 18.5 |
| 15MHz | R.50-5 TDD | OP.1 TDD | EPA5 | 8x8 Low | 70 | 18.9 |
| 20MHz | R.50-6 TDD | OP.1 TDD | EPA5 | 8x8 Low | 70 | 18.9 |

Table 8.15.2.2.1.1-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category | DL UE category |
| --- | --- | --- | --- | --- |
| 1 | 2x20 MHz | As specified in Table 8.15.2.2.1.1-2 per CC | 8 | 14, 17,18,19,20,22,23,24,25,26 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.12.3. |  |  |  |  |

<Next Changes>

9.13	CSI reporting for 8Rx UE

9.13.1	CQI reporting definition under AWGN conditions

The reporting accuracy of the channel quality indicator (CQI) under frequency non-selective conditions is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI median. The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 36.213 [6]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

9.13.1.1	Minimum requirement PUCCH 1-1 with Rank 4 (CSI Reference Symbols)

The minimum requirements for dual codeword transmission are defined in terms of a reporting spread of the wideband CQI value for codeword #1, and their BLER performance using the transport format indicated by the reported CQI median of codeword #0 and codeword #1. The precoding used at the transmitter is a fixed precoding matrix specified by the bitmap parameter codebookSubsetRestriction. The propagation condition assumed for the minimum performance requirement is defined in subclause B.1.

9.13.1.2.1	Void

9.13.1.2.2	TDD

The following requirements apply to UE Category 8 and DL Category 14, 17~20, ≥22. For the parameters specified in table 9.13.1.2.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.13.1.2.2-1: PUCCH 1-1 static test 4x8 (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -6 |  |  |  |
|  |  | dB | -3 |  |  |  |
| CRS reference signals |  |  | Antenna ports 0, 1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…, 18 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 3 |  |  |  |
| CSI reference signal configuration |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (4x8 for Rank4) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0001 0000 0000 0000 |  |  |  |
| SNR (Note 2) |  | dB | 4 | 5 | 11 | 12 |
|  |  | dB[mW/15kHz] | -94 | -93 | -87 | -86 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type for CQI/ PMI |  |  | 2 |  |  |  |
| Physical channel for RI reporting |  |  | PUSCH |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| CQI delay |  | ms | 10 or 11 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| ri-ConfigIndex |  |  | 805 (Note 4) |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: Reference measurement channel RC.23 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#7 and #2. Note 4: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. |  |  |  |  |  |  |

<Next Changes>

Table A.3.4.3.5-1: Fixed Reference Channel for CDM-multiplexed DM RS with eight CSI-RS antenna ports

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.50 TDD | R.50-1 TDD | R.50-2 TDD | R.50-3 TDD | R.50-4 TDD | R.50-5 TDD | R.50-6 TDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 50 (Note 4) | 50 (Note 4) | 50 (Note 6) | 25 (Note 7) | 50 (Note 4) | 75 (Note 8) | 100 (Note 9) |
| Uplink-Downlink Configuration (Note 3) |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated subframes per Radio Frame (D+S) |  | 3+2 | 3+2 | 3+2 | 3+2 | 3+2 | 3+2 | 3+2 |
| Modulation |  | QPSK | QPSK | QPSK | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/2 | 1/2 | 1/2 | 1/2 |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 (non CSI-RS subframe) | Bits | 3624 | 3624 | 3624 | N/A | N/A | N/A | N/A |
| For Sub-Frames 4,9 (CSI-RS subframe) | Bits | 3624 | 3624 | 3624 | 17568 | 39232 | 61664 | 78704 |
| For Sub-Frames 1,6 |  | 2664 | 2664 | 3112 | 12216 | 26416 | 42368 | 61664 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 2984 | 2984 | 3368 | 13536 | 37888 | 57336 | 78704 |
| Number of Code Blocks per Sub-Frame (Note 5) |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 (non CSI-RS subframe) |  | 1 | 1 | 1 | N/A | N/A | N/A | N/A |
| For Sub-Frames 4,9 (CSI-RS subframe) |  | 1 | 1 | 1 | 3 | 7 | 11 | 13 |
| For Sub-Frames 1,6 |  | 1 | 1 | 1 | 2 | 5 | 7 | 11 |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | 1 | 1 | 1 | 3 | 7 | 10 | 13 |
| Binary Channel Bits Per Sub-Frame |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 (non CSI-RS subframe) | Bits | 12000 | 13200 | 13200 | N/A | N/A | N/A | N/A |
| For Sub-Frames 4,9 (CSI-RS subframe) | Bits | 10400 | 11600 | 11600 | 33600 | 76800 | 115200 | 153600 |
| For Sub-Frames 1,6 |  | 7872 | 7872 | 8448 | 22848 | 55104 | 84672 | 118272 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 9840 | 9840 | 10560 | 26112 | 70848 | 108864 | 152064 |
| Max. Throughput averaged over 1 frame | Mbps | 1.556 | 1.556 | 1.684 | 7.3104 | 16.9184 | 26.54 | 35.944 |
| UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | 8 | 8 | 8 | 8 |
| UE DL Category |  |  |  |  | 14, 17,18,19,20,22,23,24,25,26 | 14, 17,18,19,20,22,23,24,25,26 | 14, 17,18,19,20,22,23,24,25,26 | 14, 17,18,19,20,22,23,24,25,26 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: as per Table 4.2-2 in TS 36.211 [4]. Note 4: 50 resource blocks are allocated in sub-frames 4,9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1,6. Note 5: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 6: 50 resource blocks are allocated in sub-frames 4,9 and 47 resource blocks (RB0–RB23 and RB27–RB49) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1,6. In sub-frame 0 and the DwPTS portion of sub-frames 1, 6, PDSCH is rate matched around RB22, RB23 and RB27. Note 7: 25 resource blocks are allocated in sub-frames 4,9 and 17 resource blocks (RB0–RB9 and RB18–RB24) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1,6. Note 8: 75 resource blocks are allocated in sub-frames 4,9 and 63 resource blocks (RB0–R31 and RB44–RB74) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1,6. Note 9: 100 resource blocks are allocated in sub-frames 4,9 and 88 resource blocks (RB0–RB43 and RB56–RB99) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1,6. Note 10: Given per component carrier per codeword. |  |  |  |  |  |  |  |  |

The reference measurement channels in Table A.3.4.3.5-2 apply for verifying TDD PMI accuracy measurement with two CRS antenna ports and eight CSI-RS antenna ports.

<Next Changes>

Table A.3.9.2-1A: Fixed Reference Channel for sustained data-rate test (TDD 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.31-7 TDD | R.31-8 TDD | R.31-9 TDD | R.31-1A TDD | R.31-10 TDD | R.31-11 TDD | R.31-12 TDD |  |
| Channel bandwidth | MHz | 10 | 15 | 20 | 20 | 10 | 15 | 20 |  |
| Allocated resource blocks |  | Note 7 | Note 11 | Note 12 | Note 13 | Note 14 | Note 15 | Note 16 |  |
| Uplink-Downlink Configuration (Note 3) |  | 1 | 1 | 1 | 5 | 1 | 1 | 1 |  |
| Number of HARQ Processes per component carrier | Processes | 7 | 7 | 7 | 15 | 7 | 7 | 7 |  |
| Allocated subframes per Radio Frame (D+S) |  | 4 | 4 | 4 | 8+1 | 4 | 4 | 4 |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |  |
| Target Coding Rate |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 |  | 0.78 | 0.77 | 0.79 | 0.41 | 0.85 | 0.84 | 0.85 |  |
| For Sub-Frames 3,8 |  | N/A | N/A | N/A | 0.41 | N/A | N/A | N/A |  |
| For Sub-Frame 7 |  | N/A | N/A | N/A | 0.41 | N/A | N/A | N/A |  |
| For Sub-Frames 0 |  | 0.82 | 0.79 | 0.81 | 0.41 | 0.75 | 0.76 | 0.74 |  |
| For Sub-Frames 1 |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |  |
| For Sub-Frames 5 |  | 0.79 | 0.79 | 0.80 | 0.41 | 0.75 | 0.76 | 0.75 |  |
| For Sub-Frames 6 |  | N/A | N/A | N/A | 0.41 | N/A | N/A | N/A |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 63776 | 93800 | 128496 | 10296 | 110136 | 161760 | 220296 |  |
| For Sub-Frames 3,8 | Bits | 0 | 0 | 0 | 10296 | N/A | N/A | N/A |  |
| For Sub-Frame 7 | Bits | 0 | 0 | 0 | 10296 | N/A | N/A | N/A |  |
| For Sub-Frame 0 | Bits | 63776 | 93800 | 128496 | 10296 | 87936 | 137792 | 187712 |  |
| For Sub-Frame 1 | Bits | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| For Sub-Frame 5 | Bits | 59256 | 90816 | 124464 | 10296 | 81176 | 128496 | 181656 |  |
| For Sub-Frame 6 | Bits | 0 | 0 | 0 | 10296 | 0 | 0 | 0 |  |
| Number of Code Blocks per Sub-Frame (Note 4) |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 |  | 11 | 16 | 21 | 2 | 18 | 27 | 36 |  |
| For Sub-Frames 3,8 |  | N/A | N/A | N/A | 2 | N/A | N/A | N/A |  |
| For Sub-Frame 7 |  | N/A | N/A | N/A | 2 | N/A | N/A | N/A |  |
| For Sub-Frame 0 |  | 11 | 16 | 21 | 2 | 15 | 23 | 31 |  |
| For Sub-Frame 1 |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |  |
| For Sub-Frame 5 |  | 10 | 15 | 21 | 2 | 14 | 21 | 30 |  |
| For Sub-Frame 6 | Bits | N/A | N/A | N/A | 2 | N/A | N/A | N/A |  |
| Binary Channel Bits Per Sub-Frame |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 81600 | 122400 | 163200 | 25200 | 129600 | 194400 | 259200 |  |
| For Sub-Frames 3,8 | Bits | 0 | 0 | 0 | 25200 | 0 | 0 | 0 |  |
| For Sub-Frame 7 | Bits | 0 | 0 | 0 | 25200 | 0 | 0 | 0 |  |
| For Sub-Frame 0 | Bits | 77856 | 118656 | 159456 | 25200 | 118080 | 181440 | 253440 |  |
| For Sub-Frame 1 | Bits | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| For Sub-Frame 5 | Bits | 75840 | 115008 | 155808 | 25200 | 109440 | 169920 | 241920 |  |
| For Sub-Frame 6 | Bits | 0 | 0 | 0 | 25200 | 0 | 0 | 0 |  |
| Number of layers |  | 4 | 4 | 4 | 1 | 8 | 8 | 8 |  |
| Max. Throughput averaged over 1 frame (Note 10) | Mbps | 25.058 | 37.222 | 50.996 | 8.237 | 77.877 | 117.962 | 161.992 |  |
| UE Category |  | ≥ 6 | ≥ 6 | ≥ 6 | ≥ 1 | 8 | 8 | 8 |  |
| UE DL Category |  |  |  |  |  | 14, 17,18,19,20,22,23,24,25,26 | 14, 17,18,19,20,22,23,24,25,26 | 14, 17,18,19,20,22,23,24,25,26 |  |
| Note 1: 1 symbol allocated to PDCCH for all tests. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: As per Table 4.2-2 in TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: The first RBG, i.e. resource blocks nPRB = 0..2 for 10MHz channel bandwidth, nPRB = 0..3 for 15MHz and 20MHz channel bandwidths are allocated for SIB transmissions in sub-frame 5. Note 6: Void. Note 7: Resource blocks nPRB = 3..49 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..49 in other available downlink sub-frames according to uplink downlink configurations used . Note 8: Void. Note 9: Void. Note10: Given per component carrier per codeword. Note11: Resource blocks nPRB = 4..74 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..74 in other downlink sub-frames. Note 12: Resource blocks nPRB = 4..99 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..99 in other downlink sub-frames. Note 13: Resource blocks nPRB = 8..35 are allocated for the user data in all sub-frames. Note 14: 84 resource blocks nPRB = 4..43 and nPRB = 56..99 are allocated for the user data in sub-frame 5, and 88 resource blocks nPRB = 0..43 and nPRB = 56..99 are allocated for the user data in sub-frame 0,and 100 resource blocks nPRB = 0..99 in other downlink sub-frames. Note 15: 59 resource blocks nPRB = 4..31 and nPRB = 44...74 are allocated for the user data in sub-frame 5, and 63 resource blocks nPRB = 0..31 and nPRB = 44..74 are allocated for the user data in sub-frame 0, and 75 resource blocks nPRB = 0..74 in other downlink sub-frames. Note 16: 38 resource blocks nPRB = 3..20 and nPRB = 30..49 are allocated for the user data in sub-frame 5, and 41 resource blocks nPRB = 0..20 and nPRB = 30..49 are allocated for the user data in sub-frame 0, and 50 resource blocks nPRB = 0..49 in other available downlink sub-frames according to uplink downlink configurations used. |  |  |  |  |  |  |  |  |  |

<Next Changes>

Table A.3.9.2-3: Fixed Reference Channel for sustained data-rate test (TDD 256QAM)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.68-5 TDD | R.68-6 TDD | R.68-7 TDD | R.68-8 TDD | R.68-9 TDD | R.68-10 TDD |
| Channel bandwidth | MHz | 10 | 15 | 20 | 10 | 15 | 20 |
| Allocated resource blocks | PRB | Note 8 | Note 7 | Note 6 | Note 10 | Note 11 | Note 12 |
| Uplink-Downlink Configuration (Note 3) |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of HARQ Processes per component carrier | Processes | 7 | 7 | 7 | 7 | 7 | 7 |
| Allocated subframes per Radio Frame (D+S) |  | 4+2 | 4+2 | 4+2 | 4 | 4 | 4 |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Target Coding Rate |  |  |  |  |  |  |  |
| For Sub-Frame 0 |  | 0.82 | 0.82 | 0.80 | 0.70 | 0.70 | 0.70 |
| For Sub-Frame 1 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frames 3 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frames 4 |  | 0.78 | 0.79 | 0.78 | 0.77 | 0.76 | 0.77 |
| For Sub-Frame 5 |  | 0.81 | 0.82 | 0.78 | 0.70 | 0.70 | 0.69 |
| For Sub-Frame 6 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 7 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frames 8 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frames 9 |  | 0.78 | 0.79 | 0.78 | 0.77 | 0.76 | 0.77 |
| Information Bit Payload |  |  |  |  |  |  |  |
| For Sub-Frame 0 | Bits | 84760 | 128496 | 169544 | 110136 | 169544 | 236160 |
| For Sub-Frame 1 | Bits | 0 | 0 | 0 | 0 | 0 | 0 |
| For Sub-Frames 3 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frames 4 | Bits | 84760 | 128496 | 169544 | 133208 | 195816 | 266440 |
| For Sub-Frame 5 | Bits | 81176 | 124464 | 161760 | 101840 | 157432 | 220296 |
| For Sub-Frame 6 | Bits | 0 | 0 | 0 | 0 | 0 | 0 |
| For Sub-Frame 7 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frames 8 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frames 9 | Bits | 84760 | 128496 | 169544 | 133208 | 195816 | 266440 |
| Number of Code Blocks per Sub-Frame (Note 4) |  |  |  |  |  |  |  |
| For Sub-Frame 0 |  | 14 | 21 | 28 | 18 | 28 | 39 |
| For Sub-Frame 1 |  | 0 | 0 | 0 | 0 | 0 | 0 |
| For Sub-Frames 3 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frames 4 |  | 14 | 21 | 28 | 22 | 32 | 44 |
| For Sub-Frame 5 |  | 14 | 21 | 27 | 17 | 26 | 36 |
| For Sub-Frame 6 |  | 0 | 0 | 0 | 0 | 0 | 0 |
| For Sub-Frame 7 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frames 8 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frames 9 |  | 14 | 21 | 28 | 22 | 32 | 44 |
| Binary Channel Bits Per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 0 | Bits | 103808 | 158208 | 212608 | 157440 | 241920 | 337920 |
| For Sub-Frame 1 | Bits | 0 | 0 | 0 | 0 | 0 | 0 |
| For Sub-Frames 3 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frames 4 | Bits | 108800 | 163200 | 217600 | 172800 | 259200 | 345600 |
| For Sub-Frame 5 |  | 101120 | 153344 | 207744 | 145920 | 226560 | 322560 |
| For Sub-Frame 6 | Bits | 0 | 0 | 0 | 0 | 0 | 0 |
| For Sub-Frame 7 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frames 8 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frames 9 | Bits | 108800 | 163200 | 217600 | 172800 | 259200 | 345600 |
| Number of layers |  | 4 | 4 | 4 | 8 | 8 | 8 |
| Max. Throughput averaged over 1 frame (Note 5) | Mbps | 33.546 | 50.995 | 67.039 | 95.678 | 143.722 | 197.867 |
| UE Categories |  | 11-12 | 11-12 | 11-12 | 8 | 8 | 8 |
| UE DL Categories |  | 13-14 | 13-14 | 13-14 | 14, 17,18,19,20,22,23,24,25,26 | 14, 17,18,19,20,22,23,24,25,26 | 14, 17,18,19,20,22,23,24,25,26 |
| Note 1: 1 symbol allocated to PDCCH for all tests. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: As per Table 4.2-2 in TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Given per component carrier per codeword. Note 6: Resource blocks nPRB = 4..99 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..99 in other downlink sub-frames. Note 7: Resource blocks nPRB = 4..74 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..74 in other downlink sub-frames. Note 8: Resource blocks nPRB = 3..49 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..49 in the available downlink sub-frames according to uplink downlink configurations used. Note 9: The first RBG, i.e. resource blocks nPRB = 0..2 for 10MHz channel bandwidth, nPRB = 0..3 for 15MHz and 20MHz channel bandwidths are allocated for SIB transmissions in sub-frame 5. Note 10: 84 resource blocks nPRB = 4..43 and nPRB = 56..99 are allocated for the user data in sub-frame 5, and 88 resource blocks nPRB = 0..43 and nPRB = 56..99 are allocated for the user data in sub-frame 0,and 100 resource blocks nPRB = 0..99 in other downlink sub-frames. Note 11: 59 resource blocks nPRB = 4..31 and nPRB = 44...74 are allocated for the user data in sub-frame 5, and 63 resource blocks nPRB = 0..31 and nPRB = 44..74 are allocated for the user data in sub-frame 0, and 75 resource blocks nPRB = 0..74 in other downlink sub-frames. Note 12: 38 resource blocks nPRB = 3..20 and nPRB = 30..49 are allocated for the user data in sub-frame 5, and 41 resource blocks nPRB = 0..20 and nPRB = 30..49 are allocated for the user data in sub-frame 0, and 50 resource blocks nPRB = 0..49 in other available downlink sub-frames according to uplink downlink configurations used. |  |  |  |  |  |  |  |

<End of the Changes>