3GPP TSG RAN WG4 Meeting #91		　　　　　　　　　　　　　　　　R4-1905684

Reno, Nevada, 13th– 17th May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | CRNum | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR of modification on reference for inter-band EN-DC including FR2 for TS 38.101-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some Tx requirement in the current -3 spec does not refer to the requirement of FR2 UL intra-band contiguous CA in TS 38.101-1, as pointed out in LS from RAN5: R4-1905318. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial modification on reference to accomodate FR2 CA requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NSA requirements are not correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Many |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections are omitted>

6.2B.1.4	Inter-band EN-DC including FR2

UE maximum output power requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.2.2 and 6.2.2A of [4] and for NR single carrier and CA operation specified in sub-clause 6.2.1 and 6.2A.1 of [3] apply.

6.2B.1.5	Inter-band EN-DC including both FR1 and FR2

UE maximum output power requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.2.2 and 6.2.2A of [4] and for NR single carrier and CA operation specified in sub-clause 6.2.1 of [2] and sub-clause 6.2.1 and 6.2A.1 of [3] apply.

<Unchanged sections are omitted>

6.2B.2.4	Inter-band EN-DC including FR2

UE maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.2.3 and 6.2.3A of [4] and for NR single carrier and CA operation specified in sub-clause 6.2.2 and 6.2A.2 of [3] apply.

6.2B.2.5	Inter-band EN-DC including both FR1 and FR2

UE maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.2.3 and 6.2.3A of [4] and for NR single carrier and CA operation specified in sub-clause 6.2.2 of [2] and sub-clause 6.2.2 and 6.2A.2 of [3] apply.

<Unchanged sections are omitted>

6.2B.3.4	Inter-band EN-DC including FR2

UE additional maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.2.4 and 6.2.4A of [4] and for NR single carrier and CA operation specified in sub-clause 6.2.3 and 6.2A.3 of [3] apply.

6.2B.3.5	Inter-band EN-DC including both FR1 and FR2

UE additional maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.2.4 and 6.2.4A of [4] and for NR single carrier and CA operation specified in sub-clause 6.2.3 of [2] and sub-clause 6.2.3 and 6.2A.3 of [3] apply.

<Unchanged sections are omitted>

6.3B	Output power dynamics for DC

6.3B.0	General

The E-UTRA and NR switching time mask defines the observation period between E-UTRA subframe and NR slot/mini-slot boundary. Both E-UTRA subframe and NR slot/mini-slot have ON power transmissions. The ON power is defined as the mean power over the symbol duration excluding any transient period. For E-UTRA subframe or NR slot/mini-slot having OFF power transmission, the general time mask for E-UTRA or NR shall apply.

Foter Inter-band EN-DC, output power dynamics requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.3 of [4] and for NR single carrier and CA operation specified in sub-clause 6.3 of [2] and sub-clause 6.3 and 6.3A of [3] apply.

<Unchanged sections are omitted>

#### 6.4B.1.3	Frequency error for inter-band EN-DC within FR1

For inter-band EN-DC with uplink assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.1 in [4] and in clause 6.4.1 in [2], respectively, with all component carriers active. If multiple component carriers are assigned to one E-UTRA band, the requirements in subclauses 6.5.1A in [4] apply for those component carriers.

#### 6.4B.1.4	Frequency error for inter-band EN-DC including FR2

Frequency error requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.5.1 and 6.5.1A of [4] and for NR single carrier and CA operation specified in sub-clause 6.4.1 and 6.4A.1 of [3] apply.

#### 6.4B.1.5	Frequency error for inter-band EN-DC including both FR1 and FR2

Frequency error requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.5.1 and 6.5.1A of [4] and for NR single carrier and CA operation specified in sub-clause 6.4.1 of [2] and sub-clause 6.4.1 and 6.4A.1 of [3] apply.

<Unchanged sections are omitted>

#### 6.4B.2.3	Transmit modulation quality for Inter-band EN-DC within FR1

For inter-band EN-DC with uplink assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.2 in [4] and in clause 6.4.2 in [2], respectively, with all component carriers active. If multiple component carriers are assigned to one E-UTRA band, the requirements in subclauses 6.5.2A in [4] apply for those component carriers.

#### 6.4B.2.4	Transmit modulation quality for Inter-band EN-DC including FR2

Transmit modulation quality requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.5.2 and 6.5.2A of [4] and for NR single carrier and CA operation specified in sub-clause 6.4.2 and 6.4A.2 of [3] apply.

#### 6.4B.2.5	Transmit modulation quality for inter-band EN-DC including both FR1 and FR2

Transmit modulation quality requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.5.2 and 6.5.2A of [4] and for NR single carrier and CA operation specified in sub-clause 6.4.2 of [2] and sub-clause 6.4.2 and 6.4A.2 of [3] apply.

<Unchanged sections are omitted>

6.5B	Output RF spectrum emissions for DC

6.5B.1	Occupied bandwidth for EN-DC

6.5B.1.1	Intra-band contiguous EN-DC

For intra-band contiguous EN-DC the occupied bandwidth is a measure of the bandwidth containing 99% of the total integrated power of the transmitted spectrum.  The OBW shall be less than the aggregated channel bandwidth for EN-DC, denoted as ENBW in sub-clause 5.3B.

6.5B.1.2	Intra-band non-contiguous EN-DC

6.5B.1.3	Inter-band EN-DC within FR1

Occupied bandwidth requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.6.1 and 6.6.1A of [4] and for NR single carrier and CA operation specified in sub-clause 6.5.1 of [2] apply.

6.5B.1.4	Inter-band EN-DC including FR2

Occupied bandwidth requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.6.1 and 6.6.1A of [4] and for NR single carrier and CA operation specified in sub-clause 6.5.1 and 6.5A.1 of [3] apply.

6.5B.1.5	Inter-band EN-DC including both FR1 and FR2

Occupied bandwidth requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.6.1 and 6.6.1A of [4] and for NR single carrier and CA operation specified in sub-clause 6.5.1 of [2] and sub-clause 6.5.1 and 6.5A.1 of [3] apply.

<Unchanged sections are omitted>

6.5B.2.4	Inter-band EN-DC including FR2

Unless otherewise stated, Out-of-band emissions requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.6.2 of [4] and for NR single carrier and CA operation specified in sub-clause 6.5.2 and 6.5A.2 of [3] apply.

6.5B.2.5	Inter-band EN-DC including both FR1 and FR2

Unless otherewise stated, Out-of-band emissions requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.6.2 of [4] and for NR single carrier and CA operation specified in sub-clause 6.5.2 of [2] and sub-clause 6.5.2 and 6.5A.2 of [3] apply.

<Unchanged sections are omitted>

6.5B.3.4	Inter-band EN-DC including FR2

General spurious requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.6.3.1 and 6.6.3.1A of [4] and for NR single carrier and CA operation specified in sub-clause 6.5.3 and 6.5A.3 of [3] apply.

6.5B.3.4.1	Spurious emission band UE co-existence

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.2B.5.1-1, no requirements for FR2 NR bands to protect E-UTRA and FR1 NR bands are applied to the constituent FR2 NR bands. Spurious emission band UE co-existence requirements specified in TS 36.101 [4] are applied to the constituent E-UTRA bands for the EN-DC configuration.

Spurious emission band UE co-existence requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.6.3.2 and 6.6.3.2A of [4] and for NR single carrier and CA operation specified in sub-clause 6.5.3.1 and 6.5A.3.1 of [3] apply.

Table 6.5B.3.4.1-1: Requirements

(Void)

6.5B.3.5	Inter-band EN-DC including both FR1 and FR2

General spurious requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.6.3.1 and 6.6.3.1A of [4] and for NR single carrier and CA operation specified in sub-clause 6.5.3.1 of [2] and sub-clause 6.5.3 and 6.5A.3 of [3] apply.

6.5B.3.5.1	Spurious emission band UE co-existence

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. Unless otherwise stated, for inter-band EN-DC configurations defined in section 5.2B.6, no requirements for FR2 NR bands to protect E-UTRA and FR1 NR bands are applied to the constituent FR2 NR bands. Spurious emission band UE co-existence requirements for constituent E-UTRA and FR1 NR bands for the inter-band EN-DC are the same as those for the corresponding EN-DC configuration without the FR2 bands specified in 6.5B.3.2.2.

Spurious emission band UE co-existence requirement for E-UTRA single carrier and CA operation specified in sub-clauses 6.6.3.2 and 6.6.3.2A of [4] and for NR single carrier and CA operation specified in sub-clause 6.5.3.2 of [2] and sub-clause 6.5.3.1 and 6.5A.3.1 of [3] apply.

Table 6.5B.3.5.1-1: Requirements

(Void)

<Unchanged sections are omitted>

6.5B.5.4	Inter-band EN-DC including FR2

Transmit intermodulation requirements specified in sub-clause 6.7.1 and 6.7.1A of [4] apply for each component carrier in E-UTRA bands.

6.5B.5.5	Inter-band EN-DC including both FR1 and FR2

Transmit intermodulation requirement specified in sub-clauses 6.7.1 and 6.7.1A of [4] and sub-clauses 6.5.4 and 6.5A.4 of [2] apply for each component carrier in E-UTRA bands and NR bands, respectively.

<Unchanged sections are omitted>