3GPP TSG-RAN WG4 Meeting #91 	R4-1905720

Reno, US, 13th– 17th May, 2019

Agenda item:	6.10.7.5.1

Source:	Intel Corporation

Title:	On the requirement for RRC based BWP switching

Document for:	Discussion

# Introduction

After RAN4 #90bis meeting, there are some remaining issues on RRC based BWP switching, which are listed as follows:

- FFS on the RRC-based BWP switch delay requirement

In this contribution we provide our view on these remaining issues for RRC based BWP switching.

# RRC based BWP switching delay

In CR [2], the RRC based BWP switching is defined as follows:

### 8.6.3 RRC based BWP switch delay For RRC-based BWP switch, after the UE receives BWP switching request, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs on the first DL or UL slot right after the beginning of DL slot n+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length, where DL slot n is the last slot containing the RRC command, and TRRCprocessingDelay is the length of the RRC procedure delay in millisecond as defined in clause 12 in TS 38.331 [2], and TBWPswitchDelayRRC=[5 ~ 8]ms is the time used by the UE to perform BWP switch. The UE is not required to transmit UL signals or receive DL signals during the time defined by TRRCprocessingDelay+TBWPswitchDelayRRC on the cell where RRC-based BWP switch occurs.

The RRC based BWP switching delay TBWPswitchDelayRRC includes the following components:

- Time for passing RRC configuration from upper layer to RF/baseband controller

- Time for preparing for RF/baseband reconfiguration (software overhead)

- Time for RF tuning (applying new RF/baseband parameters)

For component (a), it takes about 3ms for current UE implementation. For (b)+(c), the delay time can be referred to DCI-based BWP switching, which is roughly (2ms – DCI decoding time) for Type 2 delay according to TS38.133. Thus, it is proposed that the delay time for (a)+(b)+(c) is 5ms.

Thus, it is proposed:

Proposal 1: The RRC based BWP switch delay TBWPswitchDelayRRC shall be 5ms.

# Conclusion

In this contribution, the remaining issues on RRC based BWP switching are discussed and the proposals are listed as follows,

Proposal 1: The RRC based BWP switch delay TBWPswitchDelayRRC shall be 5ms.

# References

[1] R4-1904701, “draftCR on BWP switch delay (section 8.6)”, Huawei, HiSilicon, RAN4#90bis