3GPP TSG-RAN WG4 Meeting #91	R4-1905721

Reno, US, 13th– 17th May, 2019

Title:	Reply LS on RRC processing delay for BWP switching

Reply To:	R2-1818901

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG 4

To:	RAN WG 2

Cc:

Contact Person:

Name:	Jie Cui

E-mail Address:	jie.cui@intel.com

Attachments:	N.A.

1. Overall Description:

In LS R2-1818901, RAN2 asks RAN4 whether the BWP switching delays defined for DCI-based BWP switching also apply to the cases of RRC signalling based BWP switch and BWP switching by MAC entity upon initiation of random access procedure.

According to above question, RAN4 provides the following answers:

- The delay requirement for RRC based BWP switching is different from DCI-based BWP switching delay requirement. This new defined delay requirement will be captured in TS38.133 sec 8.6.3, duplicated as below,

For RRC-based BWP switch, after the UE receives BWP switching request, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs on the first DL or UL slot right after the beginning of DL slot n+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length, where DL slot n is the last slot containing the RRC command, and TRRCprocessingDelay is the length of the RRC procedure delay in millisecond as defined in clause 12 in TS 38.331 [2], and TBWPswitchDelayRRC=5 ms is the time used by the UE to perform BWP switch.

- In RAN4 meeting #90, it is approved that no RRM requirement will be specified for RACH-based BWP switch.

2. Actions:

To RAN WG1, WG2 group.

ACTION: 	RAN4 kindly asks RAN2 to take the above information into account.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #92		Aug 26 – 30, 2019	     	Ljubljana, Slovenia

TSG-RAN WG4 Meeting #92bis		Oct 14 – 18, 2019	     	China