3GPP TSG-RAN WG4 Meeting #91	R4-1905725

Reno, US, 13th– 17th May, 2019

Agenda item:	7.15.3

Source:	Intel Corporation

Title:	Performance evaluation for measurement accuracy requirement in LTE high speed scenario

Document for:	Discussion

# Introduction

In this contribution, the link level simulation results are provided for RSRP measurement in LTE high speed scenario. The purpose of the simulation is to validate whether the existing RSRP/RSRQ requirement is applicable in high speed scenario.

# Simulation assumption

The simulation assumptions for RSRP measurement are given in Table 1and Table 2.

Table 1: Simulation parameters for measurement

| Parameters | Value | Comments |
| --- | --- | --- |
| Measurement bandwidth | 6 resource blocks | Both RSRP and RSSI measured over 6 RB |
| System bandwidth | 6 resource blocks |  |
| L1 measurement period | 200 ms |  |
| Measurement sampling rate | - | Implementation dependent |
| L3 filtering | disabled |  |
| Transmit antenna | 1 |  |
| Receive antennas | 2 | The receive diversity rule as defined in TS 36.214. Both antennas with equal gain, no correlation between them. |
| DRX/DTX | OFF | DRX/DTX to be considered at later stage |
| Propagation conditions | AWGN, EVA1100Hz, HST1100Hz, HST-SFN, defined in Table 2 | AWGN for alignment purposes |
| CP Length | Normal |  |
| Ês/Iot | [-6 -3] (dB) |  |

Table 2: Parameters for high speed train conditions

| Parameter | Value |
| --- | --- |
|  | 1000 m |
|  | 5 m |
|  | 500 km/h |
| Frequency band | 2.38 GHz |
|  | 1100 Hz |

# Simulation results

Table 3 AWGN simulation results

| SNR (dB) | AWGN RSRP accuracy |  |  |  | AWGN RSRQ accuracy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5% | 50% | 95% | relative | 5% | 50% | 95% |
| -6 | 0.49 | 1.85 | 2.93 | 1.36 | 0.53 | 1.82 | 2.9 |
| -3 | 0.105 | 1.01 | 1.82 | 0.905 | 0.07 | 1.03 | 1.83 |

Table 3 EVA-1100Hz simulation results

| SNR (dB) | EVA1100 RSRP accuracy |  |  |  | EVA1100 RSRQ accuracy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5% | 50% | 95% | relative | 5% | 50% | 95% |
| -6 | -2.54 | 0.23 | 3.16 | 2.93 | -2.38 | 0.26 | 3.17 |
| -3 | -3.93 | -1.14 | 1.9 | 3.04 | -3.86 | -1.13 | 1.88 |

Table 4 HST-1100Hz simulation results

| SNR (dB) | HST1100 RSRP accuracy |  |  |  | HST1100 RSRQ accuracy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5% | 50% | 95% | relative | 5% | 50% | 95% |
| -6 | 0.59 | 1.81 | 2.93 | 1.22 | 0.66 | 1.84 | 2.87 |
| -3 | 0.037 | 1.04 | 1.88 | 1 | 0.17 | 1.09 | 1.82 |

Table 5 HST-SFN

| SNR (dB) | HST-SFN 1100Hz RSRP accuracy |  |  |  | HST-SFN 1100Hz RSRQ accuracy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5% | 50% | 95% | relative | 5% | 50% | 95% |
| -6 | 0.59 | 1.92 | 3.05 | 1.33 | 0.69 | 1.93 | 3.01 |
| -3 | 0.15 | 1.11 | 1.95 | 0.96 | 0.26 | 1.15 | 1.94 |

In the above tables, the relative accuracy is defined to be max(|5% accuracy -50% accuracy|, |95% accuracy -5% accuracy | ).

Proposal 1: The measurement accuracy requirement for RSRP/RSRQ defined for high Doppler condition in TS36.133 v16.1.0 could be reused for the LTE R16 high speed scenarios. No further enhancement is needed.

# Conclusion

This contribution shows the simulation results for RSRP and RSRQ measurement for high speed scenarios. The proposal is drawn as follows:

Proposal 1: The measurement accuracy requirement for RSRP/RSRQ defined for high Doppler condition in TS36.133 v16.1.0 could be reused for the LTE R16 high speed scenarios. No further enhancement is needed.

# Reference

[1] R4-154775, “Measurement accuracy simulation results for existing high speed scenarios”, Huawei, RAN4 Meeting

#76