3GPP TSG-RAN WG4 Meeting #91                                                                          R4-1905743

Reno, US, 13 - 17 May, 2019

Agenda Item:	7.12.3

Source:	Intel Corporation

Title:	Simulation results of RSS for Rel-16 additional MTC enhancements for LTE

Document for:	Discussion

# Introduction

In last meeting, there is agreed updated simulation assumptions for RSS based RRM measurements for Rel-16 MTC[1]. In this contribution, we will provide our simulation results.

# Simulation results

Simulation assumption is listed in Table 1.

Table 1: Simulation parameters for Rel-16 MTC RSRP measurement studies using RSS

| Parameters | Value | Comments |
| --- | --- | --- |
| RSS bandwidth | 2 consecutive resource blocks |  |
| Sample duration | {1, 2} subframes | Implementation dependent (NOTE 4) |
| RSS periodicity | 160 ms | Option 1: non-colliding RSS Option 2: Colliding RSS, |
| Ratio of RSS RE energy to CRS RE energy | 0 dB |  |
| Power level of neighbour cell relative to serving cell | {0, -3} dB |  |
| Measurement bandwidth | 2 resource blocks | RSRP measured over the 2 RSS PRBs. |
| System bandwidth | 6 resource blocks |  |
| L1 measurement period | Single shot, 480 ms, 800 ms | Single shot measurement does not include any time-domain averaging over multiple samples |
| Measurement sampling rate |  | Implementation dependent (NOTE 1) |
| L3 filtering | Disabled |  |
| Transmit antenna | 1 |  |
| Receive antennas | 1, 2 | Single and double Rx branches, respectively |
| Mobility | Stationary UEs, mobile UEs |  |
| Propagation conditions | AWGN, ETU and EPA |  |
| Doppler Frequency for stationary UEs: ETU and EPA | 1 Hz and 1 Hz, respectively |  |
| Doppler Frequency for mobile UEs: ETU and EPA | 30 Hz and 5 Hz, respectively |  |
| Channel estimation techniques | Measurement averaging techniques used currently for category M1 | Implementation dependent (NOTE 2) |
| Number of RSS subframes used for measurement |  | Implementation dependent (NOTE 3) |
| CP length | Normal |  |
| Carrier frequency | 2 GHz |  |
| Ec/Iot | -15 dB, …, 5 dB | AWGN noise |
| NOTE 1: Companies are requested to provide the details of the measurement sampling rate for interpretation and comparison of the results NOTE 2: Companies are requested to provide the details of the RS averaging techniques for interpretation and comparison of the results. NOTE 3: The number of RSS subframes used for measurement is implementation dependent, and can range from 1 to the RSS duration. Companies are requested to provide the details of the RSS subframes used for interpretation and comparison of the results. Note 4: The presented results shall be based on at least 10 000 RSS instances. |  |  |

Here, we assume that the RSS is coherent combining during the RSS duration. For example, if the RSS during is 2ms, we will jointly use the RSS during the 2ms for coherent combining and this is called single slot. We also provide simulation results where 3 and 5 samples are averaged. The simulation results with 2 RX are provided in following table without any implementation margin.

Fig.1 RSRP delta for EPA1 with 2ms

Fig.2 RSRP delta for ETU1 with 2ms

Fig.3 RSRP delta for EPA5 with 2ms

Fig.4 RSRP delta for ETU30 with 2ms

From 36.133, the RSRP accuracy requirement for non-BL UE with CE mode A and CE mode B are:

Table 9.1.26.1-1: RSRP Intra frequency absolute accuracy for non-BL CE UE with CE mode A for FDD and TDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 5.5 | 8.5 | -6 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| 8 | 11 | -6 dB | FDD-M1_A, TDD-M1_A, FDD-M1_D, FDD-M1_E, TDD-M1_E, FDD-M1_F, FDD-M1_G, FDD-M1_N | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table 9.1.26.3-1: RSRP Intra frequency absolute accuracy for non-BL CE UE with CE mode B for FDD and TDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 8 | 11 | -15≤Ês/Iot≤-12 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
| [6] | [9] | -12 dB | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| 10 | 13 | -15≤Ês/Iot≤-12 dB | FDD-M1_A, TDD-M1_A, FDD-M1_D, FDD-M1_E, TDD-M1_E, FDD-M1_F, FDD-M1_G, FDD-M1_N | N/A | -70 | -50 |
| [8] | [11] | -12 dB |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

The RSRP accuracy requirement is 5.5dB for CE mode A where SNR-6 dB. The RSRP accuracy requirement are 6dB and 8dB for CE mode B where SNR-12 dB and 15≤SNR≤-12 dB respectively.

From Fig.1~Fig.4, when 2.5dB margin is considered, it can be seem that for CE mode A, single sample can satisfy the accuracy requirement for SNR=-6dB where RSRP delta is 1.8+2.5=4.3dB.

For CE mode B, when 3 samples are used, RSRP accuracy is 2.5+2.5 = 5dB for SNR=-12 dB and 5+2.5=7.5dB for SNR=-15 dB.

Therefore, 3 samples can satisfy the RSRP requirement for SNR=-12dB and SNR=-15dB respectively.

Observation 1: For non-BL CE UE, for RSS with 2ms duration time, single sample can satisfy the accuracy requirement for SNR=-6dB in CE mode A and 3 samples can satisfy the RSRP requirement for SNR=-12dB and SNR=-15dB respectively in CE mode B for non-BL UE.

# Conclusion

In this contribution the remaining issue about RLM are discussed. The following conclusion can be drawn:

Observation 1: For non-BL CE UE, for RSS with 2ms duration time, single sample can satisfy the accuracy requirement for SNR=-6dB in CE mode A and 3 samples can satisfy the RSRP requirement for SNR=-12dB and SNR=-15dB respectively in CE mode B for non-BL UE.

# References

[1] 3GPP, RAN4-1902018, “Simulation assumptions for RSS based RRM measurements for MTC”, Ericsson