3GPP TSG-RAN4 Meeting #91	R4-1905775

# Reno, USA, 13 – 17 May, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | CRNum | rev | - | Current version: | V15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.101-2 Complete FR2 MPR/A-MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - PC1 CA MPR should not be smaller than PC3 CA MPR. For cumulative aggregated bandwidth < 400 MHz, - Change MPR for DFT-s-OFDM 64QAM outer allocations from 7.5dB to 9dB (similar to corresponding PC3 value). - Change MPR for CP-OFDM 64QAM outer allocations from 7.5dB to 9dB (similar to corresponding PC3 value). - PC2 MPR should be updated to reflect the finalized PC3 MPR. - Reuse finalized PC3 MPR for PC2 MPR since PC2 and PC3 have same max TRP. - PC4 MPR should be updated to reflect the finalized PC3 MPR. PC4 A-MPR, PC4 CA MPR/A-MPR, and PC4 UL-MIMO A-MPR are missing. - Reuse finalized PC3 MPR for PC4 MPR since PC4 and PC3 have same max TRP. - Add missing PC4 A-MPR, PC4 CA MPR/A-MPR - Add PC4 UL-MIMO A-MPR description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For PC1, - Change MPR for DFT-s-OFDM 64QAM outer allocations from 7.5dB to 9dB (similar to corresponding PC3 value). - Change MPR for CP-OFDM 64QAM outer allocations from 7.5dB to 9dB (similar to corresponding PC3 value). - For PC2, - Reuse finalized PC3 MPR for PC2 MPR since PC2 and PC3 have same max TRP. - For PC4, - Reuse finalized PC3 MPR for PC4 MPR since PC4 and PC3 have same max TRP. - Add missing PC4 A-MPR, PC4 CA MPR/A-MPR - Add PC4 UL-MIMO A-MPR description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not correct and not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.2, 6.2.2.4, 6.2.3.2, 6.2A.2.5, 6.2A.3, 6.2D.3.4. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

#### 6.2A.2.2	Maximum output power reduction for power class 1

For power class 1, MPR for UL contiguous allocations within the cumulative aggregated bandwidth is defined as:

MPRC_CA = max(MPRWT_C_CA, MPRnarrow)

Where,

MPRnarrow=14.4 dB, when BWalloc,RB is less than or equal to 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT_C_CA is the maximum power reduction due to modulation orders, transmit bandwidth configurations, and waveform types. MPRWT_contiguous is defined in Table 6.2A.2.2-1.

Table 6.2A.2.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1

| Waveform Type |  | Cumulative aggregated channel bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | < 400 MHz | >=400 MHz and <800 MHz | >=800 MHz and <=1400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 7.7 | [8.2] |
|  | QPSK | ≤ 6.5 | 8.7 | [9.7] |
|  | 16 QAM | ≤ 6.5 | 8.7 | [9.2] |
|  | 64 QAM | ≤ 9.0 | 10.7 | [11.2] |
| CP-OFDM | QPSK | ≤ 6.5 | 8.7 | [8.7] |
|  | 16 QAM | ≤ 6.5 | 8.7 | [8.7] |
|  | 64 QAM | ≤ 9.0 | 10.7 | [11.2] |

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRC_CA.

For non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A +  [14.4])

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of simultaneously transmitted UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth

< end of changes >

< start of changes >

#### 6.2.2.2	UE maximum output power reduction for power class 2

For power class 2, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

Where,

-	MPRnarrow = 2.5 dB, when the allocated RB size is less than or equal to 1.44MHz, and 0≤RBstart≤Ceil(1/3 NRB) or Ceil(2/3NRB)≤RBstart≤NRB-LCRB

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2.2-1 and Table 6.2.2.2-2.

Table 6.2.2.2-1 MPRWT for power class 2, BWchannel ≤ 200 MHz

|  |  | MPRWT, BWchannel ≤ 200 MHz |  |
| --- | --- | --- | --- |
|  |  | RBstart ≥ Ceil(1/3 NRB) AND RBend ≤ Ceil(2/3 NRB) | RBstart ≥ Ceil(1/3 NRB) AND RBend ≤ Ceil(2/3 NRB) |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | 2.0 |
|  | QPSK | 0.0 | 2.0 |
|  | 16QAM | 3.0 | 3.5 |
|  | 64QAM | 5.0 | 5.5 |
| CP-OFDM | QPSK | 3.5 | 4.0 |
|  | 16QAM | 5.0 | 5.0 |
|  | 64QAM | 7.5 | 7.5 |

Table 6.2.2.2-2 MPRWT for power class 2, BWchannel = 400 MHz

|  |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | RBstart ≥ Ceil(1/4 NRB) AND RBend ≤ Ceil(3/4 NRB) AND LCRB≤Ceil(1/4 NRB) | RBstart < Ceil(1/4 NRB) OR RBend > Ceil(3/4 NRB) OR LCRB>Ceil(1/4 NRB) |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | 3.0 |
|  | QPSK | 0.0 | 3.0 |
|  | 16QAM | 4.5 | 4.5 |
|  | 64QAM | 6.5 | 6.5 |
| CP-OFDM | QPSK | 5.0 | 5.0 |
|  | 16QAM | 6.5 | 6.5 |
|  | 64QAM | 9.0 | 9.0 |

< end of changes >

< start of changes >

#### 6.2.2.4	UE maximum output power reduction for power class 4

For power class 4, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

Where,

MPRnarrow = 2.5 dB, when the allocated RB size is less than or equal to 1.44 MHz, and 0 ≤ RBstart ≤ Ceil(1/3 NRB) or Ceil(2/3NRB) ≤ RBstart ≤NRB-LCRB

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2.4-1 and Table 6.2.2.4-2.

Table 6.2.2.4-1 MPRWT for power class 4, BWchannel ≤ 200 MHz

|  |  | MPRWT, BWchannel ≤ 200 MHz |  |
| --- | --- | --- | --- |
|  |  | RBstart ≥ Ceil(1/3 NRB) AND RBend ≤ Ceil(2/3 NRB) | RBstart ≥ Ceil(1/3 NRB) AND RBend ≤ Ceil(2/3 NRB) |
| DFT-s-OFDM |  | Pi/2 BPSK |  | 0.0 |  | 2.0 |  |
|  |  | QPSK |  | 0.0 |  | 2.0 |  |
|  |  | 16QAM |  | 3.0 |  | 3.5 |  |
|  |  | 64QAM |  | 5.0 |  | 5.5 |  |
| CP-OFDM |  | QPSK |  | 3.5 |  | 4.0 |  |
|  |  | 16QAM |  | 5.0 |  | 5.0 |  |
|  |  | 64QAM |  | 7.5 |  | 7.5 |  |

Table 6.2.2.4-2 MPRWT for power class 4, BWchannel = 400 MHz

|  |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | RBstart ≥ Ceil(1/4 NRB) AND RBend ≤ Ceil(3/4 NRB) AND LCRB≤Ceil(1/4 NRB) | RBstart < Ceil(1/4 NRB) OR RBend > Ceil(3/4 NRB) OR LCRB>Ceil(1/4 NRB) |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | 3.0 |
|  | QPSK | 0.0 | 3.0 |
|  | 16QAM | 4.5 | 4.5 |
|  | 64QAM | 6.5 | 6.5 |
| CP-OFDM | QPSK | 5.0 | 5.0 |
|  | 16QAM | 6.5 | 6.5 |
|  | 64QAM | 9.0 | 9.0 |

< end of changes >

< start of changes >

#### 6.2.3.2	A-MPR for NS_201

##### 6.2.3.2.1	A-MPR for NS_201 for power class 1

Table 6.2.3.2.1-1: AMPR for NS_201 for power class 1

| Offset Frequency | Channel Bandwidth, MHz |  |  |
| --- | --- | --- | --- |
|  | 50, 100, 200 | 400 |  |
|  | Outer | Outer | Inner |
| 0 MHz | 3.0 | 5.0 | 3.5 |
| >100 MHz, <=300 MHz | 1.5 | 2.5 | 3.5 |
| >300 MHz | 0 | 0 | 0 |
| NOTE 1: The Offset frequency is defined as the frequency from the lower band edge to the lower channel edge. NOTE 2: The back off applied is the max(MPR, AMPR), where the MPR is defined in Table 6.2.2.1-1 NOTE 3: Any undefined region, MPR applies |  |  |  |

##### 6.2.3.2.2	A-MPR for NS_201 for power class 2

Table 6.2.3.2.2-1: AMPR for NS_201 for power class 2

| Offset Frequency | Channel Bandwidth, MHz |
| --- | --- |
|  | 400 |
|  | Outer RB allocations |
| 0 MHz | 1.5 |
| >100 MHz, <=300 MHz | 0 |
| >300 MHz | 0 |
| NOTE 1: The Offset frequency is defined as the frequency from the lower band edge to the lower channel edge. NOTE 2: The back off applied is the max(MPR, AMPR), where the MPR is defined in Table 6.2.2.2-1 NOTE 3: Any undefined region, MPR applies |  |

##### 6.2.3.2.3	A-MPR for NS_201 for power class 3

Table 6.2.3.2.3-1: AMPR for NS_201 for power class 3

| Offset Frequency | Channel Bandwidth, MHz |
| --- | --- |
|  | 400 |
|  | Outer RB allocations |
| 0 MHz | ≤ 1.5 |
| >100 MHz, <=300 MHz | 0 |
| >300 MHz | 0 |
| NOTE 1: The Offset frequency is defined as the frequency from the lower band edge to the lower channel edge. NOTE 2: The back off applied is the max(MPR, AMPR), where the MPR is defined in Table 6.2.2.3-1 NOTE 3: Any undefined region, MPR applies |  |

##### 6.2.3.2.4	A-MPR for NS_201 for power class 4

Table 6.2.3.2.4-1: AMPR for NS_201 for power class 4

| Offset Frequency | Channel Bandwidth, MHz |
| --- | --- |
|  | 400 |
|  | Outer RB allocations |
| 0 MHz | ≤ 1.5 |
| >100 MHz, <=300 MHz | 0 |
| >300 MHz | 0 |
| NOTE 1: The Offset frequency is defined as the frequency from the lower band edge to the lower channel edge. NOTE 2: The back off applied is the max(MPR, AMPR), where the MPR is defined in Table 6.2.2.4-1 NOTE 3: Any undefined region, MPR applies |  |

< end of changes >

< start of changes >

#### 6.2A.2.5	Maximum output power reduction for power class 4

For power class 4, MPR for UL contiguous allocations within the cumulative aggregated bandwidth is denoted as MPRC_CA and is defined in Table 6.2A.2.5-1.

Table 6.2A.2.5-1: Maximum power reduction (MPRC_CA) for UE power class 4

|  |  | Cumulative aggregated bandwidth configuration |  |  |
| --- | --- | --- | --- | --- |
|  |  | < 400 MHz | >=400 MHz and < 800 MHz | >= 800 MHz and <= 1400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.0 | ≤ 7.7 | ≤ [8.2] |
|  | QPSK | ≤ 5.0 | ≤ 7.7 | ≤ [8.2] |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ [9.3] |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ [11.2] |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 7.5 | ≤ [8.0] |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ [9.2] |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ [11.2] |

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the highest contiguous MPR.

For non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A +7.0)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of simultaneously transmitted UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth

< end of changes >

< start of changes >

#### 6.2A.3.2	A-MPR for CA_NS_201

##### 6.2A.3.2.1	A-MPR for CA_NS_201 for power class 1

For intra-band contiguous CA, AMPR is specified as follows.

Table 6.2A.3.2.1-1: Contiguous Allocations, AMPRC_CA for CA_NS_201 for power class 1

| Offset Frequency | Cumulative Aggregated Bandwidth, MHz |  |
| --- | --- | --- |
|  | < 400 | >= 400, <= 800 |
| 0 MHz | 5.0 | 7.0 |
| > 100 MHz, <= 300 MHz | 3.5 | 4.0 |
| > 300 MHz | 0 | 0 |
| NOTE 1: The Offset frequency is defined as the frequency from the lower band edge to the lower channel edge. NOTE 2: The back off applied is the max(MPR, AMPR), where the MPR is defined in Table 6.2A.2.2-1. NOTE 3: Any undefined region, MPR applies. |  |  |

For PC1 CA non-contiguous RB allocations, the following rule for AMPR applies:

AMPR = max(AMPRC_CA, -10*A +9.0) , Offset Frequency <= 550 MHz

##### 6.2A.3.2.2	A-MPR for CA_NS_201 for power class 2

For intra-band contiguous CA, AMPR is specified as follows.

Table 6.2A.3.2.2-1: Contiguous Allocations, AMPRC_CA for CA_NS_201 for power class 2

| Offset Frequency | Cumulative Aggregated Bandwidth, MHz |  |
| --- | --- | --- |
|  | < 400 | >=400, <=800 |
| 0 MHz | 1.5 | 3.0 |
| > 100 MHz, <=300 MHz | 0 | 0 |
| > 300 MHz | 0 | 0 |
| NOTE 1: The Offset frequency is defined as the frequency from the lower band edge to the lower channel edge. NOTE 2: The back off applied is the max(MPR, AMPR), where the MPR is defined in Table 6.2A.2.3-1. NOTE 3: Any undefined region, MPR applies. |  |  |

For PC2 CA non-contiguous RB allocations, the following rule for AMPR applies:

AMPR = max(AMPRC_CA, - 10*A + 5.0) , Offset Frequency <= 550 MHz

##### 6.2A.3.2.3	A-MPR for CA_NS_201 for power class 3

For intra-band contiguous CA, AMPR is specified as follows.

Table 6.2A.3.2.3-1: Contiguous Allocations, AMPRC_CA for CA_NS_201 for power class 3

| Offset Frequency | Cumulative Aggregated Bandwidth, MHz |  |
| --- | --- | --- |
|  | < 400 | >=400, <=800 |
| 0 MHz | ≤ 1.5 | ≤ 3.0 |
| > 100 MHz, <=300 MHz | 0 | 0 |
| > 300 MHz | 0 | 0 |
| NOTE 1: The Offset frequency is defined as the frequency from the lower band edge to the lower channel edge. NOTE 2: The back off applied is the max(MPR, AMPR), where the MPR is defined in Table 6.2A.2.4-1. NOTE 3: Any undefined region, MPR applies. |  |  |

For PC3 CA non-contiguous RB allocations, the following rule for AMPR applies:

AMPR = max(AMPRC_CA, - 10*A + 5.0) , Offset Frequency <= 550 MHz

##### 6.2A.3.2.4	A-MPR for CA_NS_201 for power class 4

For intra-band contiguous CA, AMPR is specified as follows.

Table 6.2A.3.2.4-1: Contiguous Allocations, AMPRC_CA for CA_NS_201 for power class 4

| Offset Frequency | Cumulative Aggregated Bandwidth, MHz |  |
| --- | --- | --- |
|  | < 400 | >=400, <=800 |
| 0 MHz | ≤ 1.5 | ≤ 3.0 |
| > 100 MHz, <=300 MHz | 0 | 0 |
| > 300 MHz | 0 | 0 |
| NOTE 1: The Offset frequency is defined as the frequency from the lower band edge to the lower channel edge. NOTE 2: The back off applied is the max(MPR, AMPR), where the MPR is defined in Table 6.2A.2.5-1. NOTE 3: Any undefined region, MPR applies. |  |  |

For PC4 CA non-contiguous RB allocations, the following rule for AMPR applies:

AMPR = max(AMPRC_CA, - 10*A + 5.0) , Offset Frequency <= 550 MHz

< end of changes >

< start of changes >

#### 6.2D.3.4	UE maximum output power reduction with additional requirements for UL-MIMO for power class 4

For UE with UL-MIMO, the A-MPR values specified in subclause 6.2.3 shall apply to the maximum output power specified in Table 6.2D.1.4-1. The requirements shall be met with the UL-MIMO configurations specified in sub-section 6.2D.1.4.

< end of changes >