3GPP TSG-RAN WG4 Meeting #91		R4-1905780

Reno, USA, 13 - 17 May, 2019

Agenda item:	7.15.4.2

Source:	Intel Corporation

Title: 	LTE HST performance evaluation – Bidirectional HST SFN scenario

Document for	Discussion

# Introduction

The work item (WI) “Further performance enhancement for LTE in high speed scenario” was approved in RAN #80 meeting [1]. One of the objectives of this WI is to specify downlink demodulation requirements under the identified high speed scenarios. In the RAN4 #90bis meeting there were several candidate scenario options captured in a WF [2]:

- Option 1: HST-SFN bi-directional scenario (Huawei, CMCC, Intel, NTT DOCOMO, Qualcomm, MTK) - The following maximum Doppler shifts supported for downlink will be further evaluated considering the uplink and downlink channels together to decide a pair of maximum Doppler shift for downlink and uplink - Agreement: companies can bring results for the following two options of maximum Doppler with MCS sweep from 12 to 16: - Option 1: 1100Hz - Option 2: 972Hz - Option 2: HST-SFN uni-directional scenario (Ericsson, Qualcomm) - Maximum Doppler shift for downlink for uni-directional - Option 1: 1250Hz (Qualcomm) - Other options with lower values compared to above not precluded - FFS: whether both bi- and uni-directional scenario are specified or not.

Companies were encouraged to provide performance evaluations and analysis of the candidate scenarios, in order to down select options. In this contribution we provide simulation results for the Option 1: HST-SFN bi-directional scenario.

# Bidirectional “4-path” HST SFN scenario

## Channel model

Figure 1. Bidirectional channel model

Fig.1 shows an example of the “4-path” HST SFN channel model. Given a different RRH to RRH distances (Ds) and RRH to railway track distances (Dmin), deployment scenarios for open space and tunnel are considered in [3,4]. The main parameters of deployment are summarized in Table 1.

Table 1. Parameters for bidirectional deployment

| Scenario/Parameters | Ds | Dmin |
| --- | --- | --- |
| Open Space | 1000 m | 50 m |
| Tunnel | 500 m | 5 m |

Analysis, provided in [5], showed that, UE demodulation performance in the tunnel scenario is better than in the open space scenario, so assuming worst case for further analysis we can deprioritize tunnel environment and stay only on the open space.

## Simulation assumptions

For performance alignment at the first stage it is most reasonable to reuse the similar simulation assumptions which were proposed in Rel. 14 HST WI [3]. The key parameters are summarized in Table 2.

Table 2. Simulation assumptions

| Parameter | Value |
| --- | --- |
| Bandwidth | 10 MHz |
| Duplex mode | FDD |
| Antenna configuration | 2x2 |
| Transmission mode | TM3 |
| EVM | 6 % |
| Channel estimation | MMSE (frequency domain) |
| CFO tracking | Practical CFO tracking |

Our evaluations were done in band agnostic manner so we can skip such parameters like train speed and carrier frequency and consider only max Doppler shift.  For the analysis we chose the range from 800 to 1200 Hz Doppler shift. To better understand performance behaviour under more practical scenarios we also provide evaluations with receiver frequency offset produced by local oscillator.

## Simulation results

In the figure 2 we provide the PDSCH normalized throughput simulation results for the different maximum Doppler shift values without receiver frequency offset. Evaluations were done for MCS range from 12 to 16.

| Figure 2. Normalized throughput for different Doppler shift without receive frequency offset |  |
| --- | --- |

Observation #1: For the bidirectional “4-tap” HST SFN channel model with 0 Hz receiver frequency offset

- For MCS 12-13 there is no impact or at least negligible impact on the demodulation performance for all considered Doppler shift values from 800 to 1200 Hz

- For MCS 14-16 there is no impact or at least negligible impact on the demodulation performance for Doppler shift values up to 1000 Hz.

In the figure 3 we provide results under assumption that UE additionally has 200 Hz receiver frequency offset.

| Figure 3. Normalized throughput for different Doppler shift with receive frequency offset |  |
| --- | --- |

Observation #2: For the bidirectional “4-tap” HST SFN channel model with 200 Hz receiver frequency offset

- For MCS 12-13 there is no impact or at least negligible impact on the demodulation performance for Doppler shift values up to 1000 Hz with 200 Hz receiver frequency offset.

- For MCS 14-16 there is a significant impact on the demodulation performance for all considered Doppler shift values from 800 to 1200 Hz with 200 Hz receiver frequency offset.

Proposal #1:	Limit the max Doppler shift by 972 Hz (Option 2) for the HST-SFN bidirectional scenario.

# Conclusion

In this contribution we provided simulation results to show the throughput performance under the bidirectional “4-path” HST SFN channel for different Doppler shift values for MCS range from 12 to 16. Following observations and proposals were made:

Observation #1: For the bidirectional “4-tap” HST SFN channel model with 0 Hz receiver frequency offset

- For MCS 12-13 there is no impact or at least negligible impact on the demodulation performance for all considered Doppler shift values from 800 to 1200 Hz

For MCS 14-16 there is no impact or at least negligible impact on the demodulation performance for Doppler shift values up to 1000 Hz.

Observation #2: For the bidirectional “4-tap” HST SFN channel model with 200 Hz receiver frequency offset

- For MCS 12-13 there is no impact or at least negligible impact on the demodulation performance for Doppler shift values up to 1000 Hz with 200 Hz receiver frequency offset.

For MCS 14-16 there is a significant impact on the demodulation performance for all considered Doppler shift values from 800 to 1200 Hz with 200 Hz receiver frequency offset.

Proposal #1:	Limit the max Doppler shift by 972 Hz (Option 2) for the HST-SFN bidirectional scenario.

# References

- RP-181482 “New Work Item on Further performance enhancement for LTE in high speed scenario”, RAN #80, June 2018

- R4-1904832 “Way forward on UE demodulation enhancement for HST”, RAN4 # 90bis, April 2019

- TR 36.878 v13.0.0 “Study on performance enhancements for high speed scenario”

- R4-1900805 “High speed scenarios and parameters for 500km/h velocity”, NTT DOCOMO, INC.

- R4-1902948 “LTE HST performance evaluation – Bidirectional HST SFN scenario”, Intel Corporation, RAN4 #90bis, April 2019