3GPP TSG-RAN WG4#91 Meeting	R4-1905783

Reno, U.S.A., 13 – 17 May, 2019

Agenda item:	6.12.1.4.4

Title: 	Simulation results and discussion on NR RI reporting

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN4#86 Meeting (Feb. 2018), we have provided our preliminary view on the scope of the NR UE performance requirements, and raised a number of questions to trigger the discussion on the topic of NR UE CSI feedback performance requirements [1]. In RAN4#87 Meeting (May 2018), the work plan [2] was approved for NR UE demodulation and CSI feedback performance requirements. And, the agreements related to the NR UE CSI performance requirements were reached in the WF [3] as listed below:

- First focused on the mandatory features

- Overall Test Configuration & Scope

  - First priority focused on:

    - NZP CSI-RS for channel measurement
CSI-IM for interference measurement

    - CSI report types: periodic CSI report and aperiodic CSI report

    - CSI-RS resources types: periodic and aperiodic

- PMI reporting

  - In Rel-15 focused on Type-I single panel codebook

In RAN4-AH-1807 Meeting (July 2018), a WF [4] on NR UE CSI requirements was approved regarding the initial simulation assumptions for FR1 and FR2 CQI/PMI/RI tests. Also, in RAN4#88, RAN4#88bis and RAN4#89 Meetings, WFs [5, 6, 7] were agreed and companies were encouraged to provide initial simulation results and views on the assumptions and scenarios for NR UE CSI feedback tests.

In RAN4#90 and RAN4#90bis Meetings, FR1 FDD/TDD and FR2 TDD RI reporting tests’ requirements were tentatively defined [8] [9], except for two FR2 RI test cases of high SNR range. For the two FR2 RI test cases of high SNR range, it was proposed to reduce the SNR test points from [20]dB to [16]dB, and the corresponding throughput ratio requirements needed to be revisited as well [9]. In this contribution, we first present our FR2 RI reporting simulation results. And then, we share our views on the two high SNR FR2 RI reporting tests’ requirements.

# FR2 RI Reporting Results and Discussion

In RAN4#90 and RAN4#90bis Meetings, FR1 FDD/TDD and FR2 TDD RI reporting tests’ requirements were tentatively defined [8] [9], except for two FR2 RI test cases of high SNR range. For the two FR2 RI test cases of high SNR range, it was proposed to reduce the SNR test points from [20]dB to [16]dB as shown Table 1 below [9].

Table 1: FR2 2X2 RI test case list

| Test 1 (2X2) ULA Low |  | Test 2 (2X2) ULA Low |  | Test 3 (2X2) XP High |  |
| --- | --- | --- | --- | --- | --- |
| SNR | Gamma 2 | SNR | Gamma 1 | SNR | Gamma 1 |
| [0 dB] | [1.0] | [16 dB] | [1.05] | [16 dB] | [1.05] |

Given the SNR test points to be reduced, their corresponding throughput ratio requirements need to be revisited as well. In Table 2 and Table 3, we present our FR2 RI reporting simulation results for SNR vs. Gamma1.

Table 2 FR2 TDD 2x2 ULA low

| SNR | Gamma1 |
| --- | --- |
| 0 | 1.00 |
| 2 | 1.00 |
| 4 | 1.00 |
| 6 | 1.00 |
| 8 | 1.01 |
| 10 | 1.01 |
| 12 | 1.05 |
| 14 | 1.07 |
| 16 | 1.16 |
| 18 | 1.23 |
| 20 | 1.26 |

Table 3 FR2 TDD 2x2 XP high

| SNR | Gamma1 |
| --- | --- |
| 0 | 1.00 |
| 2 | 1.00 |
| 4 | 1.00 |
| 6 | 1.00 |
| 8 | 1.00 |
| 10 | 1.03 |
| 12 | 1.07 |
| 14 | 1.12 |
| 16 | 1.21 |
| 18 | 1.26 |
| 20 | 1.28 |

Based on the simulation observations in Table 2 and Table 3, and considering necessary margin, we make the following proposals for FR2 RI tests and requirements.

Proposal 1: For “Test 2” in Table 1, FR2 2x2 ULA Low RI test, the requirements are defined as follows:

- SNR point = [16]dB;

- Gamma1, γ = [1.05].

Proposal 2: For “Test 3” in Table 1, FR2 2x2 XP High RI test, the requirements are defined as follows:

- SNR point = [16]dB;

- Gamma1, γ = [1.05].

# Conclusion

In this contribution, we first present our FR2 RI reporting simulation results. And then, we share our views on the two high SNR FR2 RI reporting tests’ requirements. Based on the simulation observations in Table 2 and Table 3, and considering necessary margin, we make the following proposals for FR2 RI tests and requirements.

Proposal 1: For “Test 2” in Table 1, FR2 2x2 ULA Low RI test, the requirements are defined as follows:

- SNR point = [16]dB;

- Gamma1, γ = [1.05].

Proposal 2: For “Test 3” in Table 1, FR2 2x2 XP High RI test, the requirements are defined as follows:

- SNR point = [16]dB;

- Gamma1, γ = [1.05].

# Reference

[1] R4-1801758, “Views on NR UE performance requirements”, Intel Corporation, Feb. 2018

[2] R4-1808029, “Work plan for UE performance requirements”, Samsung, May 2018

[3] R4-1808027, “Way forward on NR UE demodulation and CSI requirements”, Ericsson, May 2018

[4] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung, July 2018

[5] R4- 1811693, “Way forward on NR UE CSI requirements”, Samsung, August 2018

[6] R4- 1814235, “Way forward on NR UE CSI requirements”, Samsung, October 2018

[7] R4- 1816491, “Way forward on NR UE CSI requirements”, Samsung, November 2018

[8] R4- 1902404, “Way forward on NR UE CSI requirements”, Samsung, Feb. – Mar. 2019

[9] R4- 1904752, “Ad-hoc agenda for NR UE demod”, Samsung, April 2019