3GPP TSG RAN WG4 Meeting #91	R4-1905787

Reno, USA, 13 - 17 May 2019

Agenda item:	9.8.3

Source:	SoftBank Corp.

Title:	TP for TR 37.716-21-21: EN-DC_1-8_n77-n257

Document for:	Approval

1	Introduction

EN-DC of 3B LTE and 1B NR of DC_1-8_n77-n257 was approved in RAN#83[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-8_n77-n257 | CA_1-8 | n77-n257 | DC_1_n77 |

Table 2: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A_n77A-n257A | DC_1A_n77A DC_8A_n77A DC_1A_n257A DC_8A_n257A | CA_1A-8A | n77A-n257A |
| DC_1A-8A_n77A-n257D | DC_1A_n77A DC_8A_n77A DC_1A_n257A DC_8A_n257A | CA_1A-8A | n77A-n257D |
| DC_1A-8A_n77A-n257G | DC_1A_n77A DC_8A_n77A DC_1A_n257A DC_8A_n257A | CA_1A-8A | n77A-n257G |
| DC_1A-8A_n77A-n257H | DC_1A_n77A DC_8A_n77A DC_1A_n257A DC_8A_n257A | CA_1A-8A | n77A-n257H |
| DC_1A-8A_n77A-n257I | DC_1A_n77A DC_8A_n77A DC_1A_n257A DC_8A_n257A | CA_1A-8A | n77A-n257I |

3.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows.

Table 3: ΔTIB of constituent 4 band combo.

| ΔTIB | B1+B8+n77 | B1+B8+n257 | B1+n77+n257 | B8+n77+n257 | Proposed Value |
| --- | --- | --- | --- | --- | --- |
| B1 | 0.3 | 0.3 | 0.6 |  | 0.6 |
| B8 | 0.6 | 0.3 |  | 0.6 | 0.6 |
| n77 | 0.8 |  | 0.8 | 0.8 | 0.8 |
| n257 |  | 0 | 0 | 0 | 0 |

Table 4: ΔRIB of constituent 4 band combo.

| ΔRIB | B1+B8+n77 | B1+B8+n257 | B1+n77+n257 | B8+n77+n257 | Proposed Value |
| --- | --- | --- | --- | --- | --- |
| B1 | 0 | 0 | 0.2 |  | 0.2 |
| B8 | 0.2 | 0 |  | 0.2 | 0.2 |
| n77 | 0.5 |  | 0.5 | 0.5 | 0.5 |
| n257 |  | 0 | 0 | 0 | 0 |

Then the proposal for this 2LTE-2NR EN-DC is:

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n77-n257 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
|  | n257 | 0 |

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n77-n257 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

5.  REFSENS

MSD of this 4DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01, TR37.863-02-01, TR 37.864-11-22, TR 37.716-21-11 and TR37.716-21-21. Therefore, no additional MSD requirement is needed.

6.  Conclusion

This paper proposes study results of EN-DC 1-8_n77-n257.

TP is shown in Annex.

7.  Reference

[1]  RP-190151  Revised WID on LTE x bands (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in Rel-16

Annex.

[Unchanged Parts Skipped]

7.X	DC_1-8_n77-n257

7.X.1	Operating bands for DC

Table 7.X.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.X.2	Channel bandwidths per operating band for DC

Table 7.X.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-8A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_1A-8A_n77A-n257D | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 330 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 430 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

7.X.3	Co-existence studies

Based on the co-existence studies of DC_1A_n77A, DC_1A_n257A, DC_8A_n77A and DC_8A_n257A captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.X.4	∆TIB and ∆RIB values

For DC_1A-8A_n77A-n257A, the TIB,c and RIB,c values are given in the tables below.

Table 7.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n77-n257 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n77-n257 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.X.5	MSD

As mentioned in 7.X.3, there is no need to specify additional MSD requirement for this UL DC configuration..

[Unchanged Parts Skipped]