3GPP TSG-RAN WG4 Meeting #91			R4-1905841

# Reno, US, 13th– 17th May, 2019

Title:	Reply LS on NR V2X UE SL transmit timing Error

Response to:	R1-1810006

Release:	Release 16

Work Item:	5G_V2X_NRSL-Core

Source:	3GPP RAN WG4

To:	3GPP RAN WG1

Cc:

Contact Person:

Name:	Yoonoh Yang

E-mail Address:	Yoonoh.yang@ lge.com

Attachments:	None.

1. Overall Description:

RAN4 sent NR V2X UE SL timing error for gNB and GNSS synchronization reference in [1] as starting point with condition RAN4 will further analyse the timing error during NR V2X WI phase. Hence RAN4 further discussed on the timing error and made the following answer.

- Question 1: RAN1 would like to ask RAN4 to confirm the validity of the evaluation assumptions or provide guidance on appropriate values for:

- AGC settling time

- TX/RX switching time (already completed in R4-1816666)

- Timing error between a UE and its synchronization reference

- Frequency error between a UE and its synchronization reference

Answer: Timing error between a UE and its synchronization reference

- GNSS synchronization reference source

| SCS of SL signals(KHz) | Te(Tc) |
| --- | --- |
| 15 | 8*64*Tc |
| 30 | 8*64*Tc |
| 60 | 7*64*Tc |
| 120 | 7*64*Tc |

- NR V2X UE synchronization reference source

| NR SyncRefUE SCS of SSSB/SPBCH (KHz) | SCS of SL signals (kHz) | Te(Tc) |
| --- | --- | --- |
| 15 | 15 | 18*64*Tc |
|  | 30 | 17*64*Tc |
|  | 60 | 16*64*Tc |
|  | 120 | 16*64*Tc |
| 30 | 15 | 12*64*Tc |
|  | 30 | 10*64*Tc |
|  | 60 | 10*64*Tc |
|  | 120 | 10*64*Tc |
| 60 | 15 | 8*64*Tc |
|  | 30 | 8*64*Tc |
|  | 60 | 7*64*Tc |
|  | 120 | 7*64*Tc |
| 120 | 15 | 6*64*Tc |
|  | 30 | 6*64*Tc |
|  | 60 | 5*64*Tc |
|  | 120 | 5*64*Tc |

- gNB synchronization reference source :

| SCS of DL SSB signals (KHz) | SCS of SL signals(KHz) | Te |
| --- | --- | --- |
| 15 | 15 | 12*64*Tc |
|  | 30 | 10*64*Tc |
|  | 60 | 10*64*Tc |
|  | 120 | 10*64*Tc |
| 30 | 15 | 8*64*Tc |
|  | 30 | 8*64*Tc |
|  | 60 | 7*64*Tc |
|  | 120 | 7*64*Tc |
| 120 | 15 | 4.5*64*Tc |
|  | 30 | 4.5*64*Tc |
|  | 60 | 3.5*64*Tc |
|  | 120 | 3.5*64*Tc |
| 240 | 15 | 4*64*Tc |
|  | 30 | 4*64*Tc |
|  | 60 | 3*64*Tc |
|  | 120 | 3*64*Tc |

2. Actions:

To RAN WG1:

RAN WG4 respectfully asks RAN WG1 to evaluate the NR V2X sidelink operation taking the above information into account.

3. Date of Next TSG-RAN Meetings:

3GPP RAN WG4 #92	    26th – 30th August 2019				Ljubljana, Slovenia

3GPP RAN WG4 #92bis	   14th – 18th October 2019				China, CN

Reference

[1] R4-1905241, “reply LS on NR V2X UE RF parameters for NR V2X service”,, LG Electronics