# 3GPP TSG-RAN WG4 Meeting #91                                                        R4-1905856

# Reno, Nevada, USA, 13th - 17th May, 2019

Agenda Item:	6.4.2

Source:	ZTE Corporation

Title:	Clarification on ULSUP FDM with single Tx

Document for:	Approval

# Introduction

In RAN4 #90bis, during the discussions on two CRs related to ULSUP FDM when single Tx is allowed, one concern was raised on whether or not E-UTRA UL and NR SUL are allowed to be transmitted simultaneously from RAN2 perspective [1][2][3].

In this contribution, we investigate the related RAN2 signaling and find that in this case simultaneous transmission of E-UTRA UL and NR SUL are not allowed even for ULSUP FDM according to the current signaling design, although in RAN4 understanding it is allowed instead.

# Discussion

When a UE configured with EN-DC requires single UL transmission, “tdm-PatternConfig” is setup to indicate when an E-UTRA uplink is transmitted and when an NR uplink is transmitted as shown below IE [4]:

RRCConnectionReconfiguration-v1510-IEs ::= SEQUENCE {

nr-Config-r15					CHOICE {

release							NULL,

setup							SEQUENCE {

endc-ReleaseAndAdd-r15	BOOLEAN,

nr-SecondaryCellGroupConfig-r15	OCTET STRING				OPTIONAL,	-- Need ON

p-MaxEUTRA-r15					P-Max						OPTIONAL	-- Need ON

}

}																	OPTIONAL,	-- Need ON

sk-Counter-r15					INTEGER (0.. 65535)					OPTIONAL,	-- Need ON

nr-RadioBearerConfig1-r15		OCTET STRING						OPTIONAL,	-- Need ON

nr-RadioBearerConfig2-r15		OCTET STRING						OPTIONAL,	-- Need ON

tdm-PatternConfig-r15			CHOICE {

release							NULL,

setup							SEQUENCE {

subframeAssignment-r15			SubframeAssignment-r15,

harq-Offset-r15					INTEGER (0.. 9)

}

}																	OPTIONAL,	-- Cond FDD-PCell

nonCriticalExtension			RRCConnectionReconfiguration-v1530-IEs		OPTIONAL

}

tdm-PatternConfig

UL/DL reference configuration indicating the time during which a UE configured with EN-DC is allowed to transmit. This field is used when power control or IMD issues require single UL transmission as specified in TS 38.101-3 [85] and TS 38.213 [88].

And this is also reflected in another IE MRDC-Parameters shown below [5]:

Therefore, in the case when single uplink transmission is required and ULSUP is performed via FDM, i.e., E-UTRA uplink is FDMed with the NR SUL, the E-UTRA uplink cannot be transmitted simultaneously with the NR SUL from RAN2 perspective, although there is no issue to do so in RAN4’s understanding.

Observation 1: In the case when single uplink transmission is required and ULSUP is performed via FDM, i.e., E-UTRA uplink is FDMed with the NR SUL, the E-UTRA uplink cannot be transmitted simultaneously with the NR SUL from RAN2 perspective, although there is no issue to do so in RAN4’s understanding.

With this observation, the current RAN4 spec need to clarify explicitly at this point in order to avoid confusion, and a possible simple way is to add one note to the SUL band combination table in TS 38.101-3.

Proposal 1: Add one note to the SUL band combination table in TS 38.101-3, clarifying that even in ULSUP FDM mode, for the band combinations allowing single Tx, E-UTRA UL and NR SUL are not allowed to be transmitted simultaneously.

# Conclusion

In this contribution, we have the following observation and proposal for ULSUP FDM when single Tx is allowed:

Observation 1: In the case when single uplink transmission is required and ULSUP is performed via FDM, i.e., E-UTRA uplink is FDMed with the NR SUL, the E-UTRA uplink cannot be transmitted simultaneously with the NR SUL from RAN2 perspective, although there is no issue to do so in RAN4’s understanding.

Proposal 1: Add one note to the SUL band combination table in TS 38.101-3, clarifying that even in ULSUP FDM mode, for the band combinations allowing single Tx, E-UTRA UL and NR SUL are not allowed to be transmitted simultaneously.

# Reference

[1] Chairman notes for RAN4#90bis

[2] R4-1905135	Draft CR on SUL band combinations to TS 38.101-3, Huawei

[3] R4-1904320	draft CR to 38.101-3: ULSUP-TDM related clarifications, Nokia

[4] TS 36.331 v15

[5] TS 38.331 v15.5.0