# 3GPP TSG-RAN WG4 Meeting #90bis                                                        R4-1905858

# Reno, NV, USA, 13th - 17th May, 2019

Agenda Item:	6.12.2.3.1

Source:	ZTE Corporation

Title:	Simulation results for multi-slot PUCCH

Document for:	Approval

# Introduction

In RAN4 #90bis, a WF was approved on multi-slot NR PUCCH with one open issue related to frequency hopping:

Frequency hopping

  - Option 1: Inter-slot frequency hopping enabled and intra-slot frequency hopping disabled

  - Option 2: both intra and inter slot frequency hopping are disabled

  - Option 3: Inter-slot frequency hopping disabled, and intra-slot frequency hopping enabled

Agreement:

Option 1 and 3 for further simulation

In this contribution, we provide some numerical results for both agreed options.

# Discussion

The simulation setup is summarized as the following table:

| PUCCH Format |  | Format 1 |
| --- | --- | --- |
| BandWidth/SCS |  | 40 MHz/ 30kHz |
| Antenna |  | 1T8R |
| Propagation condition |  | TDLC300-100; |
| Carrier frequency (GHz) |  | 4GHz; |
| (#Bits, #Symbols, #PRBs) |  | (2, 14,1) |
| initialCyclicShift |  | 0 |
| StartingSymbolIndex |  | 0 |
| Index of orthogonal sequence (time-domain-OCC) |  | 0 |
| Inter/Intra-slot frequency hopping |  | Option 1: Enabled/Disabled |
|  |  | Option 2: Disabled/Enabled |
| number of slot |  | 2 |
| Test metric | DTX to Ack probability <1% | √ |
|  | Missed Ack probability < 1% | √ |
|  | NACK2ACK < 0.1% | √ |

And the numerical results are shown in Fig. 1:

Fig. 1, simulation results for inter/intra-slot frequency hopping for multi-slot PUCCH

From Fig. 1, inter-slot frequency hopping leads to 1~1.5 dB gain over intra-slot frequency hopping. Therefore we propose to enable inter-slot frequency hopping to define multi-slot PUCCH performance requirements.

Observation 1: inter-slot frequency hopping leads to 1~1.5 dB gain over intra-slot frequency hopping.

Proposal 1: Enables inter-slot frequency hopping to define multi-slot PUCCH performance requirements.

# Conclusion

In this contribution, we provide some numerical results for multi-slot PUCCH and have the following observation and proposal:

Observation 1: inter-slot frequency hopping leads to 1~1.5 dB gain over intra-slot frequency hopping.

Proposal 1: Enables inter-slot frequency hopping to define multi-slot PUCCH performance requirements.

# Reference

[1] R4-1904719 WF on multi-slot PUCCH, ZTE