3GPP TSG-WG RAN4 Meeting #91	R4-1905883

# Reno, USA, 13th – 17th May, 2019

Source: 	Nokia, Nokia Shanghai Bell

Title: 					TP for TR 36.716-02-01 Introduction of CA_7-46

Agenda Item: 			7.2.3 [LTE_CA_R16_2BDL_1BUL-Core]

Document for:	Approval

# Introduction

This TP introduces 3-band E-UTRA CA configurations of CA_3A-7C-46A, 3A-7C-46C, 3A-7C-46D, and 3A-7C-46E to TR 36.716-03-01.

**************************** Start of changes ********************************************

## 5.X	CA_7-46

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |

Table 5.X.1-2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_7C-46E | - | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  | 120 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

### 5.X.2	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c values for three band CA_7-46 are already specified in TS 36.101 Table 6.2.5-2 (two bands) for ΔTIB,c  and Table 7.3.1-1A (two bands) for ΔRIB,c.

### 5.X.3	REFSENS requirements

Reference sensitivity requirements shall be specified in TS 36.101 Table 7.3.1A-0eA according to Table 5.X.3-1 below.

Table 5.X.3-1: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_7C-46E | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |

**************************** End of changes ********************************************