3GPP TSG-WG RAN4 Meeting #91	R4-1905887

# Reno, USA, 13th – 17th May, 2019

Source: 	Nokia, Nokia Shanghai Bell, [AT&T]

Title: 					TP for TR 36.716-02-02 Introduction of CA_2-14

Agenda Item: 			7.5.3 [LTE_CA_R16_2BDL_2BUL-Core]

Document for:	Approval

# Introduction

This TP introduces 2-band 2UL E-UTRA CA configurations of CA_2A-14A.

**************************** Start of changes ********************************************

## 5.X	CA_2-14

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| 14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | TDD |

Table 5.X.1-2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-14A | CA_2A-14A | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 14 |  |  | Yes | Yes |  |  |  |  |

### 5.X.2	Co-existence studies

The following table shows the 2 UL harmonics and IMD products of band 2 and band 14.

Table 5.X.2-1: CA_2A-14A UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 788 | 798 | 1850 | 1910 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1576 | 1596 | 3700 | 3820 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2364 | 2394 | 5550 | 5730 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1052 | 1122 | 2638 | 2708 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -334 | -254 | 2902 | 3032 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3426 | 3506 | 4488 | 4618 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 454 | 544 | 4752 | 4942 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4214 | 4304 | 6338 | 6528 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -2244 | -2104 | 5276 | 5416 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -6852 | -6602 | -1342 | -1242 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 8188 | 8438 | 5002 | 5102 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -4154 | -3954 | 1306 | 1456 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 7126 | 7326 | 6064 | 6214 |

Table 5.X.2-1 concludes that no harmonic or IMD product up to 5th order falls into UE’s own DL bands.

The spurious emission towards UE coexisting bands are specified for Regions 3 according to band 2 and band 14 coexisting bands.

Table 5.X.2-2: Requirements for uplink inter-band carrier aggregation CA_2-14

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_2-14 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 3, 9 |
| NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 9: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD. |  |  |  |  |  |  |  |

### 5.X.3	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c values for two band CA_2-14 are already specified in TS 36.101 Table 6.2.5-2 (two bands) for ΔTIB,c and Table 7.3.1-1A (two bands) for ΔRIB,c.

### 5.X.4	REFSENS requirements

Since there is no harmonic or IMD relation, no MSD is required due to harmonic or IMD.

**************************** End of changes ********************************************