3GPP TSG-RAN WG4 #91 Meeting                                        	R4-1906043

Reno, USA, 13 – 17 May, 2019

Source: 	LG Electronics

Title: 	TP on correcting MSD of x bands (3, 4, 5) DL including Band 46 with 2 bands UL

Agenda item:	7.6.2

Document for:	Approval

1	Introduction

MSD values of x bands (3, 4, 5) DL including band 46 with 2 bands UL have been derived for those who have self-interference problems in [1]. However, for band combinations including band 46, the requirements are already specified in Table 7.3.1A-0eA. Moreover, when band 46 has self-interference problems by operating dual uplink CA, then the requirements do not need to apply in exclusion zone. Therefore, in this contrition, we remove the MSD values of x bands (3, 4, 5) DL including band 46 with 2 bands UL and replace them with applying the requirements from Table 7.3.1A-0eA with the exclusion zone area.

=== Text Proposal for TR36.716-03-02 section 6.x ===

6.6 LTE-A inter-band CA: Band 2 and Band 5 and Band 46 DL with 2 bands UL

----- Unchanged sections omitted -----

6.6.1.3	MSD

When uplink CA_2A_5A is paired with downlink CA_2A-5A-46D, the 4th and 5th  order IMD products by band 2 and band 5 falls into the own Rx frequency band 46.

For band combinations including band 46, there is no need to study for MSD since the requirements are already specified in Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49) and when band 46 have self-interference problems by operating dual uplink CA, then these requirements do not need to apply in exclusion zone which is frequency range within (harmonics frequency region + delta FHD in Table 7.3.1A-0eC) and IMD frequency region as below.

Table 6.6.1.3-1: IMD frequency range for UL CA_2A-5A and DL CA_2A-5A-46D

| DL_CA configuration | UL_CA configuration | Exclusion zone center frequency | Exclusion zone BW |
| --- | --- | --- | --- |
| CA_2A-5A-46D | CA_2A-5A | 3*fc_5A + 1*fc_2A | 3*BW_5A + 1*BW_2A |
| CA_2A-5A-46D | CA_2A-5A | 2*fc_5A – 3*fc_2A | 2*BW_5A + 3*BW_2A |

----- Unchanged sections omitted -----

6.7 LTE-A inter-band CA: Band 5 and Band 46 and Band 66 DL with 2 bands UL

----- Unchanged sections omitted -----

6.7.1.4	MSD

When uplink CA_5A-46A is paired with downlink CA_5A-46D-66A, the 5th order IMD product by band 5 and band 46 falls into the own Rx frequency band 66.

In Table 6.7.1.4-1, evaluated MSD values of UL CA_5A-46ADL_CA_5A-46D-66Ais shown.

Table 6.7.1.4-1: MSD summary for UL CA_5A_46A and DL CA_5A-46D-66A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_5A-46D-66A | CA_5A-46A | 5 | 834 | 5 | 25 | 879 | 5 | N/A | FDD | IMD5 |
|  |  | 46 | 5491 | 20 | 100 | 5491 | 20 |  | TDD |  |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | 0.3 | FDD |  |

When uplink CA_5A-66A is paired with downlink CA_5A-46D-66A, the 4th and 5th order IMD products by band 5 and band 66 falls into the own Rx frequency band 46.

For band combinations including band 46, there is no need to study for MSD since the requirements are already specified in Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49) and when band 46 have self-interference problems by operating dual uplink CA, then these requirements do not need to apply in exclusion zone which is frequency range within (harmonics frequency region + delta FHD in Table 7.3.1A-0eC) and IMD frequency region as below.

Table 6.7.1.4-2: IMD frequency range for UL CA_5A-66A and DL CA_5A-46D-66A

| DL_CA configuration | UL_CA configuration | Exclusion zone center frequency | Exclusion zone BW |
| --- | --- | --- | --- |
| CA_5A-46D-66A | CA_5A-66A | 3*fc_66A + 1*fc_5A | 3*BW_66A + 1*BW_5A |
| CA_5A-46D-66A | CA_5A-66A | 2*fc_66A – 3*fc_5A | 2*BW_66A + 3*BW_5A |
| CA_5A-46D-66A | CA_5A-66A | 2*fc_66A + 3*fc_5A | 2*BW_66A + 3*BW_5A |

----- Unchanged sections omitted -----

6.9 LTE-A inter-band CA: Band 13 and Band 46 and Band 66 DL with 2 bands UL

----- Unchanged sections omitted -----

6.9.1.3	MSD

When uplink CA_13A_66A is paired with downlink CA_13A-46D-66A, the 4th and 5th order IMD products by band 13 and band 66 falls into the own Rx frequency band 46.

For band combinations including band 46, there is no need to study for MSD since the requirements are already specified in Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49) and when band 46 have self-interference problems by operating dual uplink CA, then these requirements do not need to apply in exclusion zone which is frequency range within (harmonics frequency region + delta FHD in Table 7.3.1A-0eC) and IMD frequency region as below.

6.9.1.3-1: IMD frequency range for UL CA_13A-66A and DL CA_13A-46D-66A

| DL_CA configuration | UL_CA configuration | Exclusion zone center frequency | Exclusion zone BW |
| --- | --- | --- | --- |
| CA_13A-46D-66A | CA_13A-66A | 2*fc_66A + 2*fc_13A | 2*BW_66A + 2*BW_13A |
| CA_13A-46D-66A | CA_13A-66A | 2*fc_66A + 3*fc_13A | 2*BW_66A + 3*BW_13A |

----- Unchanged sections omitted -----

6.10 LTE-A inter-band CA: Band 2 and Band 13 and Band 46 DL with 2 bands UL

----- Unchanged sections omitted -----

6.10.1.3	MSD

When uplink CA_2A_13A is paired with downlink CA_2A-13A-46D, the 4th order IMD product by band 2 and band 13 falls into the own Rx frequency band 46.

For band combinations including band 46, there is no need to study for MSD since the requirements are already specified in Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49) and when band 46 have self-interference problems by operating dual uplink CA, then these requirements do not need to apply in exclusion zone which is frequency range within (harmonics frequency region + delta FHD in Table 7.3.1A-0eC) and IMD frequency region as below.

6.10.1.3-1: IMD frequency range for UL CA_2A-13A and DL CA_2A-13A-46D

| DL_CA configuration | UL_CA configuration | Exclusion zone center frequency | Exclusion zone BW |
| --- | --- | --- | --- |
| CA_2A-13A-46D | CA_2A-13A | 3*fc_13A + 1*fc_2A | 3*BW_13A + 1*BW_2A |

----- End of TP -----

References

- TR 36.716-03-02 v0.5.0