3GPP TSG-RAN WG4 Meeting #91	R4-1906053

# Reno, USA, May 13 – 17, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5476 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of LTE-A Inter-band CA Rel-16 for new x bands (x=3,4,5) DL with 2 bands UL |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R16_xBDL_2BUL |  |  |  |  |  |  | Date: |  |  | 2019-05-16 |  |
| Category: | B |  |  |  |  |  |  | Release: |  |  | Rel-16 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |  |
| Reason for change: |  | New LTE-A Inter-band CA band combinations for x bands (X=3,4,5) DL with 2 bands UL in Rel-16 have been studied and introduced in corresponding TR in rel-16. |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following operating bands are completed and introduce in rel-16 - New 3 bands DL with 2 bands UL CA band lists are introduced to Table 5.6A.1-2a: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (three bands). 3 bands DL; 2 bands UL. 2A-48A-66A; 48A-66A. 3A-8A-38A; 3A-8A. 1A-1A-3C-28A; 1A-3A 1A-28A 3A-28A. 2A-13A-66A 2A-13A-66B 2A-13A-66C 2A-2A-13A-66A 2A-13A-66A-66A; 2A-13A 13A-66A. 1A-3A-42D; 1A-3A 1A-42A, 3A-42A. 1A-3A-3A-7A 1A-3A-3A-7A-7A; 1A-3A 1A-7A 3A-7A. - For some CA band combinations with IMD problems, the MSD exception requirements are defined in 7.3.1A - When the band 46 has self-interference problems with operating dual uplink CA, the requirements of Refesens (band 46) do not need to be applied in exlcusion zone. The MSD values of some x bands (x= 3,4,5) DL including band 46 with 2 bands UL need to be removed due to some errors and the exclusion zone area is specified for those bands . IMD frequency range for UL CA_2A-5A and DL CA_2A-5A-46D DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_2A-5A-46D; CA_2A-5A; 3*fc_5A + 1*fc_2A; 3*BW_5A + 1*BW_2A. CA_2A-5A-46D; CA_2A-5A; 2*fc_5A – 3*fc_2A; 2*BW_5A + 3*BW_2A. IMD frequency range for UL CA_2A-5A and DL CA_2A-5A-46D DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_2A-5A-46D; CA_2A-5A; 3*fc_5A + 1*fc_2A; 3*BW_5A + 1*BW_2A. CA_2A-5A-46D; CA_2A-5A; 2*fc_5A – 3*fc_2A; 2*BW_5A + 3*BW_2A. IMD frequency range for UL CA_13A-66A and DL CA_13A-46D-66A DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_13A-46D-66A; CA_13A-66A; 2*fc_66A + 2*fc_13A; 2*BW_66A + 2*BW_13A. CA_13A-46D-66A; CA_13A-66A; 2*fc_66A + 3*fc_13A; 2*BW_66A + 3*BW_13A. IMD frequency range for UL CA_2A-13A and DL CA_2A-13A-46D DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_2A-13A-46D; CA_2A-13A; 3*fc_13A + 1*fc_2A; 3*BW_13A + 1*BW_2A. |
| Consequences if not approved: |  | New x bands (x=3,4,5) DL with 2 bands UL LTE-CA band combinations won’t be supported in Rel-16 and there will be MSD values left on some band combinations including the band 46 which does not need to be specified. |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6A.1, 7.3.1A |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  |  | Test specifications |  |  |  | TS/TR ... CR ... 36.521-1 |  |  |  |
| (show related CRs) |  |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | N/A |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  | No |  |  |  |  |  |  |  |  |

<Start of Changes>

5.6A.1	Channel bandwidths per operating band for CA

The requirements for carrier aggregation in this specification are defined for carrier aggregation configurations with associated bandwidth combination sets. For inter-band carrier aggregation, a carrier aggregation configuration is a combination of operating bands, each supporting a carrier aggregation bandwidth class. For intra-band contiguous carrier aggregation, a carrier aggregation configuration is a single operating band supporting a carrier aggregation bandwidth class.

For each carrier aggregation configuration, requirements are specified for all bandwidth combinations contained in a bandwidth combination set, which is indicated per supported band combination in the UE radio access capability. A UE can indicate support of several bandwidth combination sets per band combination.

Requirements for intra-band contiguous carrier aggregation are defined for the carrier aggregation configurations and bandwidth combination sets specified in Table 5.6A.1-1. Requirements for inter-band carrier aggregation are defined for the carrier aggregation configurations and bandwidth combination sets specified in Table 5.6A.1-2, Table 5.6A.1-2a, Table 5.6A.1-2b and Table 5.6A.1-2c. Requirements for intra-band non-contiguous carrier aggregation are defined for the carrier aggregation configurations and bandwidth combination sets specified in Table 5.6A.1-3.

The DL component carrier combinations for a given CA configuration shall be symmetrical in relation to channel centre unless stated otherwise in Table 5.6A.1-1, Table 5.6A.1-2, Table 5.6A.1-2a, Table 5.6A.1-2b and Table 5.6A.1-2c.

Table 5.6A.1-2a: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (three bands)

----- Unchanged sections omitted -----

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 5) | E-UTRA Bands | 1.4 MHz | 3 MHz |  | 5 MHz |  | 10 MHz |  |  | 15 MHz |  |  | 20 MHz |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-5A | CA_1A-3A CA_1A-5A6 CA_3A-5A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-1A-3A-5A | - | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1C-3A-5A | - | 1 | See CA_1C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-3A-7A-7A | CA_1A-3A, CA_1A-7A, CA_3A-7A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 | See the CA_3A-3A Bandwidth combination set 0 in Table below |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See the CA_7A-7A Bandwidth combination set 1 in Table below |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3C-5A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-7A | CA_1A-3A CA_1A-7A CA_3A-7A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-1A-3A-7A | - | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-1A-3C-7A | - | 1 | See the CA_1A-1A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-1A-3C-28A | CA_1A-3A, CA_1A-28A CA_3A-28A | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-3A-7A | CA_1A-3A, CA_1A-7A, CA_3A-7A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See the CA_3A-3A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-3A-7C | 7C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-7A-7A | CA_1A-3A CA_1A-7A CA_3A-7A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-7C | CA_1A-3A, CA_1A-7A, CA_3A-7A, CA_7C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3C-7A | CA_1A-3A, CA_1A-7A, CA_3A-7A, CA_3C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 1 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3C-7C | CA_1A-3A, CA_1A-7A, CA_3A-7A, CA_3C, CA_7C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-8A | CA_1A-3A CA_1A-8A CA_3A-8A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 40 | 2 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 3 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-3A-8A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3C-8A | CA_1A-3A CA_1A-8A CA_3A-8A CA_3C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-3A-42C | CA_1A-3A, CA_1A-42A, CA_3A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-11A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-18A | CA_1A-3A, CA_1A-18A6, CA_3A-18A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-3A-19A | CA_1A-3A CA_1A-19A6 CA_3A-19A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-3A-3A-19A | CA_1A-3A CA_1A-19A6 CA_3A-19A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-3A-26A | CA_1A-3A, CA_1A-26A, CA_3A-26A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-3A-20A | CA_1A-3A, CA_3A-20A, CA_1A-20A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-3A-20A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3C-20A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-21A | CA_1A-3A, CA_1A-21A, CA_3A-21A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-3A-3A-21A | CA_1A-3A, CA_1A-21A, CA_3A-21A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-3A-28A | CA_1A-3A, CA_1A-28A, CA_3A-28A6 | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-1A-3A-28A | - | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-3A-28A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3C-28A | CA_3C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-32A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-38A | CA_1A-3A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3C-38A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-40A | CA_1A-3A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-40C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3C-40A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3C-40C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-41A9 | CA_1A-3A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 41 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-41C9 | CA_1A-3A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-41D9 | CA_1A-3A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-42A | CA_1A-3A, CA_1A-42A, CA_3A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-3A-42A | CA_1A-3A, CA_1A-42A, CA_3A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-42C | CA_1A-3A, CA_1A-42A, CA_1A-42C, CA_3A-42A, CA_3A-42C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-42D | CA_1A-3A, CA_1A-42A, CA_3A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-43A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 50 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 43 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-46A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |
|  | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_1A-3A-46C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-46D | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-46E | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46E in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-5A-40A | CA_1A-5A6 | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-5A-41A11 | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_1A-5A-46A | CA_1A-5A6 | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_1A-5A-7A | CA_1A-5A6 CA_1A-7A CA_5A-7A | 1 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 1 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-5A-7A-7A | CA_1A-5A6 CA_1A-7A CA_5A-7A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-5A-46C | CA_1A-5A6 | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-5A-46D | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-8A | CA_1A-7A, CA_1A-8A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-7A-8A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-20A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 1 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 2 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7C-20A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7A-26A | CA_1A-7A CA_1A-26A, CA_7A-26A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-7A-7A-26A | CA_1A-7A CA_1A-26A, CA_7A-26A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-7A-28A | CA_1A-7A, CA_1A-28A, CA_7A-28A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 1 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 2 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7A-7A-28A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7C-28A | CA_1A-7A, CA_1A-28A, CA_7A-28A, CA_7C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7A-32A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7A-38A16 | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7A-40A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7A-40C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-42A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7A-46A | CA_1A-7A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_1A-7A-46A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_1A-7A-46C | CA_1A-7A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-46C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-46D | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-46D | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-46E | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46E in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-8A-11A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-8A-20A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-8A-28A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-8A-38A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-8A-40A | CA_1A-8A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-8A-40C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-11A-18A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 1 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-11A-28A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-18A-28A | CA_1A-18A6 CA_1A-28A CA_18A-28A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 1 |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-18A-42A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-18A-42C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-19A-21A | CA_1A-19A6 CA_1A-21A CA_19A-21A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-19A-28A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-19A-42A | CA_1A-19A6, CA_1A-42A, CA_19A-42A6 | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-19A-42C | CA_1A-19A6 CA_1A-42A CA_19A-42A6 | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-20A-28A12 | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-20A-32A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-20A-42A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-20A-43A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 40 | 0 |
|  |  | 20 |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 43 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-21A-28A | CA_1A-21A, CA_1A-28A, CA_21A-28A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-21A-42A | CA_1A-21A, CA_1A-42A, CA_21A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-21A-42C | CA_1A-21A CA_1A-42A CA_21A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-21A-42D | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 95 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-28A-40A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-28A-40C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-28A-42A | CA_1A-28A, CA_1A-42A, CA_28A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-28A-42C | CA_1A-28A, CA_1A-42A, CA_28A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-32A-42A | - | 1 |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  | 55 | 0 |
|  |  | 32 |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |
|  |  | 42 |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |
| CA_1A-32A-43A | - | 1 |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  | 55 | 0 |
|  |  | 32 |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |
|  |  | 43 |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |
| CA_1A-41A-42A10 | CA_1A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-41A-42C10 | CA_1A-42A, CA_42C, CA_1A-42C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-41C-42A10 | CA_1A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-41C-42C10 | CA_1A-42A, CA_42C, CA_1A-42C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42A-43A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 55 | 0 |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 43 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-4A-5A | CA_2A-4A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-4A-5A | CA_2A-5A CA_4A-5A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-12A-66A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-14A-66A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-5B | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-7A | CA_2A-4A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-4A-7A-7A | CA_2A-4A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 | See the CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-7C | . | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-4A-5A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-12A | CA_2A-4A CA_4A-12A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-12A-12A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-12B | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-4A-12A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-4A-12A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-13A | CA_2A-13A CA_4A-13A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 13 |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-28A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-4A-29A | CA_2A-4A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-30A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-71A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-4A-71A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-5A-7A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-5A-12A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5A-12A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-12A-12A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-46C | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-5A-66A-66A | CA_2A-5A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5A-66B | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5A-66C | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-7A-12A | - | 2 | See CA_2A-2A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-7A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-12B-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-13A-66A | CA_2A-13A CA_13A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-5A-12B | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-13A | CA_2A-13A6 | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-28A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-5A-29A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-30A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5A-30A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2C-5A-30A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5B-30A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2C-5B-30A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-46A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_2A-5A-46D | CA_2A-5A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-46E | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 110 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-5A-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5B-66A-66A | CA_2A-5A CA_5A-66A | 2 |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes | 80 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-66B |  | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-66C |  | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-66D | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5B-66A |  | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-5B-66B | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5B-66C | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5B-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5B-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-7A-12A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-12B | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 | See CA_12B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-28A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-7A-30A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-46A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |
| CA_2A-7A-7A-46A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_2A-7A-46C | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-7A-46C | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-46D | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-7A-46D | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-46E | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-7A-46E | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 140 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-7A-7A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-7C-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7C-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-7C-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-12A-30A | CA_2A-12A6 | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-12A-30A | - | 2 | See CA_2A-2A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2C-12A-30A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-12A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 2 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 1 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-12A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-12A-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-12A-66C | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-12B-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-12B-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_2A-13A-46C | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46D | CA_2A-13A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46E | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 110 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-48A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-13A-48A-48A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-48C | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-48D | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-48A-48C | CA_2A-13A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-66A | CA_2A-13A CA_13A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-13A-66D | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-66A-66A | CA_2A-13A CA_13A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-66A-66B | CA_2A-13A CA_13A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66B Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-66A-66C | CA_2A-13A CA_13A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-66B | CA_2A-13A CA_13A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-66C | CA_2A-13A CA_13A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-13A-66B | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-13A-66A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-14A-30A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-14A-30A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-14A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-14A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-14A-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-14A-66A-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-29A-30A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-29A-30A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2C-29A-30A | - | 2 | See CA_2C Bandwidth Combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-29A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-30A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-30A-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-30A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46A-48A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46A-48C | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46A-48D | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46A-48E | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 48 | See the CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46C-48A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46C-48C | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46D-48A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46A-46A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 46 | See CA_46A-46A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46C-48D | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46C-48E | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 140 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46C-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46A-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 | See the CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46C-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See the CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46D-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 46 | See the CA_46D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See the CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46E-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 140 | 0 |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See the CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46A-46C-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 46 | See CA_46A-46C Bandwidth Combination Set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46D-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46D-48C | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46E-48A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46E-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46E-48C |  | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 140 | 0 |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48A-66A | CA_48A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-48C-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-48D-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-48E-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 48 | See CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-48A-48A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-48A-48C-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 48 | See CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-48A-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-66A-71A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-66A-71A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-66A-66A-71A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-66C-71A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-5A-7A | CA_3A-5A, CA_3A-7A, CA_5A-7A | 3 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-5A-7A-7A | CA_3A-5A, CA_3A-7A, CA_5A-7A | 3 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-5A-28A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-5A-40A | CA_3A-5A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 3 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 1 |
|  |  | 5 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_3A-5A-40A-40A | - | 3 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-5A-41A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_3C-7A-8A | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-7A-8A | CA_3A-7A, CA_3A-8A, CA_7A-8A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-7A-7A-8A | CA_3A-7A, CA_3A-8A, CA_7A-8A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 2 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-7A-8A | CA_3A-7A, CA_3A-8A, CA_7A-8A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 2 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-8A | CA_3A-7A, CA_3A-8A, CA_7A-8A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 40 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 1 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 2 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-20A | CA_3A-7A CA_3A-20A CA_7A-20A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-3A-7A-20A | - | 3 | See CA_3A-3A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-7A-20A | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-7C-20A | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7C-20A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7A-26A | CA_3A-7A, CA_3A-26A, CA_7A-26A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_3A-7A-7A-26A | CA_3A-7A, CA_3A-26A, CA_7A-26A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_3A-7A-28A | CA_3A-7A, CA_3A-28A6, CA_7A-28A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-3A-7A-28A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-3A-7C-28A | 7C | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7A-7A-28A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7C-28A | CA_3A-7A, CA_7C, CA_7A-28A | 3 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 3 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 1 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-7A-28A | CA_3C | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-7C-28A | CA_3C CA_7C | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7A-32A | CA_3A-7A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-7A-32A | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7A-38A7 | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-7A-38A7 | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7A-40A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7A-40C | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-42A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7A-46A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_3A-7A-46C | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-46D | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-46E | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-8A-11A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-8A-20A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-8A-28A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-8A-32A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-8A-38A | CA_3A-8A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-8A-38A | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-8A-40A | CA_3A-8A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-8A-40C | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-11A-18A | CA_3A-11A, CA_3A-18A, CA_11A-18A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_3A-11A-26A | CA_3A-11A, CA_3A-26A, CA_11A-26A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_3A-11A-28A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-18A-42A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-18A-42C | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-19A-21A | CA_3A-19A, CA_3A-21A, CA_19A-21A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_3A-3A-19A-21A | CA_3A-19A, CA_3A-21A, CA_19A-21A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_3A-19A-42A | CA_3A-19A, CA_3A-42A, CA_19A-42A6 | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-19A-42C | CA_3A-19A CA_3A-42A CA_19A-42A6 | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-19A-42D | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 95 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-20A-28A12 | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-3A-20A-28A12 | - | 3 | See CA_3A-3A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 20 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-20A-28A12 | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 20 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-20A-32A | CA_3A-20A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-20A-42A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-20A-43A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 40 | 0 |
|  |  | 20 |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 43 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-21A-28A | CA_3A-21A, CA_3A-28A6, CA_21A-28A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-21A-42A | CA_3A-21A, CA_3A-42A, CA_21A-42A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-21A-42C | CA_3A-21A, CA_3A-42A, CA_21A-42A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-21A-42D | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 95 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-38A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-28A-38A | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-28A-40A | CA_3A-28A6 | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-28A-40C | CA_3A-28A6 | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-40D | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-41A | CA_3A-41A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 41 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-28A-41C | CA_3A-41A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-42A | CA_3A-28A6, CA_3A-42A, CA_28A-42A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-28A-42A-42A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42A-42A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-42C | CA_3A-28A6, CA_3A-42A, CA_28A-42A, CA_42C | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-42A-42C | CA_42C | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42A-42C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-42C-42C | CA_42C | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 110 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C-42C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-42D | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-32A-42A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 55 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-32A-43A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 55 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 43 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-32A-46A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_3A-32A-46C | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-32A-46D | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-32A-46E | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46E in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41A-42A | CA_3A-41A, CA_41A-42A, CA_3A-42A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-41A-42A-42A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42A-42A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41A-42C | CA_3A-41A, CA_3A-41C, CA_3A-42A, CA_41A-42A, CA_41A-42C, CA_42C | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41A-42A-42C | CA_42C | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42A-42C Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41A-42C-42C | CA_42C | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42C-42C Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41C-42A | CA_3A-41A, CA_3A-41C, CA_3A-42A, CA_41A-42A, CA_41C CA_41C-42A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-41C-42C | CA_3A-41A, CA_3A-41C, CA_3A-42A, CA_3A-42C, CA_41A-42A, CA_41A-42C CA_41C, CA_41C-42A, CA_42C | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42A-43A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 55 | 0 |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 43 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_4A-5A-12A | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-5A-12A-12A | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-5A-12B | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-5A-12A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-5A-13A | CA_4A-13A6 | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-5A-29A | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-5A-30A | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-5A-30A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-5B-30A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-5B-30A | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-7A-12A | - | 4 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-7A-28A | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_4A-12A-30A | CA_4A-12A | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-12A-30A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-29A-30A | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-29A-30A | - | 4 | See CA_4A-4A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_5A-7A-28A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-7A-46A | CA_5A-7A | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_5A-7A-46C | CA_5A-7A | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-7A-46D | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-12A-46A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_5A-12A-46C | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-12A-46D | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-12A-48A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-12A-48C | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-12A-48D | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-30A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-30A-66A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-30A-66A | - | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5B-30A-66A-66A | - | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-46A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-46E-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 110 | 0 |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-46A-66A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-46C-66A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-46D-66A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 110 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-46E-66A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 130 | 0 |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-8A-20A | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_7A-8A-38A13 | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-8A-40A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-12A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-40A-41A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 40 |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_5A-46C-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-46D-66A | CA_5A-46A CA_5A-66A | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-48A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-48A-66A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-8A-40C | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-12A-66A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-12B-66A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 12 | See CA_12B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-20A-28A12 | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-20A-32A | CA_7A-20A | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-20A-38A8 | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-28A-40A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-28A-40C | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-20A-42A | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-28A-38A14 | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-29A-66A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-7A-29A-66A | - | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7C-29A-66A | - | 7 | See CA_7C Bandwidth combination set 2 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-30A-66A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-32A-46A | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_7A-32A-46C | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-32A-46D | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-32A-46E | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46E in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-46A-66A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_8A-11A-28A | - | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_8A-20A-28A15 | - | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 20 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_8A-28A-41A | - | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 41 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_8A-39A-41A | - | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 39 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_12A-30A-66A | - | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_12A-30A-66A-66A | - | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46A-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-46A-66A-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46C-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-46C-66A-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46D-66A | CA_13A-66A | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-46D-66A-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 110 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46E-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 110 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-48A-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-48A-48A-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-48C-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-48D-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-48E-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 110 | 0 |
|  |  | 48 | See CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-48A-48C-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48A-48C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-48A-66A-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48A-66B | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48A-66C | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_14A-30A-66A | - | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_14A-30A-66A-66A | - | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_19A-21A-42A | CA_19A-21A, CA_19A-42A6, CA_21A-42A | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 50 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_19A-21A-42C | CA_19A-21A, CA_19A-42A6, CA_21A-42A | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 70 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-32A-42A | - | 20 |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | 45 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_20A-32A-43A | - | 20 |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | 45 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 43 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_20A-38A-40A |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 55 | 0 |
|  |  | 38 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_20A-38A-40A-40A | - | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 75 | 0 |
|  |  | 38 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-38A-40C | - | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 75 | 0 |
|  |  | 38 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-38A-40D | - | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 95 | 0 |
|  |  | 38 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40D Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-28A-42A | CA_21A-28A, CA_21A-42A, CA_28A-42A | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 45 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_21A-28A-42C | CA_21A-28A, CA_21A-42A, CA_28A-42A | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 65 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-26A-41A | - | 25 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 26 | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_25A-25A-26A-41A | - | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 65 | 0 |
|  |  | 26 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_25A-25A-26A-41C | - | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 85 | 0 |
|  |  | 26 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-26A-41C | - | 25 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 26 | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-41A-42A | CA_41A-42A | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_28A-41A-42A-42A | - | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-41A-42C | CA_41A-42A, CA_42C | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-41A-42A-42C | CA_42C | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42A-42C Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-41A-42C-42C | CA_42C | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 110 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42C-42C Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-41C-42A | CA_41A-42A | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_28A-41C-42C | CA_42C | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_29A-30A-66A | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_29A-30A-66A-66A | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_29A-46A-66A | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_29A-66A-70A | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 45 | 0 |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 70 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_29A-66A-66A-70A | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 65 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 70 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_29A-66A-70C | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 70 | See CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_29A-66A-66A-70C | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 66 | See the CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_29A-66C-70A | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 65 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 70 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_29A-66C-70C | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 66 | See the CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_32A-42A-43A | - | 32 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  | 60 | 0 |
|  |  | 42 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |
|  |  | 43 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |
| CA_46A-48A-66A | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | 60 | 0 |
|  |  | 48 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |
|  |  | 66 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |
| CA_46A-48A-71A | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | 60 | 0 |
|  |  | 48 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |
|  |  | 71 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |
| CA_46C-48A-48A-71A | - | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46A-48C-66A | - | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | 80 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46A-48D-66A | - | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | 100 | 0 |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46A-48E-66A | - | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | 120 | 0 |
|  |  | 48 | See the CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46C-48A-66A | - | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46C-48C-66A | - | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46C-48D-66A | - | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46C-48E-66A | - | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 | 0 |
|  |  | 48 | See the CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46D-48A-66A | - | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46D-48C-66A | - | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46E-48A-66A | - | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46E-48C-66A | - | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46A-48A-48A-71A | - | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | 80 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46A-48C-71A | - | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | 80 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46C-48A-71A | - | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46C-48C-71A | - | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_66A-70A-71A | - | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 70 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_66C-70A-71A | - | 66 | See the CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 70 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_66A-70C-71A | - | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 65 | 0 |
|  |  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_66A-66A-70A-71A | - | 66 | See the CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 70 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_66A-66A-70C-71A | - | 66 | See the CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 85 | 0 |
|  |  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_66C-70C-71A | - | 66 | See the CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 85 | 0 |
|  |  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| NOTE 1: The CA Configuration refers to a combination of an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For each band combination, all combinations of indicated bandwidths belong to the set. NOTE 3: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. NOTE 4: A terminal which supports a DL CA configuration shall support all the lower order fallback DL CA combinations and it shall support at least one bandwidth combination set for each of the constituent lower order DL combinations containing all the bandwidths specified within each specific combination set of the upper order DL combination. NOTE 5: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 6: If the UE supports any uplink CA configuration for corresponding downlink CA configuration it shall support this uplink CA configuration. NOTE 7: UL carrier shall be supported in Band 3 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. NOTE 8: UL carrier shall be supported in Band 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] NOTE 9: UL carrier is only supported on Band 1 or Band 3 not Band 41 because the fall back mode 1UL/2DL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. NOTE 10: UL carrier is only supported on Band 1 or Band 42 not Band 41 because the fall back mode 1UL/2DL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. NOTE 11: UL carrier is only supported on Band 1 or Band 5 not Band 41 because the fall back mode 1UL/2DL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. NOTE 12: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. NOTE 13: UL carrier shall be supported in Band 8 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. NOTE 14: UL carrier shall be supported in Band 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. NOTE 15: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. NOTE 16: UL carrier shall be supported in Band 1 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.6A.1-2b: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (four bands)

----- Unchanged sections omitted -----

Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-46A CA_1A-3A-46C CA_1A-3A-46D CA_1A-3A-46E | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_1A-5A-7A-46A CA_1A-5A-7A-46C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-5A-46A CA_1A-5A-46C CA_1A-5A-46D | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-5A-46A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 46 |  |  |  |  |  | -90 |  |
| CA_2A-5A-46A-66A CA_2A-5A-46C-66A CA_2A-5A-46D-66A CA_2A-5A-46E-66A CA_2A-5A-46A-66A-66A CA_2A-5A-46C-66A-66A CA_2A-5A-46D-66A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_2A-7A-46A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_2A-13A-46A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 13 |  |  | -97 | -94 |  |  |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-7A-46A CA_1A-7A-46C CA_1A-7A-46D CA_1A-7A-46E | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-46A CA_1A-46C CA_1A-46D CA_1A-46E | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_2A-5A-46C | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-5A-46D CA_2A-5A-46E | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 46 |  |  |  |  |  | -90 |  |
| CA_2A-7A-46A CA_2A-7A-7A-46A CA_2A-7A-46C CA_2A-7A-7A-46C CA_2A-7A-46D CA_2A-7A-7A-46D CA_2A-7A-46E CA_2A-7A-7A-46E | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_2A-13A-46C | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-13A-46A-66A CA_2A-13A-46C-66A CA_2A-13A-46D-66A CA_2A-13A-46A-66A-66A CA_2A-13A-46C-66A-66A CA_2A-13A-46D-66A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_2A-46A CA_2A-46E CA_2A-2A-46A CA_2A-2A-46D CA_2A-46C CA_2A-46A-46A CA_2A-2A-46C CA_2A-46A-46C CA_2A-46D CA_2A-46A-46D | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-46A-48A10,11 CA_2A-46A-48C10,11 CA_2A-46A-48D10,11 CA_2A-46A-48E10,11 CA_2A-46C-48A10,11 CA_2A-46C-48C10,11 CA_2A-46C-48D10,11 CA_2A-46C-48E10,11 CA_2A-46D-48A10,11 CA_2A-46D-48C10,11 CA_2A-46E-48A10,11 CA_2A-46E-48C10,11 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-46A-48A12 CA_2A-46A-48C12 CA_2A-46A-48D12 CA_2A-46A-48E12 CA_2A-46C-48A12 CA_2A-46C-48C12 CA_2A-46C-48D12 CA_2A-46C-48E12 CA_2A-46D-48A12 CA_2A-46E-48A12 CA_2A-46D-48C12 CA_2A-46E-48C12 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-46A-48A-66A10,11 CA_2A-46A-48C-66A10,11 CA_2A-46A-48D-66A10,11 CA_2A-46C-48A-66A10,11 CA_2A-46C-48C-66A10,11 CA_2A-46C-48D-66A10,11 CA_2A-46D-48A-66A10,11 CA_2A-46D-48C-66A10,11 CA_2A-46E-48A-66A10,11 | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| CA_2A-46A-48A-66A12 CA_2A-46A-48D-66A12 CA_2A-46C-48C-66A12 CA_2A-46C-48D-66A12 CA_2A-46D-48A-66A 12 CA_2A-46D-48C-66A12 CA_2A-46E-48A-66A12 | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| CA_2A-46A-48C-66A12 CA_2A-46C-48A-66A12 | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |  |
|  | 46 |  |  |  |  |  | -83 | TDD |  |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |  |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |  |
| CA_2A-46A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_2A-46A-46A-66A CA_2A-46C-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |  |
|  | 46 |  |  |  |  |  | -90 | TDD |  |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |  |
| CA_2A-46A-46C-66A CA_2A-46D-66A CA_2A-46E-66A CA_2A-46A-66A-66A, CA_2A-46C-66A-66A, CA_2A-46D-66A-66A, CA_2A-46E-66A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_2A-49A9 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 49 |  |  |  | -95.5 |  | -92.5 | TDD |
| CA_3A-7A-46C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-7A-46D | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-7A-46E | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-46A CA_3A-46C CA_3A-46D CA_3A-46E | 3 |  | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_3C-46A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-3A-46A CA_3C-46C CA_3C-46D | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-7A-46A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-32A-46A CA_3A-32A-46C CA_3A-32A-46D CA_3A-32A-46E | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | SDL |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-7A-32A-46A CA_3A-7A-32A-46C CA_3A-7A-32A-46D CA_3A-7A-32A-46E | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_4A-46A CA_4A-46C CA_4A-46A-46A CA_4A-46A-46C CA_4A-46D CA_4A-46A-46D | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-7A-46A CA_5A-7A-46C CA_5A-7A-46D | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-12A-46A, CA_5A-12A-46C, CA_5A-12A-46D | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 12 |  |  | -97 | -94 |  |  |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-46A CA_5A-46C CA_5A-46D CA_5A-46E CA_5B-46A CA_5B-46C CA_5B-46D CA_5B-46E | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_5A-46A-66A | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 |  |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_5A-46C-66A | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_5A-46D-66A, CA_5A-46E-66A, CA_5A-46A-66A-66A, CA_5A-46C-66A-66A, CA_5A-46D-66A-66A, CA_5A-46E-66A-66A | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 |  |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_7A-46A CA_7A-46C CA_7A-7A-46C CA_7A-46D CA_7A-46E CA_7C-46A CA_7C-46C CA_7C-46D | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_7A-7A-46A CA_7A-7A-46D CA_7A-7A-46E | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_7A-32A-46A CA_7A-32A-46C CA_7A-32A-46D CA_7A-32A-46E | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | SDL |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_7A-46A-66A | 7 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
|  | 66 |  |  | -99 | -96 | -94.2 | -93 | FDD |
| CA_8A-46A CA_8A-46D CA_8A-46E CA_8B-46C CA_8B-46D | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_8A-46C | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_8B-46A | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46A | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46C | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46D | 11 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46E | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_12A-46A CA_12A-46C CA_12A-46D | 12 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_12A-46E | 12 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_13A-46A CA_13A-46C CA_13A-46D CA_13A-46E | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_13A-46A-66A CA_13A-46C-66A CA_13A-46D-66A CA_13A-46A-66A-66A CA_13A-46C-66A-66A CA_13A-46D-66A-66A CA_13A-46E-66A | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_19A-46A CA_19A-46C CA_19A-46D CA_19A-46E | 19 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_21A-46A CA_21A-46C CA_21A-46D CA_21A-46E | 21 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_25A-46A CA_25A-46C CA_25A-46D | 25 | -101.2 | -98.2 | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_26A-46A | 26 |  | -99.7 | -97.57 | -94.57 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_28A-46A CA_28A-46C CA_28A-46D CA_28A-46E | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_29A-46A-66A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_39A-46A | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39A-46C | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39C-46A | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39A-46E CA_39C-46C CA_39C-46D | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 |  |
| CA_40A-46A CA_40A-46D CA_40A-46E CA_40C-46C CA_40C-46D CA_40D-46A CA_40D-46C | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_40A-46C | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_40C-46A | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_41A-46A CA_41A-46C CA_41A-46D CA_41A-46E CA_41C-46A CA_41C-46C CA_41C-46D CA_41D-46A CA_41D-46C | 41 |  |  | -98 | -95 | -93.2 | -92 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_42A-46A | 42 |  |  | -99 | -96 | -94.2 | -93 | TDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
| CA_46A-48A CA_46A-48A-48A CA_46A-48C CA_46A-48D CA_46A-48E CA_46C-48A CA_46C-48A-48A CA_46C-48C CA_46C-48D CA_46C-48E CA_46D-48A CA_46D-48C CA_46D-48A-48A CA_46E-48A CA_46E-48C | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| CA_46A-48A-66A10,11 CA_46A-48C-66A10,11 CA_46A-48D-66A10,11 CA_46A-48E-66A10,11 CA_46C-48A-66A10,11 CA_46C-48C-66A10,11 CA_46C-48D-66A10, 11 CA_46C-48E-66A10,11 CA_46D-48A-66A10,11 CA_46D-48C-66A 10,11 CA_46E-48A-66A10,11 CA_46E-48C-66A10,11 | 46 |  |  |  |  |  | -83 | FDD |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| CA_46A-48A-66A12 CA_46A-48C-66A12 CA_46A-48D-66A12 CA_46A-48E-66A12 CA_46C-48A-66A12 CA_46C-48C-66A12 CA_46C-48D-66A12 CA_46C-48E-66A12 CA_46D-48A-66A12 CA_46D-48C-66A 12 CA_46E-48A-66A12 CA_46E-48C-66A12 | 46 |  |  |  |  |  | -83 | FDD |
|  | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| CA_46A-48A-71A CA_46A-48A-48A-71A CA_46A-48C-71A CA_46C-48A-71A CA_46C-48C-71A CA_46C-48A-48A-71A | 46 |  |  |  |  |  | -83 | TDD |  |
|  | 48 |  |  | -99 | -96 | -94.2 | -93 | TDD |  |
|  | 71 |  |  | -97.2 | -94.2 | -92.0 | -87.5 | FDD |  |
| CA_46A-66A CA_46A-46A-66A CA_46A-66A-66A CA_46A-66C CA_46A-46C-66A CA_46A-46D-66A CA_46C-66A CA_46C-66A-66A CA_46D-66A CA_46D-66A-66A CA_46E-66A CA_46E-66A-66A | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_46A-70A | 46 |  |  |  |  |  | -90 | TDD |
|  | 70 |  |  | -100 | -97 | -95.2 |  | FDD |
| CA_46A-71A CA_46C-71A CA_46D-71A | 46 |  |  |  |  |  | -90 | FDD |
|  | 71 |  |  | -97.2 | -94.2 | -92.0 | -87.5 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1. NOTE 3: The signal power is specified per port. NOTE 4: Void NOTE 5: The requirement for B46 does not apply when there is at least one individual RE within the B46 downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 7.3.1A-0eC. NOTE 6: Void NOTE 7: 7 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 8: When Band 46 have self interference problems by dual uplink CA, then the requirements not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. NOTE 9: The requirement for B49 does not apply when there is at least one individual RE within the B49 downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 7.3.1A-0eD. NOTE 10: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band which excludes band 46 and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2-46-48, CA_46-48-66, and CA_2-46-48-66. For harmonic issue not related with band 46, the uplink configuration of CA_2-48, CA_48-66 and CA_2-48-66 in Table 7.3.1A-0b can be used. NOTE 11: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 12: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_1A-3A-46A; CA_1A-3A; 2*fc_1A + fc_3A; 2*BW_1A + BW_3A. CA_1A-3A-46A; CA_1A-3A; fc_1A – 2*fc_3A; BW_1A + 2*BW_3A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_1A-5A-46A; CA_1A-5A; 2*fc_1A + 2*fc_5A; 2*BW_1A + 2*BW_5A. CA_1A-7A-46A; CA_1A-7A; 3*fc_7A - fc_1A; 3*BW_7A + BW_1A. CA_5A-7A-46A; CA_5A-7A; 2*fc_7A + fc_5A; 2*BW_7A + BW_5A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_2A-5A-46D; CA_2A-5A; 3*fc_5A + 1*fc_2A; 3*BW_5A + 1*BW_2A. CA_2A-5A-46D; CA_2A-5A; 2*fc_5A – 3*fc_2A; 2*BW_5A + 3*BW_2A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_5A-46D-66A; CA_5A-66A; 3*fc_66A + 1*fc_5A; 3*BW_66A + 1*BW_5A. CA_5A-46D-66A; CA_5A-66A; 2*fc_66A – 3*fc_5A; 2*BW_66A + 3*BW_5A. CA_5A-46D-66A; CA_5A-66A; 2*fc_66A + 3*fc_5A; 2*BW_66A + 3*BW_5A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_13A-46D-66A; CA_13A-66A; 2*fc_66A + 2*fc_13A; 2*BW_66A + 2*BW_13A. CA_13A-46D-66A; CA_13A-66A; 2*fc_66A + 3*fc_13A; 2*BW_66A + 3*BW_13A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_2A-13A-46D; CA_2A-13A; 3*fc_13A + 1*fc_2A; 3*BW_13A + 1*BW_2A. |

----- Unchanged sections omitted -----

Table 7.3.1A-0g: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA | EUTRA CA | EUTRA band | UL Fc | UL BW | UL | DL Fc | DL BW | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB | (MHz) | (MHz) | (dB) |  |  |
| CA_1A-3A-28A | CA_1A-28A | 1 | 1975 | 5 | 25 | 2165 | 5 | N/A | FDD | N/A |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A |  | N/A |
|  |  | 3 | 1723.5 | 5 | 25 | 1818.5 | 5 | 4.0 |  | IMD5 |
|  | CA_3A-28A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD | N/A |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A |  | N/A |
|  |  | 1 | 1949 | 5 | 25 | 2139 | 5 | 11.0 |  | IMD4 |
| CA_1A-3A-40A | CA_1A-3A | 1 | 1950 | 5 | 25 | 2140 | 5 | N/A | FDD | N/A |
|  |  | 3 | 1735 | 5 | 25 | 1830 | 5 | N/A | FDD | N/A |
|  |  | 40 | 2380 | 5 | 25 | 2380 | 5 | 8.0 | TDD | IMD5 |
| CA_1A-3A-41A | CA_1A-3A | 1 | 1977.5 | 5 | 25 | 2167.5 | 5 | N/A | FDD | N/A |
|  |  | 3 | 1712.5 | 5 | 25 | 1807.5 | 5 | N/A | FDD | N/A |
|  |  | 41 | 2507.5 | 5 | 25 | 2507.5 | 5 | 5.0 | TDD | IMD5 |
| CA_1A-3A-42A | CA_1A-3A | 1 | 1922.5 | 5 | 25 | 2112.5 | 5 | N/A | FDD | N/A |
|  |  | 3 | 1782.5 | 5 | 25 | 1877.5 | 5 | N/A | FDD | N/A |
|  |  | 42 |  |  |  | 3425 | 5 | 13.0 | TDD | IMD4 |
| CA_1A-5A-7A | CA_1A-7A | 1 | 1968 | 5 | 25 | 2158 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2512 | 10 | 50 | 2632 | 10 | N/A |  | N/A |
|  |  | 5 | 835 | 5 | 25 | 880 | 5 | 1.0 |  | IMD5 |
| CA_1A-5A-40A | CA_1A-5A | 1 | 1977.5 | 5 | 25 | 2167.5 | 5 | N/A | FDD | N/A |
|  |  | 5 | 826.5 | 5 | 25 | 871.5 | 5 | N/A | FDD | N/A |
|  |  | 40 | 2305 | 10 | 50 | 2305 | 10 | 9.0 | TDD | IMD4 |
| CA_1A-7A-26A | CA_1A-7A | 1 | 1965 | 5 | 25 | 2155 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 | N/A |  | N/A |
|  |  | 26 | 830 | 5 | 50 | 875 | 5 | 3.5 |  | IMD5 |
| CA_1A-7A-28A | CA_1A-7A | 1 | 1935 | 5 | 25 | 2125 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 | N/A |  | N/A |
|  |  | 28 | 730 | 10 | 50 | 785 | 10 | 4.5 |  | IMD5 |
|  | CA_1A-28A | 1 | 1935 | 5 | 25 | 2125 | 5 | N/A | FDD | N/A |
|  |  | 28 | 730 | 10 | 50 | 785 | 10 | N/A |  | N/A |
|  |  | 7 | 2545 | 10 | 50 | 2665 | 10 | 28.0 |  | IMD2 |
| CA_1A-28A-42A | CA_1A-28A | 1 | 1955 | 5 | 25 | 2145 | 5 | N/A | FDD | N/A |
|  |  | 28 | 735 | 5 | 25 | 790 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3425 | 5 | 25 | 3425 | 5 | 15.0 | TDD | IMD3 |
|  | CA_28A-42A | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3560 | 5 | 25 | 3560 | 5 | N/A | TDD | N/A |
|  |  | 1 | 1949 | 5 | 25 | 2139 | 5 | 11.0 | FDD | IMD3 |
| CA_2A-12A-30A | CA_2A-12A | 2 | 1885 | 5 | 25 | 1965 | 5 | N/A | FDD | N/A |
|  |  | 12 | 708.5 | 5 | 25 | 738.5 | 5 | N/A |  | N/A |
|  |  | 30 | 2308 | 5 | 25 | 2353 | 5 | 12.0 |  | IMD4 |
| CA_2A-2A-4A-5A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | N/A |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  | N/A |
|  |  | 4 | 1732 | 5 | 25 | 2132 | 5 | 7.6 |  | IMD4 |
| CA_2A-4A-13A | CA_2A-13A | 2 | 1855 | 5 | 25 | 1935 | 5 | N/A | FDD | N/A |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  | N/A |
|  |  | 4 | 1746 | 5 | 25 | 2146 | 5 | 7.6 |  | IMD4 |
|  | CA_4A-13A | 4 | 1750 | 5 | 25 | 2150 | 5 | N/A | FDD | N/A |
|  |  | 13 | 780 | 5 | 25 | 749 | 5 |  |  | N/A |
|  |  | 2 | 1860 | 5 | 25 | 1940 | 5 | 6.2 |  | IMD4 |
| CA_2A-2A-5A-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | N/A |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  | N/A |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | 7.2 |  | IMD4 |
| CA_2A-5B-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | N/A |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  | N/A |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | 7.2 |  | IMD4 |
| CA_2A-13A-66A-66B | CA_2A-13A | 2 | 1860 | 5 | 25 | 1940 | 5 | N/A | FDD | N/A |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  | N/A |
|  |  | 66 | 1736 | 5 | 25 | 2156 | 5 | 7.2 |  | IMD4 |
| CA_2A-48A-66A | CA_48A-66A | 2 | 1880 | 5 | 25 | 1960 | 5 | 28.3 | FDD-TDD | IMD2 |
|  |  | 48 | 3695 | 5 | 25 | 3695 | 5 | N/A |  | N/A |
|  |  | 66 | 1735 | 5 | 25 | 2135 | 5 | N/A |  | N/A |
| CA_3A-5A-7A | CA_3A-5A | 3 | 1780 | 10 | 50 | 1875 | 10 | N/A | FDD | N/A |
|  |  | 5 | 845 | 5 | 25 | 890 | 5 | N/A |  | N/A |
|  |  | 7 | 2505 | 10 | 50 | 2625 | 10 | 30.0 |  | IMD21 |
|  | CA_3A-7A | 3 | 1725 | 10 | 50 | 1820 | 10 | N/A | FDD | N/A |
|  |  | 7 | 2565 | 10 | 50 | 2685 | 10 | N/A |  | N/A |
|  |  | 5 | 840 | 5 | 25 | 885 | 5 | 19.0 |  | IMD3 |
| CA_3A-7A-8A | CA_3A-7A | 3 | 1735 | 5 | 25 | 1830 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2530 | 10 | 50 | 2650 | 10 |  |  | N/A |
|  |  | 8 | 895 | 5 | 25 | 940 | 5 | 18.0 |  | IMD3 |
|  | CA_3A-8A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD | N/A |
|  |  | 8 | 890 | 5 | 25 | 935 | 5 |  |  | N/A |
|  |  | 7 | 2550 | 10 | 50 | 2670 | 10 | 29.0 |  | IMD2+IMD34 |
| CA_3A-7A-20A | CA_3A-7A | 3 | 1737 | 5 | 25 | 1832 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2543 | 10 | 50 | 2663 | 10 | N/A |  | N/A |
|  |  | 20 | 847 | 10 | 20 | 806 | 10 | 10.5 |  | IMD2 |
|  | CA_3A-20A | 3 | 1775 | 10 | 50 | 1870 | 10 | N/A | FDD | N/A |
|  |  | 20 | 855 | 5 | 25 | 896 | 5 | N/A |  | N/A |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 | 26.0 |  | IMD21 |
| CA_3A-7A-26A | CA_3A-7A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2560 | 10 | 50 | 2680 | 10 | N/A |  | N/A |
|  |  | 26 | 835 | 5 | 25 | 880 | 5 | 17.5 |  | IMD3 |
| CA_3A-7A-26A | CA_3A-26A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD | N/A |
|  |  | 26 | 845 | 5 | 25 | 890 | 5 | N/A |  | N/A |
|  |  | 7 | 2505 | 10 | 50 | 2625 | 10 | 29.0 |  | IMD21 |
| CA_3A-7A-28A | CA_3A-7A | 3 | 1747 | 5 | 25 | 1842 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2543 | 5 | 25 | 2663 | 5 | N/A |  | N/A |
|  |  | 28 | 741 | 5 | 25 | 796.0 | 5 | 20.0 |  | IMD2 |
|  | CA_3A-28A | 3 | 1712.5 | 5 | 25 | 1807.5 | 5 | N/A | FDD | N/A |
|  |  | 28 | 743 | 5 | 25 | 798 | 5 | N/A |  | N/A |
|  |  | 7 | 2562 | 5 | 25 | 2682 | 5 | 17.0 |  | IMD3 |
|  | CA_7A-28A | 7 | 2543 | 5 | 25 | 2663 | 5 | N/A | FDD | N/A |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A |  | N/A |
|  |  | 3 | 1737.5 | 5 | 25 | 1832.5 | 5 | 26.0 |  | IMD2 |
| CA_3A-7A-32A | CA_3A-7A | 3 | 1775 | 5 | 25 | 1870 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 | N/A |  | N/A |
|  |  | 32 | - | - | - | 1470 | 5 | 10.5 |  | IMD4 |
| CA_3A-8A-38A | CA_3A-8A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD-TDD | N/A |
|  |  | 8 | 890 | 5 | 25 | 935 | 5 | N/A |  | N/A |
|  |  | 38 | 2610 | 5 | 25 | 2610 | 5 | 26.4 |  | IMD2 |
|  | CA_3A-8A | 3 | 1750 | 5 | 25 | 1845 | 5 | N/A | FDD-TDD | N/A |
|  |  | 8 | 900 | 5 | 25 | 945 | 5 | N/A |  | N/A |
|  |  | 38 | 2600 | 5 | 25 | 2600 | 5 | 15.7 |  | IMD3 |
| CA_3A-11A-18A | CA_3A-11A | 3 | 1725 | 5 | 25 | 1820 | 5 | N/A | FDD | N/A |
|  |  | 11 | 1440 | 5 | 25 | 1448 | 5 | N/A |  | N/A |
|  |  | 18 | 825 | 5 | 25 | 870 | 5 | 4.9 |  | IMD5 |
|  | CA_11A-18A | 11 | 1432 | 5 | 25 | 1481 | 5 | N/A | FDD | N/A |
|  |  | 18 | 820 | 5 | 25 | 865 | 5 | N/A |  | N/A |
|  |  | 3 | 1753 | 5 | 25 | 1848 | 5 | 4.0 |  | IMD5 |
| CA_3A-11A-26A | CA_3A-11A | 3 | 1725 | 5 | 25 | 1820 | 5 | N/A | FDD | N/A |
|  |  | 11 | 1440 | 5 | 25 | 1448 | 5 | N/A |  | N/A |
|  |  | 26 | 825 | 5 | 25 | 870 | 5 | 4.9 |  | IMD5 |
|  | CA_3A-26A | 3 | 1782.5 | 5 | 25 | 1877.5 | 5 | N/A | FDD | N/A |
|  |  | 26 | 816.5 | 5 | 25 | 861.5 | 5 | N/A |  | N/A |
|  |  | 11 | 1435.5 | 5 | 25 | 1483.5 | 5 | 5.0 |  | IMD5 |
|  | CA_11A-26A | 11 | 1440 | 5 | 25 | 1488 | 5 | N/A | FDD | N/A |
|  |  | 26 | 824 | 5 | 25 | 869 | 5 | N/A |  | N/A |
|  |  | 3 | 1761 | 5 | 25 | 1856 | 5 | 4.5 |  | IMD5 |
| CA_3A-19A-21A | CA_19A-21A | 19 | 832.5 | 5 | 25 | 877.5 | 5 | N/A | FDD | N/A |
|  |  | 21 | 1460.4 | 5 | 25 | 1508.4 | 5 | N/A |  | N/A |
|  |  | 3 | 1774.6 | 5 | 25 | 1869.6 | 5 | 4.0 |  | IMD5 |
| CA_3A-21A-28A | CA_3A-21A | 3 | 1782 | 5 | 25 | 1877 | 5 | N/A | FDD | N/A |
|  |  | 21 | 1451 | 5 | 25 | 1499 | 5 | N/A |  | N/A |
|  |  | 28 | 734 | 5 | 25 | 789 | 5 | 3.0 |  | IMD5 |
| CA_3A-28A-41A | CA_3A-41 | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD | N/A |
|  |  | 41 | 2510 | 5 | 25 | 2510 | 5 | N/A | TDD | N/A |
|  |  | 28 | 735 | 5 | 25 | 790 | 5 | 26.0 | FDD | IMD21 |
| CA_3A-41A-42A | CA_41A-42A | 41 | 2640 | 10 | 50 | 2640 | 10 | N/A | TDD | N/A |
|  |  | 42 | 3425 | 10 | 50 | 3425 | 10 |  | TDD | N/A |
|  |  | 3 | 1760 | 5 | 25 | 1855 | 5 | 16.0 | FDD | IMD3 |
| CA_5A-46D-66A | CA_5A_46D | 5 | 834 | 5 | 25 | 879 | 5 | N/A | FDD-TDD | N/A |
|  |  | 46 | 5491 | 20 | 100 | 5491 | 20 |  |  | N/A |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | 0.3 |  | IMD5 |
| CA_19A-21A-42A | CA_19A-21A | 19 | 842.5 | 5 | 25 | 887.5 | 5 | N/A | FDD | N/A |
|  |  | 21 | 1450.4 | 5 | 25 | 1498.4 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3508.7 | 5 | 25 | 3508.7 | 5 | 13.0 | TDD | IMD4 |
|  | CA_21A-42A | 21 | 1460.4 | 5 | 25 | 1508.4 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3500 | 5 | 25 | 3500 | 5 | N/A | FDD | N/A |
|  |  | 19 | 836.2 | 5 | 25 | 881.2 | 5 | 13.0 | TDD | IMD4 |
| CA_28A-41A-42A | CA_41A-42A | 41 | 2672 | 10 | 50 | 2672 | 10 | N/A | TDD | N/A |
|  |  | 42 | 3460 | 10 | 50 | 3460 | 10 |  | TDD | N/A |
|  |  | 28 | 733 | 5 | 25 | 788 | 5 | 26.0 | FDD | IMD2 |
| CA_1A-21A-42A6 | CA_1A-42A | 1 |  |  |  |  |  |  | FDD | N/A |
|  |  | 42 |  |  |  |  |  |  | TDD | N/A |
|  |  | 21 |  |  |  |  |  |  | FDD | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE 3: Void NOTE 4: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 5: For operations with 4 antenna ports, the MSD in the applicable bands shall be modified by the absolute value of ΔRIB,4R in Table 7.3.1-1a when MSD > 0. NOTE 6: Due to the spectrum holdings of the operator, the deployed frequency ranges do not result MSD to interested downlink channel. Therefore, no requirements apply for this CA configuration. |  |  |  |  |  |  |  |  |  |  |

<End of Changes>