3GPP TSG-RAN4 Meeting #91	R4-1906080

# Reno, USA, 13th – 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | 0109 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Blocking requirement for MSR/NR operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For NR CBW > 20MHz, a 20 MHz LTE interferer shall be used.But according to table 7.5.5.1.1-1, this interferer should be at 7.5 kHz distance from NR channel edge, which would make wanted NR and interferer LTE overlapping. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating distance in between wanted NR and LTE interferer for NR CBW > 20MHz so that they are not overlapping and blocking requirement could be correctly specified (frequency sepration should be 30 Mhz instead). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Blocking requirement for NR MSR won’t be correctly specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 37.105 CR 0141 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 37.141-1 CR 0160 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 7.5.5.1.1	General blocking test requirement

For the general blocking requirement, the interfering signal shall be a UTRA FDD signal as specified in clause A.1 in 3GPP TS 25.141 [10].

For RIBs supporting operation in non-contiguous spectrum, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 15 MHz. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least 15 MHz. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted and interfering signal coupled to the RIB, using the parameters in tables 7.5.5.1.1-1 and 7.5.5.1.1-2, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in subclause 7.2.5.4.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in subclause 7.2.5.2.

For multi-band RIBs, the requirement applies according to table 7.5.5.1.1-1 for the in-band blocking frequency ranges of each supported operating band.

Table 7.5.5.1.1-1: General blocking requirement

| Base Station Type | Mean power of interfering signal [dBm] | Wanted Signal mean power [dBm] (NOTE 1) | Centre Frequency of Interfering Signal | Interfering signal centre frequency minimum frequency offset from the Base Station RF Bandwidth edge or edge of sub-block inside a gap [MHz] |
| --- | --- | --- | --- | --- |
| Wide Area BS | -40 + y - ΔOTAREFSENS (NOTE 7, 9) | EISREFSENS + x dB (NOTE 2, 5, 9) | FUL_low - ΔfOOB to FUL_high + ΔfOOB | (±7.5+z) (Note 11) |
|  | -40 + y - ΔminSENS (NOTE 7, 10) | EISminSENS + x dB (NOTE 2, 5, 10) |  |  |
| Medium Range BS | -35 + y - ΔOTAREFSENS (NOTE 7, 9) | EISREFSENS + x dB (NOTE 3, 5, 9) |  |  |
|  | -35 + y - ΔminSENS (NOTE 7, 10) | EISminSENS + x dB (NOTE 3, 5, 10) |  |  |
| Local Area BS | -30 + y - ΔOTAREFSENS (NOTE 7, 9) | EISREFSENS + x dB (NOTE 4, 5, 9) |  |  |
|  | -30 + y - ΔminSENS (NOTE 7, 10) | EISminSENS + x dB (NOTE 4, 5, 10) |  |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 7.3 and 7.2. NOTE 2: For WA BS that does not support NR, “x” is equal to 6 in case of E-UTRA or UTRA wanted signals and equal to 3 in case of GSM/EDGE wanted signal. NOTE 3: For MR BS that does not support NR, “x” is equal to 6 in case of UTRA wanted signals, 9 in case of E-UTRA wanted signal and 3 in case of GSM/EDGE wanted signal. NOTE 4: For LA BS that does not support NR, “x” is equal to 11 in case of E-UTRA wanted signal, 6 in case of UTRA wanted signal and equal to 3 in case of GSM/EDGE wanted signal. NOTE 5: For a BS that supports NR but does not support UTRA, x is equal to 6. NOTE 6: For a BS capable of multi-band operation, “x” in Note 2, 3, 4, 5 applies in case of interfering signals that are in the in-band blocking frequency range of the operating band where the wanted signal is present or in an adjacent or overlapping band. For other in-band blocking frequency ranges of the interfering signal for the supported operating bands, “x” is equal to 1.4 dB. NOTE 7: For a BS that not supporting NR, “y” is equal to zero for all BS classes. For a BS that supports NR but does not support UTRA, “y” is equal to -3 for the WA and MR BS class and -5 for the LA BS class. NOTE 8: The downlink frequency range of an FDD operating band is excluded from the general blocking requirement. NOTE 9: This test requirement is only applied in the OTA REFSENS conformance test directions. NOTE 10: This test requirement is only applied in the OTA minSENS receiver target reference direction. NOTE 11: For NR wanted signal channel bandwidth greater than 20 MHz, z = 22.5. For all other cases, z = 0. |  |  |  |  |

Table 7.5.5.1.1-2: Void

NOTE:	The requirement in tables 7.5.5.1.1-1 and 7.5.5.1.1-2 assumes that two operating bands, where the downlink operating band (see table 4.4-1 and table 4.4-2 in 3GPP TS 37.141 [13].) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.