3GPP TSG RAN WG4 Meeting #91		R4-1906150

Reno, USA, 13th – 17th May 2019

Source: 	NTT DOCOMO, INC.

Title: 	Clarification on QCL assumption for RRM requirements

Agenda item:	6.10.8.3

Document for:	Discussion

- Introduction

At the last RAN4 meeting, almost all the requirements for L1-RSRP reporting were completed, but it was also agreed that RAN4 needs further clarification on QCL Type-D as highlighted by light blue as below. In this contribution, we provide our views on this issue.

Agreements in RAN4#90bis Agreement: for Rx beam sweeping for CSI-RS based L1-RSRP reporting, - When CSI-RS repetition is OFF, - For CSI-RS based L1-RSRP reporting in FR2, scaling factor N should be defined as following: - N = 1 could apply if TCI state is configured to all the CSI-RS resources in the resource set. - TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON - QCL Type-D needs further clarification in the next meeting - No requirements if TCI state is not configured - When CSI-RS repetition is ON, - The requirements apply provided TCI is provided for all resources in the resource set.

- Discussion

In TS 38.133, some of RRM requirements are defined based on QCL-TypeD assumption especially for the requirements related to whether UE needs to perform Rx beam sweeping. For example, if RS resource to be measured is QCL-TypeD with other RS resource on which UE can train Rx beams, UE does not need to perform additional Rx beam sweeping on that measured RS resource, i.e., N = 1 can apply to such requirements. Moreover, scheduling restriction would also happen if RS resource to be measured is not QCL-TypeD with DMRS of PDCCH and PDSCH. However, applicable QCL-TypeD assumption is not clearly mentioned in the current specification even though both direct and indirect QCL-TypeD relation between source and target RS resources could be applicable. Figure 1 (a) shows direct QCL-TypeD relation between CSI-RS for L1-RSRP reporting and SSB for L1-RSRP reporting, which is the valid configuration as N = 1 condition for L1-RSRP reporting based on the previous agreements. In this case, UE does not need to perform Rx beam sweeping for L1-RSRP reporting on the configured CSI-RS resources since UE would train Rx beam on the SSB configured for L1-RSRP reporting, and N = 1 can apply to CSI-RS based L1-RSRP reporting requirements. On the other hand, the condition that CSI-RS for L1-RSRP reporting is QCL-TypeD with SSB for L1-RSRP could also indicate indirect QCL relation as shown in Figure 1 (b). In this case, CSI-RS for L1-RSRP reporting is QCL-TypeD with another CSI-RS, e.g. CSI-RS for tracking, and the CSI-RS for tracking is also QCL Type D with SSB for L1-RSRP reporting. This case is also applicable since UE could assume same TRP-Tx beam for all RS resources and utilize same Rx beam. Hence, not only direct but also indirect QCL relation should be taken into consideration for RRM requirements related to QCL-TypeD assumption.

Observation 1: Both direct and indirect QCL-TypeD relation would be applicable for RRM requirements.

Proposal 1: In RRM requirements, QCL-TypeD should be applicable for both direct and indirect QCL relation.

Based on above discussion, we propose to introduce new section of applicability for QCL-TypeD assumption into TS 38.133 as in our companion contribution [1].

Figure 1: Direct and indirect QCL-TypeD relation between CSI-RS and SSB

- Conclusion

In this contribution, we provided our views on QCL assumption in RRM requirements, and we made following observation and proposal.

Observation 1: Both direct and indirect QCL-TypeD relation would be applicable for RRM requirements.

Proposal 1: In RRM requirements, QCL-TypeD should be applicable for both direct and indirect QCL relation.

# Reference

- 3GPP, R4-1906151, NTT DOCOMO, “[draft] Clarification CR on applicability for QCL assumption for RRM requirements,” May. 2019