3GPP TSG-RAN4 Meeting #91	R4-1906165

# Reno, US,  13th- 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0040 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-02: Addition of MU evaluation for testing output power, ACLR and OBUE in RC test method in subclause 12.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR adds the MU evalaution for testing output power, ACLR and OBUE in RC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of subclause 12.6.1.2.2.1A for FR2 output power - Addition of subclause 12.6.2.2.2.1A for FR2 ACLR - Addition of subclause 12.6.3.2.2.1A for FR2 OBUE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, RC cannot be used for output power, ACLR and OBUE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Subclause 12.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 12.6	Conformance testing for OTA in band TRP requirements

### 12.6.1	BS output power

#### 12.6.1.1	FR1

##### 12.6.1.1.1	General

For the frequency range up to 4.2 GHz, the same MU values as for E-UTRA were adopted [9]. It is expected that the test chamber setup, calibration and measurement procedures for E-UTRA and NR will be highly similar. All uncertainty factors were judged to be the same.

For the frequency range 4.2 - 6 GHz, all MU factors, including instrumentation related MU were judged to be the same as for the 3 - 4.2 GHz range, and thus the total MU for 4.2 – 6 GHz is the same as for 3 - 4.2 GHz. This assessment was made under the assumption of testing BS designed for licensed spectrum; for unlicensed spectrum the MU may differ.

##### 12.6.1.1.2	MU value

The MU value was agreed to be 1.4 dB for 0 – 3 GHz bands and 1.5 dB for 3 – 6 GHz bands. The MU in 4.2-6 GHz is valid for BS designed to operate in licensed spectrum.

##### 12.6.1.1.3	TT value

The TT value was agreed to be the same as the MU value.

#### 12.6.1.2	FR2

##### 12.6.1.2.1	General

The MU assessment was carried out using a CATR chamber only however other chamber types are not precluded if suitable MU assessment is done.

##### 12.6.1.2.2 	MU assessment

###### 12.6.1.2.2.1	CATR

A CATR MU budget was assessed in order to determine acceptable MU for the EIRP accuracy measurement in FR2. The CATR test setup and calibration and measurement procedures for FR2 are expected to be similar to those of FR1, although the test chamber dimensions and associated MU values will scale due to the shorter wavelengths and larger relative array apertures.

Table 12.6.1.2.2.1-1: Compact antenna test range uncertainty assessment for EIRP measurements for BS output power

| UID (Note 1) | Uncertainty Source | Uncertainty value 24.25<f <29.5GHz | Uncertainty value 37<f <40GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB)(dB) 24.25<f <29.5GHz | Standard uncertainty ui (dB)(dB) 37<f <40GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| E2-1 | Misalignment DUT & pointing error | 0.35 | 0.35 | Exp. normal | 2 | 1 | 0,174 | 0,174 |
| E2-2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.5 | 0.7 | Gaussian | 1 | 1 | 0,5 | 0,7 |
| E2-3 | Standing wave between DUT and test range antenna | 0.03 | 0.03 | U-shaped | √2 | 1 | 0,02 | 0,02 |
| E2-4 | RF leakage, test range antenna cable connector terminated. | 0.01 | 0.01 | Normal | 1 | 1 | 0,01 | 0,01 |
| E2-5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0,4 | 0.4 |
| E2-16 | Frequency flatness | 0.25 | 0.25 | Normal | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| E2-6 | Network Analyzer | 0.3 | 0.3 | Normal | 1 | 1 | 0,3 | 0,3 |
| E2-7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.43 | 0.57 | U-shaped | √2 | 1 | 0,21 | 0,26 |
| E2-8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0 | 0 |
| E2-4 | RF leakage, test range antenna cable connector terminated. | 0.01 | 0.01 | Normal | 1 | 1 | 0,01 | 0,01 |
| E2-9 | Influence of the calibration antenna feed cable | 0.21 | 0.29 | U-shaped | √2 | 1 | 0,15 | 0,2 |
| E2-10 | SGH Calibration uncertainty | 0.52 | 0.52 | Rectangular | √3 | 1 | 0,3 | 0,3 |
| E2-11 | Misalignment positioning system | 0.2 | 0.2 | Exp. normal | 2 | 1 | 0 | 0 |
| E2-1 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0,25 | 0,25 |
| E2-12 | Rotary joints | 0 | 0 | U-shaped | √2 | 1 | 0 | 0 |
| E2-3 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0,06 | 0,06 |
| E2-5 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0,009 | 0,009 |
| E2-15 | Switching uncertainty | 0.43 | 0.43 | Rectangular | √3 | 1 | 0,1 | 0,1 |
| Combined standard uncertainty (1σ) (dB)(dB) |  |  |  |  |  |  | 0,91 | 1,05 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB)(dB) |  |  |  |  |  |  | 1,78 | 2,06 |
| Note 1: UID are referenced to TR 37.843 [9]. |  |  |  |  |  |  |  |  |

###### 12.6.1.2.2.1A	Reverberation chamber

Table 12.6.1.2.2.1A-1: Reverberation chamber uncertainty assessment for TRP measurements for BS output power

| UID | Uncertainty source | Uncertainty value 24.25<f<29.5 GHz | Uncertainty Value 37<f<40 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) 24.25<f<29.5GHz | Standard uncertainty ui (dB) 37<f<40 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.26 | 0.37 | Gaussian | 1 | 1 | 0.26 | 0.37 |
| 2 | Impedance mismatch in the receiving chain | 0.20 | 0.20 | U-shaped | √2 | 1 | 0.14 | 0.14 |
| 3 | Random uncertainty | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.90 | 0.94 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.77 | 1.85 |
| NOTE: This MU budget is applicable if the data tests in procedure step 4) of the RC test procedure in [26] are fulfilled. |  |  |  |  |  |  |  |  |

###### 12.6.1.2.2.2	Summary

Based on the above evaluation and root sum square combining of the dB values for the MU and the SE (see subclause 12.10), the MU was decided to be 2.1 dB for the frequency range 24.25<f<29.5GHz and 2.4 dB for the frequency range 37<f<40GHz.

##### 12.6.1.2.3	TT values

The TT values were agreed to be the same as the MU values.

### 12.6.2 	Adjacent Channel Leakage Ratio (ACLR)

#### 12.6.2.1	FR1

##### 12.6.2.1.1	General

For the frequency range up to 4.2 GHz, the same MU values as for E-UTRA were adopted [9]. It is expected that the test chamber setup, calibration and measurement procedures for E-UTRA and NR will be highly similar. All uncertainty factors were judged to be the same.

For the frequency range 4.2 - 6 GHz, all MU factors, including instrumentation related MU were judged to be the same as for the 3 - 4.2 GHz range, and thus the total MU for 4.2 – 6 GHz is the same as for 3 - 4.2 GHz. This assessment was made under the assumption of testing BS designed for licensed spectrum; for unlicensed spectrum the MU may differ.

##### 12.6.2.1.2	MU value

For relative ACLR, the MU value was agreed to be 1.0 dB for 0 – 3 GHz bands and 1.2 dB for 3 – 6 GHz bands. The MU in 4.2-6 GHz is valid for BS designed to operate in licensed spectrum.

For absolute ACLR, the MU value was agreed to be 2.2 dB for 0 – 3 GHz bands and 2.7 dB for 3 – 6 GHz bands. The MU in 4.2-6 GHz is valid for BS designed to operate in licensed spectrum.

##### 12.6.2.1.3	TT value

For relative ACLR, the TT was agreed to be the same as the MU.

For absolute ACLR, the TT was agreed to be 0dB.

#### 12.6.2.2	FR2

##### 12.6.2.2.1	General

The MU assessment was carried out using a CATR chamber only. However, other chamber types are not precluded if suitable MU assessment is done.

##### 12.6.2.2.2	MU assessment

###### 12.6.2.2.2.1	CATR

A CATR MU budget was assessed in order to determine acceptable MU for the EIRP accuracy measurement in FR2. The CATR test setup and calibration and measurement procedures for FR2 are expected to be similar to those of FR1, although the test chamber dimensions and associated MU values will scale due to the shorter wavelengths and larger relative array apertures. However, it is noted that in order to achieve the test instrument uncertainties that were assumed, calibration of the spectrum analyzer may be needed.

For relative ACLR, the MU budget for CATR was assessed as follows:

Table 12.6.2.2.2.1-1	CATR uncertainty assessment for relative ACLR measurement

| Relative ACLR uncertainty budget |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID (Note 1) | Uncertainty Source | Uncertainty value 24.25<f <29.5GHz | Uncertainty value 37<f <40GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB)(dB) 24.25<f <29.5GHz | Standard uncertainty ui (dB)(dB) 37<f <40GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| E2-1 | Misalignment DUT & pointing error | 0,3 | 0,3 | Exp. normal | 2 | 1 | 0,15 | 0,15 |
| E2-2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.75 | 0.9 | Gaussian | 1 | 1 | 0.75 | 0.9 |
| E2-5 | QZ ripple with DUT | 0.4 | 0.4 | Normal | 1 | 1 | 0.4 | 0.4 |
| E2-16 | Frequency flatness | 0.25 | 0.25 | Normal | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| E2-6 | Network Analyzer | 0.3 | 0.3 | Normal | 1 | 1 | 0.3 | 0.3 |
| E2-7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.43 | 0.74 | U-shaped | √2 | 1 | 0.30 | 0.52 |
| E2-8 | Insertion loss variation in receiver chain | 0.18 | 0.20 | Rectangular | √3 | 1 | 0.10 | 0.12 |
| E2-15 | Switching uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Combined standard uncertainty (1σ) (dB)(dB) |  |  |  |  |  |  | 1.00 | 1.18 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB)(dB) |  |  |  |  |  |  | 1.96 | 2.31 |
| Note 1: UID are referenced to TR 37.843 [9]. |  |  |  |  |  |  |  |  |

For absolute ACLR, the MU budget for CATR was assessed as follows:

Table 12.6.2.2.2.1-2: Compact antenna test range uncertainty assessment for EIRP measurements for absolute ACLR

| UID (Note 1) | Uncertainty Source | Uncertainty value 24.25<f <29.5GHz | Uncertainty value 37<f <40GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB)(dB) 24.25<f <29.5GHz | Standard uncertainty ui (dB)(dB) 37<f <40GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| E2-1 | Misalignment DUT & pointing error | 0.3 | 0.3 | Exp. normal | 2 | 1 | 0,15 | 0,15 |
| E2-2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.9 | 0.9 | Gaussian | 1 | 1 | 0,9 | 0.9 |
| E2-3 | Standing wave between DUT and test range antenna | 0.03 | 0.03 | U-shaped | √2 | 1 | 0,02 | 0,02 |
| E2-4 | RF leakage, test range antenna cable connector terminated. | 0.01 | 0.01 | Normal | 1 | 1 | 0,01 | 0,01 |
| E2-5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0,4 | 0.4 |
| E2-16 | Frequency flatness | 0.25 | 0.25 | Normal | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| E2-6 | Network Analyzer | 0.2 | 0.2 | Normal | 1 | 1 | 0,2 | 0,2 |
| E2-7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.72 | 0.72 | U-shaped | √2 | 1 | 0,36 | 0,36 |
| E2-8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0 | 0 |
| E2-4 | RF leakage, test range antenna cable connector terminated. | 0.1 | 0.1 | Normal | 1 | 1 | 0,1 | 0,1 |
| E2-9 | Influence of the calibration antenna feed cable | 0.00 | 0.00 | U-shaped | √2 | 1 | 0,00 | 0,00 |
| E2-10 | SGH Calibration uncertainty | 0.5 | 0.5 | Rectangular | √3 | 1 | 0,29 | 0,29 |
| E2-11 | Misalignment positioning system | 0.2 | 0.2 | Exp. normal | 2 | 1 | 0 | 0 |
| E2-1 | Misalignment SGH and pointing error | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| E2-12 | Rotary joints | 0 | 0 | U-shaped | √2 | 1 | 0 | 0 |
| E2-3 | Standing wave between SGH and test range antenna | 0.03 | 0.03 | U-shaped | √2 | 1 | 0,02 | 0,02 |
| E2-5 | QZ ripple with SGH | 0.07 | 0.07 | Normal | 1 | 1 | 0,07 | 0,07 |
| E2-15 | Switching uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0,06 | 0,06 |
| Combined standard uncertainty (1σ) (dB)(dB) |  |  |  |  |  |  | 1.21 | 1,21 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB)(dB) |  |  |  |  |  |  | 2.37 | 2,37 |
| Note 1: UID are referenced to TR 37.843 [9]. |  |  |  |  |  |  |  |  |

###### 12.6.2.2.2.1A	Reverberation chamber

Table 12.6.2.2.2.1A-1: Reverberation chamber uncertainty assessment for relative ACLR measurement

| UID | Uncertainty source | Uncertainty value 24.25<f<29.5 GHz | Uncertainty Value 37<f<40 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) 24.25<f<29.5GHz | Standard uncertainty ui (dB) 37<f<40 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.26 | 0.37 | Gaussian | 1 | 1 | 0.26 | 0.37 |
| 2 | Impedance mismatch in the receiving chain | 0.20 | 0.20 | U-shaped | √2 | 1 | 0.14 | 0.14 |
| 3 | Random uncertainty | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.90 | 0.94 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.77 | 1.85 |
| NOTE: This MU budget is applicable if the data tests in procedure step 4) of the RC test procedure in [26] are fulfilled. |  |  |  |  |  |  |  |  |

Table 12.6.2.2.2.1A-2: Reverberation uncertainty assessment for EIRP measurements for absolute ACLR

| UID | Uncertainty source | Uncertainty value 24.25<f<29.5 GHz | Uncertainty Value 37<f<40 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) 24.25<f<29.5GHz | Standard uncertainty ui (dB) 37<f<40 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.40 | 0.60 | Gaussian | 1 | 1 | 0.40 | 0.60 |
| 2 | Impedance mismatch in the receiving chain | 0.20 | 0.20 | U-shaped | √2 | 1 | 0.14 | 0.14 |
| 3 | Random uncertainty | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.95 | 1.05 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.87 | 2.06 |
| NOTE: This MU budget is applicable if the data tests in procedure step 4) of the RC test procedure in [26] are fulfilled. |  |  |  |  |  |  |  |  |

###### 12.6.2.2.2.2	Summary

Based on the above evaluation and root sum square combining of the dB values for the MU and the SE (see subclause 12.10), the MU for absolute ACLR was decided to be 2.7 dB for the frequency range 24.25<f<29.5GHz and 2.7 dB for the frequency range 37<f<40GHz. The MU for relative ACLR was decided to be 2.3 dB for the frequency range 24.25<f<29.5GHz and 2.6 dB for the frequency range 37<f<40GHz.

##### 12.6.2.2.3	TT values

The TT was agreed to be the same as the MU.

### 12.6.3	Operating Band Unwanted Emissions (OBUE)

#### 12.6.3.1	FR1

##### 12.6.3.1.1	General

For the frequency range up to 4.2 GHz, the same MU values as for E-UTRA were adopted [9]. It is expected that the test chamber setup, calibration and measurement procedures for E-UTRA and NR will be highly similar. All uncertainty factors were judged to be the same.

For the frequency range 4.2 - 6 GHz, all MU factors, including instrumentation related MU were judged to be the same as for the 3 - 4.2 GHz range, and thus the total MU for 4.2 – 6 GHz is the same as for 3 - 4.2 GHz. This assessment was made under the assumption of testing BS designed for licensed spectrum; for unlicensed spectrum the MU may differ.

##### 12.6.3.1.2	MU value

The MU value was agreed to be 1.4 dB for 0 – 3 GHz bands and 1.5 dB for 3 – 6 GHz bands. The MU in 4.2-6 GHz is valid for BS designed to operate in licensed spectrum.

##### 12.6.3.1.3	TT value

The TT value was agreed to be the same as the MU value for 0-10MHz from the carrier and 0dB for >10MHz from the carrier.

#### 12.6.3.2	FR2

##### 12.6.3.2.1	General

The MU assessment was carried out using a CATR chamber only. However other chamber types are not precluded if suitable MU assessment is done.

##### 12.6.3.2.2	MU assessment

###### 12.6.3.2.2.1	CATR

A CATR MU budget was assessed in order to determine acceptable MU for the EIRP accuracy measurement in FR2. The CATR test setup and calibration and measurement procedures for FR2 are expected to be similar to those of FR1, although the test chamber dimensions and associated MU values will scale due to the shorter wavelengths and larger relative array apertures. However, it is noted that in order to achieve the test instrument uncertainties that were assumed, calibration of the spectrum analyzer may be needed.

Table 12.6.3.2.2.1-1: Compact antenna test range uncertainty assessment for EIRP measurements for occupied bandwidth unwanted emissions

| UID (Note 1) | Uncertainty Source | Uncertainty value 24.25<f <29.5GHz | Uncertainty value 37<f <40GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB)(dB) 24.25<f <29.5GHz | Standard uncertainty ui (dB)(dB) 37<f <40GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| E2-1 | Misalignment DUT & pointing error | 0.3 | 0.3 | Exp. normal | 2 | 1 | 0,15 | 0,15 |
| E2-2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.9 | 0.9 | Gaussian | 1 | 1 | 0,9 | 0.9 |
| E2-3 | Standing wave between DUT and test range antenna | 0.03 | 0.03 | U-shaped | √2 | 1 | 0,02 | 0,02 |
| E2-4 | RF leakage, test range antenna cable connector terminated. | 0.01 | 0.01 | Normal | 1 | 1 | 0,01 | 0,01 |
| E2-5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0,4 | 0.4 |
| E2-16 | Frequency flatness | 0.25 | 0.25 | Normal | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| E2-6 | Network Analyzer | 0.2 | 0.2 | Normal | 1 | 1 | 0,2 | 0,2 |
| E2-7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.72 | 0.72 | U-shaped | √2 | 1 | 0,36 | 0,36 |
| E2-8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0 | 0 |
| E2-4 | RF leakage, test range antenna cable connector terminated. | 0.1 | 0.1 | Normal | 1 | 1 | 0,1 | 0,1 |
| E2-9 | Influence of the calibration antenna feed cable | 0.00 | 0.00 | U-shaped | √2 | 1 | 0,00 | 0,00 |
| E2-10 | SGH Calibration uncertainty | 0.5 | 0.5 | Rectangular | √3 | 1 | 0,29 | 0,29 |
| E2-11 | Misalignment positioning system | 0.2 | 0.2 | Exp. normal | 2 | 1 | 0 | 0 |
| E2-1 | Misalignment SGH and pointing error | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| E2-12 | Rotary joints | 0 | 0 | U-shaped | √2 | 1 | 0 | 0 |
| E2-3 | Standing wave between SGH and test range antenna | 0.03 | 0.03 | U-shaped | √2 | 1 | 0,02 | 0,02 |
| E2-5 | QZ ripple with SGH | 0.07 | 0.07 | Normal | 1 | 1 | 0,07 | 0,07 |
| E2-15 | Switching uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0,06 | 0,06 |
| Combined standard uncertainty (1σ) (dB)(dB) |  |  |  |  |  |  | 1.21 | 1,21 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB)(dB) |  |  |  |  |  |  | 2.37 | 2,37 |
| Note 1: UID are referenced to TR 37.843 [9]. |  |  |  |  |  |  |  |  |

###### 12.6.3.2.2.1A	Reverberation chamber

Table 12.6.3.2.2.1A-1: Reverberation chamber uncertainty assessment for TRP measurements for occupied bandwidth unwanted emissions

| UID | Uncertainty source | Uncertainty value 24.25<f<29.5 GHz | Uncertainty Value 37<f<40 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) 24.25<f<29.5GHz | Standard uncertainty ui (dB) 37<f<40 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.40 | 0.60 | Gaussian | 1 | 1 | 0.40 | 0.60 |
| 2 | Impedance mismatch in the receiving chain | 0.20 | 0.20 | U-shaped | √2 | 1 | 0.14 | 0.14 |
| 3 | Random uncertainty | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.95 | 1.05 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.87 | 2.06 |
| NOTE: This MU budget is applicable if the data tests in procedure step 4) of the RC test procedure in [26] are fulfilled. |  |  |  |  |  |  |  |  |

###### 12.6.3.2.2.2	Summary

Based on the above evaluation and root sum square combining of the dB values for the MU and the SE (see subclause 12.10), the MU was decided to be 2.7 dB for the frequency range 24.25<f<29.5GHz and 2.7 dB for the frequency range 37<f<40GHz.

##### 12.6.3.2.3	TT value

The TT value was agreed to be the same as the MU (2.7dB) for 0-10% of the BS channel bandwidth away from the carrier, and 0dB for >10% of BS channel bandwidth from the carrier.