TSG-RAN Working Group 4 (Radio) meeting #91	R4-1906174

Reno, US, 13th – 17th May 2019

Source:	Ericsson

Title:	TP to TR 38.820: Addition of BS requirement sets in subclause 8.2

Agenda item:	10.4.7

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#90bis in Xian) a skeleton structure for the technical report TR 38.820 was approved in [1]. At previous meeting the need for support for multiple NR BS types within the frequency range 7 to 24 GHz have been discussed in [2, 3].

In this contribution we continue to drive the need to handle different types of requirement set types, e.g. Conducted, Hybrid and OTA. At the end on this contribution a text proposal for TR 38.820, subclause 8.2 is provided for approval.

# Discussion

Traditionally, RF core specifications developed by 3GPP RAN4 defines operating bands for which unique requirements with corresponding requirement pass/fail levels are defined. The differentiation of requirements with respect to operating bands was sufficient for traditional base station where all requirements were defined at a common anchor point, the RF connector. When NR specifications was developed for base stations, it was decided to introduced BS types to differentiate requirements for traditional base stations, AAS base stations without access to connectors and millimetre wave AAS base stations. To allow for support of millimetre wave frequencies RAN4 introduced the concept of frequency range. The frequency domain was divided into two frequency regions, according to Table 2-1.

Table 2-1: Frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1 | 410 – 7125 |
| FR2 | 24250 – 52600 |

The intension with the frequency ranges (FR1 and FR2) was to differentiate requirement applicability and levels between base stations operating within FR1 and FR2. Within each frequency range, specific operating bands attached with specific requirements levels are defined for both UE and BS.

In addition, of operating bands and frequency ranges, the current BS NR RF core specification also supports multiple requirement sets for different types of base station implementations. In was decided to categorize base station requirements based on where the requirements are defined as requirement sets. In Table 2-2, currently defined requirement sets are described.

Table 2-2: Requirement anchor point, and requirement set category

| Requirement set category | Anchor point | Description |
| --- | --- | --- |
| C | TAB | All requirements are defined at the RF connector. This is the traditional approach used in TS 36.104 and TS 37.104. |
| H | TAB and RIB | All requirements defined for requirement category C is applicable. However, emission requirement is scaled to capture the whole system and TX IMD is extended. In addition, two OTA requirements are defined for output power and sensitivity. The category is referred to as hybrid, since both conducted, and OTA is included. This requirement set category was introduced in TS 37.105 for AAS BS. |
| O | RIB | All requirements are defined as OTA requirements at the RIB. This requirement set category was introduced in TS 37.105 for eAAS BS. |

From the description above, it can be noticed that the requirement levels, coverage and concepts are different between C, H and FR1 O and FR2 O.

Therefore, it was decided to categorize BS types supported be the specification as described in Table 2-3. The base station type notation is created by frequency range (from Table 2-1) and requirement set category (Table 2-2).

Table 2-3: BS type description

| BS type | Frequency range | Requirement set category | Description |
| --- | --- | --- | --- |
| 1-C | FR1 | C | NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors. |
| 1-H | FR1 | H | NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB. |
| 1-O | FR1 | O | NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB. |
| 2-O | FR2 | O | NR base station operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB. |

The BS type map towards different requirement set, where each set supports a complete set of requirements for a specific base station implementation. The BS type is constituted by combination of frequency range (FR1 or FR2) and requirement set category (Conducted, Hybrid or OTA). The BS type concept allows the specification to define different requirements in terms of the following examples;

- For FR1, multiple requirement sets are defined to support different types of base station implementations. E.g. non-AAS and AAS.

- For FR1, 1-C and 1-H have different requirement levels. E.g. emission levels are scaled to capture emission for a complete system for Hybrid requirements.

- Requirement levels will be different between requirement sets. E.g. different noise figure assumptions between FR1 and FR2.

- Requirement concepts may be different between requirements sets. E.g. the declaration of OTA reference sensitivity is different between FR1 and FR2.

- Requirements coverage may be different between requirements sets. E.g. OTA co-location requirement is defined for FR1, but not for FR2.

In Table 2-4, the applicability table from the current version of NR BS RF core specification is captured. The applicability table points out applicable requirement subclauses for each requirement set. Not applicable requirement is marked not applicable (N/A). It must notice that all details are not covered in the applicability table. The intension is to define the requirement coverage.

Table 2-4: Rel-15 requirement applicability

| Requirement | Requirement set |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1-C | 1-H | 1-O | 2-O |
| BS output power | 6.2 | 6.2 | N/A | N/A |
| Output power dynamics | 6.3 | 6.3 |  |  |
| Transmit ON/OFF power | 6.4 | 6.4 |  |  |
| Transmitted signal quality | 6.5 | 6.5 |  |  |
| Occupied bandwidth | 6.6.2 | 6.6.2 |  |  |
| ACLR | 6.6.3 | 6.6.3 |  |  |
| Operating band unwanted emissions | 6.6.4 | 6.6.4 |  |  |
| Transmitter spurious emissions | 6.6.5 | 6.6.5 |  |  |
| Transmitter intermodulation | 6.7 | 6.7 |  |  |
| Reference sensitivity level | 7.2 | 7.2 |  |  |
| Dynamic range | 7.3 | 7.3 |  |  |
| In-band selectivity and blocking | 7.4 | 7.4 |  |  |
| Out-of-band blocking | 7.5 | 7.5 |  |  |
| Receiver spurious emissions | 7.6 | 7.6 |  |  |
| Receiver intermodulation | 7.7 | 7.7 |  |  |
| In-channel selectivity | 7.8 | 7.8 |  |  |
| Performance requirements | 8 | 8 |  |  |
| Radiated transmit power | N/A | 9.2 | 9.2 | 9.2 |
| OTA base station output power |  | N/A | 9.3 | 9.3 |
| OTA output power dynamics |  |  | 9.4 | 9.4 |
| OTA transmit ON/OFF power |  |  | 9.5 | 9.5 |
| OTA transmitted signal quality |  |  | 9.6 | 9.6 |
| OTA occupied bandwidth |  |  | 9.7.2 | 9.7.2 |
| OTA ACLR |  |  | 9.7.3 | 9.7.3 |
| OTA out-of-band emission |  |  | 9.7.4 | 9.7.4 |
| OTA transmitter spurious emission |  |  | 9.7.5 | 9.7.5 |
| OTA transmitter intermodulation |  |  | 9.8 | N/A |
| OTA sensitivity |  | 10.2 | 10.2 | N/A |
| OTA reference sensitivity level |  | N/A | 10.3 | 10.3 |
| OTA dynamic range |  |  | 10.4 | N/A |
| OTA in-band selectivity and blocking |  |  | 10.5 | 10.5 |
| OTA out-of-band blocking |  |  | 10.6 | 10.6 |
| OTA receiver spurious emission |  |  | 10.7 | 10.7 |
| OTA receiver intermodulation |  |  | 10.8 | 10.8 |
| OTA in-channel selectivity |  |  | 10.9 | 10.9 |
| Radiated performance requirements |  |  | 11 | 11 |

# Conclusion

As a first step, RAN4 needs to capture NR requirement set category and consider what support that is needed for the frequency range 7 to 24 GHz. As a starting point it is good to capture in TR 38.820, the current type of requirement anchor points used for FR1 and FR2 requirements. In a companion contribution [4], proposals for moving forward on BS types is presented.

It is proposed to approve the attached text proposal in TR 38.820, to capture NR BS requirement set categories in the technical report.

# References

[1]	R4-1903328, “TR 38.820: updated TR skeleton for 7-24GHz SI”, Huawei, RAN4#90bis

[2] 	R4-1900824, “On requirement set considerations in relation to FR1-FR2 intermediate frequency support”, Ericsson, RAN4#90

[3]	R4-1903484, “On BS types for the 7-24GHz range”, Ericsson, RAN4#90bis

[4]	R4-1906073, “Proposals for moving forward on BS types”, Ericsson

TEXT PROPOSAL:

# 8	NR BS requirements study

## 8.1	General

## 8.2	BS architecture and requirement types

Editor’s note:	Study applicability (or lack thereof) of conducted/hybrid/radiated requirement sets for these bands/frequencies, with a focus on how to define a boundary frequency and/or boundary conditions for conducted domain and OTA domain requirements.

•	Consider different BS architecture options. Note in Rel-15, there are non-AAS architectures (BS type 1-C for FR1) and AAS BS architectures (BS type 1-H, BS type 1-O for FR1 and BS type 2-O for FR2) for NR BS.

### 8.2.1	Requirement sets

The NR RF core specification defines multiple requirement sets. The requirement set support is different for FR1 and FR2. In table 8.2-1, the requirement set defined in NR are listed. Together, the requirement set category and frequency range defines the NR BS type.

Table 8.2.1-1: Requirement set category

| Requirement set category | Anchor point | Description |
| --- | --- | --- |
| C | TAB | All requirements are defined at the RF connector. This is the traditional approach used in TS 36.104 and TS 37.104. |
| H | TAB and RIB | All requirements defined for requirement category C is applicable. However, emission requirement is scaled to capture the whole system and TX IMD is extended. In addition, two OTA requirements are defined for output power and sensitivity. The category is referred to as hybrid, since both conducted, and OTA is included. This requirement set category was introduced in TS 37.105 for AAS BS. |
| O | RIB | All requirements are defined as OTA requirements at the RIB. This requirement set category was introduced in TS 37.105 for eAAS BS. |

Consider likely antenna array sizes to cover the identified deployment scenarios

## 8.2	BS RF

Editor’s note: Study RF characteristics for example frequencies including both TX and RX. In particular:

•	Study how to design TX emissions mask for these frequencies

•	Study the spurious region and its upper frequency limit for practical measurements

•	Study how to design TDD TX switching time requirement

•	Study need for requirement coverage; e.g. co-location related requirements (TX IMD etc.) for each example frequency

•	Study how to design requirements where approach differs between FR1 and FR2; e.g. RX sensitivity requirement, RX ACS, blocking, RX IMD requirements

## 8.3	BS EMC

Editor’s note: Investigate the need for EMC characteristics updates due to the existing EMC requirements and their frequency range limitations.