3GPP TSG-RAN WG4 Meeting #91              	R4-1906240

Reno, USA, May 13th – 17th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR section 8.1.2.3, 8.1.3.3 Requirement for simultaneous SSB and CSI-RS RLM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 3.5.2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NR RLM requirements are not complete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update requirement for overlapping CSI-RS and SSB for RLM under each section based on the agreement made in the WF and discussions during the meeting. - Compared to the CR draft that was almost agreeable in the last meeting, the changes are highlighted by green. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections 8.1.2.3, 8.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<Start of change 1>>

#### 8.1.2.3	Measurement restrictions for SSB based RLM

The UE shall be able to measure CSI-RS in same symbols as SSB according to the requirements in this section.

For FR1, when the SSB is within the active BWP and has same SCS than CSI-RS, the UE shall be able to perform SSB measurement without restrictions.

For FR1, when the SSB is within the active BWP and has different SCS than CSI-RS, the UE shall be able to perform SSB measurement with following restrictions according to its capabilities:

-	If CSI-RS and SSB are in the same symbol, the UE measurement capability depends on the whether the UE supports simultaneousRxDataSSB-DiffNumerology.

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform SSB measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology the UE is not expected to perform CSI-RS measurements,

For FR2, when the SSB is within the active BWP but is not within an SMTC, and has same SCS than CSI-RS, the UE shall be able to perform CSI-RS measurement without restrictions, provided that the SSB and CSI-RS are QCL TypeD and QCL information is known to the UE.

For FR2, when the SSB is within the active BWP but is not within an SMTC, and has different SCS than CSI-RS, the UE shall be able to perform CSI-RS measurement with the following restrictions according to its capabilities, provided that the SSB and CSI-RS are QCL TypeD and the QCL information is known to the UE:

-	If CSI-RS and SSB are in the same symbol, the UE measurement capability depends on the whether the UE supports simultaneousRxDataSSB-DiffNumerology.

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform SSB measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology the UE is not expected to perform CSI-RS measurements,

-	If CSI-RS and SSB are TDM’ed, the UE shall be able to perform CSI-RS measurement with restrictions: the UE is not expected to measure CSI-RS on symbols on 1 data symbol before each consecutive SSB symbols and 1 data symbol after each consecutive SSB symbols within the SMTC window duration

For FR1 and FR2, if the CSI-RS resource is in a resource set configured with repetition ON, longer evaluation period can be expected, and no requirements are defined.

<<End of change 1>>

<<Start of change 2>>

#### 8.1.3.3	Measurement restrictions for CSI-RS based RLM

The UE shall be able to measure CSI-RS in same symbols as SSB according to the requirements in this section.

For FR1, when the SSB is within the active BWP and has same SCS than CSI-RS, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB is within the active BWP and has different SCS than CSI-RS, the UE shall be able to perform CSI-RS measurement with following restrictions according to its capabilities:

-	If CSI-RS and SSB are in the same symbol, the UE measurement capability depends on the whether the UE supports simultaneousRxDataSSB-DiffNumerology.

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology the UE is not expected to perform CSI-RS measurements,

For FR2, when the SSB is within the active BWP but is not within an SMTC, and has same SCS than CSI-RS, the UE shall be able to perform CSI-RS measurement without restrictions, provided that the SSB and CSI-RS are QCL TypeD and QCL information is known to the UE.

For FR2, when the SSB is within the active BWP but is not within an SMTC, and has different SCS than CSI-RS, the UE shall be able to perform CSI-RS measurement with the following restrictions according to its capabilities, provided that the SSB and CSI-RS are QCL TypeD and the QCL information is known to the UE:

-	If CSI-RS and SSB are in the same symbol, the UE measurement capability depends on the whether the UE supports simultaneousRxDataSSB-DiffNumerology.

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology the UE is not expected to perform CSI-RS measurements,

-	If CSI-RS and SSB are TDM’ed, the UE shall be able to perform CSI-RS measurement with restrictions: the UE is not expected to measure CSI-RS on symbols on 1 data symbol before each consecutive SSB symbols and 1 data symbol after each consecutive SSB symbols within the SMTC window duration

For FR1 and FR2, if the CSI-RS resource is in a resource set configured with repetition ON, longer evaluation period can be expected, and no requirements are defined.

<<End of change 2>>