3GPP TSG-RAN WG4 Meeting #91	R4-1906276

# Reno, NV, USA, 13 – 17 May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | <CR#> | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of P-Max for FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For FR2 it is not possible to limit the UE output power by cell-specific indication if needed in deployments. The radiated power (EIRP) in the direction of maximum cannot be controlled but the TRP is not larger than the total conducted power that can be limited. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The TRP is limited by a cell-specific P-Max inidcation with an offset of a tentative 10 dB. The latter is included to modify the value range of the current P-Max IE from (-30..33) dBm to (-20..43) dBm more suitable for the power classes defined. This change does not require ASN.1 changes. Impact analysis: The implementation of this CR is conditioned on a transition period for conformance testing of UE designs compliant with Rel-15 (early UEs exempted). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to limit the UE power by cell-specific indication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

### 6.2.4	Configured transmitted power

The UE can configure its maximum output power. The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11].

The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds

PPowerclass – MAX(MAX(MPRf,c, A- MPRf,c,) + ΔMBP,n, P-MPRf,c) – MAX{T(MAX(MPRf,c, A- MPRf,c,)), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax

while the corresponding measured total radiated power PTMAX,f,c is bounded by

PTMAX,f,c ≤ MIN(PEMAX,f,c + [10], TRPmax)

with PPowerclass the UE power class as specified in sub-clause 6.2.1, EIRPmax the applicable maximum EIRP as specified in sub-clause 6.2.1, MPRf,c as specified in sub-clause 6.2.2 , A-MPRf,c as specified in sub-clause 6.2.3, ΔMBP,n the peak EIRP relaxation as specified in section 6.2.1, PEMAX,f,c the value given by the P-Max IE [13] and TRPmax the maximum TRP for the UE power class as specified in sub-clause 6.2.1.

P-MPRf,c is the allowed maximum output power reduction  and maxUplinkDutyCycle as defined in TS 38.331 [13] is the UEreported maximum duty cyle to facilitate the compliance described below. The evaluation period for maxUplinkDutyCycle is 10ms.

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPRf,c for carrier f of serving cell c only for the above cases. For UE conformance testing P-MPRf,c shall be 0 dB

NOTE 1:	P-MPRf,c  was introduced in the PCMAX,f,c equation such that the UE can report to the gNB the available maximum output transmit power. This information can be used by the gNB for scheduling decisions.

NOTE 2:	P-MPRf,c and maxUplinkDutyCycle may impact the maximum uplink performance for the selected UL transmission path.

The tolerance T(∆P) for applicable values of ∆P (values in dB) is specified in Table 6.2.4-1.

Table 6.2.4-1: PUMAX,f,c tolerance

| Operating Band | ∆P (dB) | Tolerance T(∆P) (dB) |
| --- | --- | --- |
| n257, n258, n260, n261 | P = 0 | 0 |
|  | 0 < P ≤ 2 | [1.5] |
|  | 2 < P ≤ 3 | [2.0] |
|  | 3 < P ≤ 4 | [3.0] |
|  | 4 < P ≤ 5 | [4.0] |
|  | 5 < P ≤ 10 | [5.0] |
|  | 10 < P ≤ 15 | [7.0] |
|  | 15 < P ≤ X | [8.0] |
| NOTE: X is the value such that Pumax,f,c lower bound, PPowerclass - P – T(P) = minimum output power specified in subclause 6.3.1 |  |  |

## 6.2A	Transmitter power for CA

< end of changes >