3GPP TSG-RAN WG4 Meeting #91 	R4-1906324

# Reno, NV, U.S., 13 – 17 May, 2019

Source:	Ericsson, Nokia, Intel

Title:	Outcome of ECC SE21#105 on Category B spurious emissions in ERC Recommendation 74-01

Agenda item:	6.7.2.5.1

Document for:	Information

# Background

CEPT/ECC SE21 finalized the update of ERC Recommendation 74-01 [1] on spurious emissions in December 2018. After approval by ECC WG SE, it was put on public consultation for six weeks and resolution of the public consultation comments was made at SE21 meeting #105 on 14-17 April.

This document summarizes the outcome of the Public Consultation resolution at SE21 #105. At the meeting, the mobile industry was represented by Ericsson, Nokia, Huawei, Intel and Qualcomm.

# Comments in Public Consultation and their resolution

The Public Consultation for ERC Recommendation 74-01 ended on 1 April. All comments were resolved at SE21 meeting #105 and the revised version of the recommendation is available in [2]. The following is a summary of the updates and new entries made to the recommendation that are relevant to 3GPP RAN4 and what additional changes was done due to the PC resolution.

In Recommends 6:

- Provisions for assessment of Active Antenna Systems through TRP measurements.

In Annex 2, Table A.2.1:

- General limits for terminals and base stations in item 2.1.1. (unchanged)

- New entry 2.1.6 for AAS BS operating below 6 GHz:

  - Reference to Note 4, defining the limits as TRP.

  - At PC resolution, it was agreed to restrict point 2.1.6 to cover operation below 6 GHz, instead of below 24.25 GHz.

  - It was also agreed to add a new Note to 2.16 and 2.1.7: “Note xx: There is currently no information on base stations using AAS and beamforming with integrated antennas operating between 6 and 24.25 GHz. These could be considered in a future revision as necessary.”

- New entry 2.1.7 for AAS BS operating above 24.25 GHz:

  - Existing limits apply up to 18 GHz.

  - A new limit -20 dBm/10 MHz apply above 18 GHz.

  - Reference is made to Figure 7 for specific frequency separations.

- New entry 2.1.8 for AAS UE operating above 24.25 GHz:

  - Existing limits apply up to 7.25 GHz.

  - Dual limits apply above 7.25 GHz: -13 dBm/MHz and -10 dBm/100 MHz.

  - Reference to Note 4, defining the limits as TRP.

  - Reference to Note 5 stating that both limits are simultaneously applicable.

  - At PC resolution, it was agreed to change the applicability of the new dual limits to be above 7.25 GHz instead of above 6 GHz.

- A new note 4 with a definition of TRP, applicable to all AAS limits.

  - At PC resolution, it was clarified that limits apply separately for Tx and Rx.

In Annex 2, Figure 7:

- A new mask applicable for AAS BS in the mmWave range (above 24.25 GHz; entry 2.1.7):

  - At PC resolution, the reference bandwidth was modified to consistently be 10 MHz across the mask, making the limit -5 dBm/10 MHz in the central part of the mask. Some editorial corrections were also made.

An excerpt from the output version of the recommendation [2] is given in the Annex.

# Summary

The information above and in the annex should be of interest for the continuing work in RAN WG4.

The Draft Revision of ERC Recommendation 74-01 [2] will be presented for final approval at the ECC WG SE meeting on 27-29 May 2019. The new version is then published by the ECC.

# References

- ERC Recommendation 74-01, “Unwanted emissions in the spurious domain”, Cardiff 2011, Published version (URL).

- CEPT/ECC, “Draft Revision of ERC Recommendation 74-01”, After PC resolution, Temporary document TEMP01rev5, ECC WG SE21 #105, Munich, Germany, 15-17 April 2019.

- “Minutes of SE21 meeting #105, Munich, Germany, 15-17 April, 2019”, Document SE21(18)126.

# ANNEX: 	Excerpt from the revised version of ERC Recommendation 74-01 
(after Public Consultation resolution)

The following excerpts are taken from the revised version of the recommendation [2] (Temporary document), to be included as Annex 6 in the meeting report [3] from ECC SE21 meeting #105.

Change marks given below are relative to the version that was put on public consultation.

- that the appropriate spurious domain emission limits should be applicable to all services/equipment as detailed by Table 2. Unless the Peak Envelope Power (PEP) is explicitly quoted in this Recommendation the spurious domain emission limits specified from the transmitter into the antenna port are in terms of mean power. The mean power (P) of any spurious domain transmission from a burst transmitter is the mean power averaged over the burst duration;

Note 1: 	In special cases, such as those referred in considering k),), tighter limits may be required;

Note 2:	It is recognised that in some cases of narrow-band and/or high power transmitters for all categories of services, there may be difficulties in meeting limits close to  250% of the necessary bandwidth. These cases are reported in the specific annexes referred to in Table 2.

Note 3: 	When a system is coupled toan "Active Antenna System", the limits of Table 2 should be met by the entire system; therefore compliance should be verified through an e.i.r.p. measurement (either near-field or far-field) and subsequent conversion to absolute power/attenuation values delivered to the transmission line, taking into account only the conventional "passive" gain (directivity) of the antenna, or as a TRP measurement (either near-field or far-field) as described in Annex 2.

LAND MOBILE SERVICE, MARITIME MOBILE SERVICE AND Short range devices requirements

Limits

Table 6: Spurious domain emission limits for the land mobile service, maritime mobile service and short range devices

| Reference number | Type of equipment | Frequency of the spurious domain emission f | Limits: mean power or, when applicable, average power during bursts duration in the reference bandwidth |
| --- | --- | --- | --- |
| 2.1.1 | Terminals and Base Stations (in transmit mode), except the equipment specified below | 9 kHz f 1 GHz | -36 dBm |
|  |  | 1 GHz < f Fupper (see recommends 3) | -30 dBm |
| 2.1.2 | Short range devices; RLAN; CB; Cordless Telephones; Radio Microphones (all systems in transmit mode) | f within the bands : 87.5-118 MHz, 174-230 MHz, 470-694 MHz | -54 dBm |
|  |  | 9 kHz f 1 GHz (except 87.5118 MHz, 174230 MHz, 470694 MHz) | -36 dBm |
|  |  | 1 GHz < f Fupper (see recommends 3) | -30 dBm |
| 2.1.3 | Short range inductive devices operating below 30 MHz (in transmit mode) (Note 6) () | 9 kHz f 10 MHz | 27 dBA/m (at 9 kHz then decreasing by 10 dB/decade) (Note 3) |
|  |  | 10 MHz < f 30 MHz | -3.5 dBA/m (Note 3) |
|  |  | f within the bands: 87.5-118 MHz, 174-230 MHz, 470-694 MHz | -54 dBm |
|  |  | 30 MHz < f 1 GHz (except 87.5118 MHz, 174230 MHz, 470694 MHz) | -36 dBm |
|  |  | 1GHz < f Fupper (see recommend 3) | -30 dBm |
| 2.1.4 | Receivers and idle/standby transmitters except the equipment specified below | 9 kHz f 1 GHz | - 57 dBm |
|  |  | 1 GHz < f Fupper (see recommends 11) | - 47 dBm |
| 2.1.5 | Short range inductive receivers and idle/standby transmitters operating below 30 MHz | 9 kHz ≤ f < 4.78 MHz | 5.5 dBµA/m, decreasing by 3 dB/octave (Note 3) |
|  |  | 4.78 MHz ≤ f < 30 MHz | -22 dBµA/m (Note 3) |
|  |  | 30 MHz ≤ f < 1 GHz | -57 dBm |
| 2.1.6 | Base Stations using AAS and beamforming with integrated antennas operating below 6 GHz (Note XX) | 9 kHz f 1 GHz | -36 dBm(Note 4) |
|  |  | 1 GHz < f Fupper (see recommends 3) | -30 dBm (Note 4) |
| 2.1.7 | Base Stations using AAS and beamforming with integrated antennas operating above 24.25 GHz (Note XX) | 9 kHz f 1 GHz | -36 dBm(Note 4) (Note XX) |
|  |  | 1 GHz < f 18 GHz | -30 dBm (Note 4) (Note XX) |
|  |  | 18 GHz < f Fupper (see recommends 3) | -20 dBm/10 MHz (other limits apply for specific frequency separations, see Figure 7) (Note 4) |
| 2.1.8 | Terminals operating above 24.25 GHz using AAS and beamforming with integrated antennas | 9 kHz f 1 GHz | -36 dBm(Note 4) |
|  |  | 1 GHz < f 7.25 GHz | -30 dBm(Note 4) |
|  |  | 7.25 GHz < f Fupper (see recommends 3) | -13 dBm/MHz and -10 dBm/100 MHz (Note 4) (Note 5) |
| [Notes to be moved up here after public consultation and before publication] |  |  |  |

Note 1: ƒ is the frequency of the spurious domain emission

Note 2: For systems that use digital modulation and narrow-band high power (1 Watt) analogue modulated systems, the reference bandwidth is specified in section 2 of this annex, while for any other analogue modulation the reference bandwidth specified in recommend 4 is applicable.

Note 3: 	Levels are H-field limit at 10 m distance, measured by shielded loop antenna as specified by CISPR.

Note 4: For Terminals and Base Stations using AAS and beamforming with integrated antennas, the metric for unwanted emission is defined as Total Radiated Power (TRP):  and applies separately for the  transmitting and receiving phases of operation.

,

,.

For Radio systems using AAS and beamforming with integrated antennas the metric for unwanted emission is defined as Total Radiated Power (TRP).

where PD(r,,) is the power density in W/m2 at a distance r of two orthogonal polarizations.

Over-the-air spatial parameters are specified in a Cartesian coordinate system (x, y, z) using spherical coordinates (r,,). is the angle in the x/y plane and it is between the x-axis and the projection of the vector onto the x/y plane and is defined between -180° and +180°.  is the angle between the projection of the vector in the x/y plane and the vector and is defined between -90° and +90.

The TRP measurement procedure for emissions in the spurious domain with large frequency separation are slightly different from that of the wanted signal and unwanted emissions closer to the wanted signal.

For the wanted signal, if the directivity is known, it is enough to measure the peak e.i.r.p. and adjust it with the directivity to obtain TRP. Care must be taken for using this method for unwanted emissions since its directivity can be different from the wanted signal directivity, in particular at large frequency separations from the carrier. If the directivity is not known but it can be verified that the emission is from the antenna array, TRP can be obtained by measuring e.i.r.p. in the cardinal cuts and using pattern multiplication to extrapolate the e.i.r.p. values in other directions. Pattern multiplication exploits the fact that the array factor of a rectangular array can be separable in two factors along the two symmetry planes. If none of the above options are applicable, the final option is to measure the e.i.r.p. on a full-sphere grid with the reference angular step. The measured values can then be integrated to obtain the TRP. All the above options are accurate methods for TRP assessment provided that the mentioned pre-conditions are fulfilled.

For spurious emissions at larger frequency separations from the carrier, it is important to first identify the frequencies with notable emissions for further measurement. This is achieved through a pre-scan which can be performed on a very sparse grid. If no notable emission is identified at a certain frequency during the pre-scan, no further investigation is needed for that frequency. For frequencies which are identified for further measurement, a number of orthogonal cuts (two or three) or a sparse spherical grid can be used. These methods are based on the assumption that spurious emissions at larger frequency separation are not fully correlated and are less directive. For spurious emissions, an upper bound assessment for TRP is enough to ensure the compliance and the exact TRP value is of less importance. Therefore, a systematic correction factor is added to the estimated TRP value in order to ensure an overestimate with a certain confidence. The systematic correction factor is dependent on the chosen grid type and angular step. Orthogonal cut measurements should be measured with the reference angular steps. The sparse full sphere measurement can be performed on angular steps larger than the reference step and smaller than 15 degrees. A larger angular step results in a shorter measurement time while it imposes a larger systematic correction.

For studies, the TRP needs to be reflected in terms of conducted power (P). The link between conducted power and TRP is based on antenna efficiency that could be transformed in terms of additional losses (L). The relation between these terms are P=TRP+L. (L needs to be defined with a positive number)

Note 5: Both limits are simultaneously applicable.

Note 6:  Limits for Wireless Power Transmission for Electric Vehicles systems are under study and may need further consideration.

Note xx : There is currently no information on base stations using AAS and beamforming with integrated antennas operating between 6 and 24.25 GHz. These could be considered in a future revision as necessary.

.

Application of reference bandwidths to digitally modulated and narrow-band high power analogue modulated systems

Narrow-band analogue modulated systems, with output power higher than 1 Watt and operated above 30 MHz, as well as digitally modulated systems are, although generally providing good spectrum efficiency, unable to comply with the above limits for nearby the centre frequencies due to the wideband noise generated by such systems. It is therefore necessary to provide specific steps of reference bandwidth in order to produce suitable transition area for the spectral density.

The specific mask with reference bandwidths is shown in Figure 5 for frequencies between 30 MHz and 1 GHz and in Figure 6 for frequencies above 1 GHz, with frequency limits which are a function of the channel separation or the necessary bandwidth (NB).

As an alternative for mobile services operating in a band assigned for MFCN in particular for bandwidths of 5 MHz and above and defined with specific frequency arrangements and technical conditions (referred to as the “operating band”), the transition points in the reference bandwidth masks below can be set relative to the operating band edges instead of relative to the centre frequency of emission. In this case, the offsets should consider spectrum management aspects and allow for reasonable transmitter implementation, accounting for the largest necessary bandwidth possible in the operating band.

Figure 5: Specific mask for spurious domain emissions with reference bandwidths for mobile services operating between 30 MHz and 1 GHz (see Table 7)

Table 7: Frequency references for Figure 5

| Parameter | Value |
| --- | --- |
| Fa* | 100 kHz or 4 times NB, whichever is the greater |
| Fb* | 500 kHz or 10 times NB, whichever is the greater |
| (*): The frequency limits are defined from the centre frequency of the emission. For measurement purposes, the reference bandwidths given in Figure 5apply to the frequency range extending from the 250% NB point to the first frequency limit indicated, or from Fa to Fb as appropriate. |  |

Figure 6: Specific mask for spurious domain emissions with reference bandwidths for mobile services operating above 1 GHz (see Table 8)

Table 8: Frequency references for Figure 6

| Parameter | Value |
| --- | --- |
| Fa* | 500 kHz or 10 times NB, whichever is the greater |
| Fb* | 1 MHz or 12 times NB, whichever is the greater |
| (*): The frequency limits are defined from the centre frequency of the emission. For measurement purposes, the reference bandwidths given in Figure 6apply to the frequency range extending from the 250% NB point to the first frequency limit indicated, or from Fa to Fb as appropriate. |  |

Base stations in the mobile services using AAS and beamforming operating above 24.25 GHz although generally providing good spectrum efficiency, are unable to comply with the limits in Table 6, reference number 2.1.7 nearby the assigned operating band (with bandwidth WB) due to the wideband noise generated by such systems. It is therefore necessary to provide specific steps of the limits at certain offsets from the operating band in order to produce suitable transition range for the spectral density.

The specific limits are shown as a mask in Figure 7.

Figure 7 : Specific mask for spurious domain emissions with reference bandwidths for Base Stations using AAS and beamforming operating above 24.25 GHz (see Table 9)

Table 9: Frequency references for Figure 7

| Parameter | Value |
| --- | --- |
| Fc* | 1 GHz or 0.5 times WB , whichever is the greater |
| Fd* | 2 GHz or WB , whichever is the greater |
| Fe* | 10 GHz or 4 times WB or, whichever is the greater |
| (*): The frequency limits are defined from the edge of the AAS Base Station transmitter operating band emission. Regardless of the offset Fc, Fd and Fe as derived from the table, the limits do not apply for f ≤ 18 GHz. |  |

Boundary between the out-of-band and spurious domains for multicarrier transmitters

A multicarrier transmitter is intended to operate only within a single contiguous allocation to a service and application.

For base stations and mobile stations in the mobile service with multicarrier transmitters, the transmitter bandwidth is used instead of the necessary bandwidth for determining the boundary between the out-of-band and spurious domains. In the context of multicarrier base stations and mobile stations in the mobile service, the transmitter bandwidth is defined as the width of the frequency band covering the envelope of the transmitted carriers, which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions for all transmitted carriers.

For wide-band transmitters, Note 2 of recommends 1 identifies that Recommendation ITU-R SM.1539 [2] and Appendix 3 of the ITU Radio Regulations [3] give further guidance on the boundary between the out-of-band and spurious domains. However, for multicarrier base stations and mobile stations in the mobile service, operating up to 6 GHz, using the definitions of BU and BN in Annex 1 of Recommendation ITU-R SM.1539, , the upper threshold value for applying the 250% boundary definition should be BU = 10 MHz and the necessary band

In the case of multicarrier PMR transmitters further studies are needed. For the time being the above provisions may apply for these transmitters only on a case-by-case basis.

Definition of BU from Recommendation ITU-R SM.1539: BU is the upper threshold value for BN (necessary bandwidth) above which the frequency separation between the centre frequency and the spurious boundary equals 1.5 * BN + BU.