3GPP TSG-RAN4 Meeting #91	R4-1906358

# Reno, US, 13th – 17th  May, 2019

Agenda item:		6.10.8.1.2

Source:		MediaTek Inc.

Title:		Discussion on requirements for BFD in FR2

Document for:	Discussion

- Introduction

In the last meeting, for CSI-RS based BFD, N=1 will apply if the CSI-RS for BFD is not in a resource set configured with repetition ON. In this paper, it further makes a clarification the N factor for BFD.

- Scaling factor N for CSI-RS BFD in FR2

For BFD, the CSI-RS for BFD will be QCL-ed with the RS in the active TCI state of CORESET configured in the UE active BWP [1]. Therefore, if the RX beam for CSI-RS for BFD can rely on the RS in the active TCI sate, then N=1 applies.

However, there could be one special case that N=1 is not able to apply:

- if the RS in the active TCI state is the same as CSI-RS for BFD, e.g. CSI-RS#1, and

- TCI state for CSI-RS#1 is not given.

In this case, it should allow UE to perform RX beam sweeping on CSI-RS#1, since its TCI information is  missing.

Figure 1. One CSI-RS resource is used for BFD and L1-RSRP at the same time.

Proposal 1: For CSI-RS BFD in FR2, the requirements doesn’t apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for RLM, and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

- Conclusion

In the contribution, we discuss the requirement for BFD in FR2. We have the following observations and proposals:

Proposal 1: For CSI-RS BFD in FR2, the requirements doesn’t apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for RLM, and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

- Reference

[1] TS38.213, NR; Physical layer procedures for control, V15.5.0.