3GPP TSG-RAN WG4 Meeting #91                 	R4-1906364

Reno, Nevada, US, 13th – 17th May 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR PUSCH simulation results

Agenda item:	6.12.2.2.1

Document for:	Discussion

Introduction

In the RAN4#90b meeting, the latest way forward detailing how to contribute the input for NR PUSCH demodulation performance requirements was agreed [1]. A finalized procedure to arrive at performance requirement SNR numbers from the contributions of all companies was furthermore agreed in [2] and the corresponding script is provided in [3].

In this contribution we deliver corrections to some of our previously delivered results for the agreed simulation cases.

Simulation Results

Simulation settings - transform precoding disabled/CP-OFDM

For transform precoding disabled case, the following simulation assumptions were agreed in [1]. The decision to remove the 1+0 DMRS configuration for FR1 was unfortunately only captured in the WF text and not the WF configuration table:

Table 1: Simulation parameters – Transform precoding disabled

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1, 2 | 1, 2 |
| Number of Rx | 2, 4, 8 | 2 |
| Number of layers | 1, 2 | 1, 2 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1, 1+1 | 1, 1+1 |
| symbols length | 14 | 10 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A, type B | type B |
| Frequency domain resource | full RB allocation of the applicable BW | full RB allocation of the applicable BW |
| MCS index | 2, 16, 20 (20 only for 1Tx) | 2, 16, 20 (20 only for 1Tx) |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation condition | MCS 2: TDL-B 100ns, 400 Hz MCS 16: TDL-C 300ns, 100 Hz MCS 20: TDL-A 30ns, 10Hz | MCS 2: TDL-A 30ns, 300Hz MCS 16: TDL-A 30ns, 300Hz MCS 20: TDL-A 30ns, 75Hz |
| SCS and BW | 15kHz: 5MHz, 10MHz, 20MHz 30kHz: 10MHz, 20MHz, 40MHz, 100MHz 60kHz(FR2): 50MHz, 100MHz 120kHz: 50MHz, 100MHz, 200MHz |  |
| PTRS | Not configured | QPSK: Not configured 16 and 64 QAM: (Configured with KPTRS =2, LPTRS =1) and (not configured) |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group | Disabled | Disabled |
| Frequency hopping | Disabled | Disabled |
| Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |

Simulation settings - transform precoding enabled/DFT-s-OFDM

For transform precoding enabled case, the following simulation assumptions agreed in [1]. The decision to remove the 1+0 DMRS configuration for FR1 was unfortunately only captured in the WF text and not the WF configuration table:

Table 2: Simulation parameters – Transform precoding enabled

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | enabled | enabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2, 4, 8 | 2 |
| Number of layers | 1 | 1 |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1+1, 1 | 1, 1+1 |
| symbols length | 14 | 10 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A, type B | type B |
| Frequency domain resource | 15kHz: 25 PRB (contig., middle of test BW) 30kHz: 24 PRB (contig., middle of test BW) | 60kHz: 30 PRB (contig., middle of test BW) 120kHz: 30 PRB (contig., middle of test BW) |
| MCS index | 2 | 2 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation condition | MCS 2: TDL-B 100ns, 400 Hz | MCS 2: TDL-A 30ns, 300Hz |
| SCS and BW | 15kHz: 5 MHz 30kHz: 10 MHz | 60kHz: 50 MHz 120kHz: 50 MHz |
| PTRS | Not configured | Not configured |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group | Disabled | Disabled |
| Frequency hopping | Disabled | Disabled |
| Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |

Simulation results PUSCH CP-OFDM 1T2R

Simulation results for PUSCH CP-OFDM 1T2R and with the parameters defined are shown in Table 3.

Table 3: Simulation results for PUSCH CP 1T2R

| PUSCH 1T2R |  |  |  |  |  | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| CP-OFDM | FR1 Mappying type A | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -4,68 | -2,18 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 7,81 | 10,31 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 9,91 | 12,41 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -4,84 | -2,34 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 7,62 | 10,12 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 9,73 | 12,23 |
|  | FR1 Mappying type B | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -4,41 | -1,91 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 7,87 | 10,37 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 9,9 | 12,4 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -4,8 | -2,3 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 7,64 | 10,14 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 9,75 | 12,25 |
|  | FR2 with PTRS | 60kHz, 100MHz | MCS16 | TDLA30-300 | DMRS 1 | 9,06 | 12,16 |
|  |  |  |  |  | DMRS 1+1 | 8,18 | 11,28 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 10,64 | 13,74 |
|  |  |  |  |  | DMRS 1+1 | 10,58 | 13,68 |
|  |  | 120kHz, 100MHz | MCS16 | TDLA30-300 | DMRS 1 | 7,76 | 10,86 |
|  |  |  |  |  | DMRS 1+1 | 7,62 | 10,72 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 10,18 | 13,28 |
|  |  |  |  |  | DMRS 1+1 | 10,3 | 13,4 |
|  | FR2 without PTRS | 60kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -4,87 | -1,77 |
|  |  |  |  |  | DMRS 1+1 | -4,99 | -1,89 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 8,47 | 11,57 |
|  |  |  |  |  | DMRS 1+1 | 7,69 | 10,79 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 10,23 | 13,33 |
|  |  |  |  |  | DMRS 1+1 | 10,12 | 13,22 |
|  |  | 120kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -5,45 | -2,35 |
|  |  |  |  |  | DMRS 1+1 | -5,39 | -2,29 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 7,27 | 10,37 |
|  |  |  |  |  | DMRS 1+1 | 7,13 | 10,23 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 9,75 | 12,85 |
|  |  |  |  |  | DMRS 1+1 | 9,85 | 12,95 |
| CP-OFDM | FR1 Mappying type A | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -4,4 | -1,9 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 7,84 | 10,34 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 9,75 | 12,25 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -3,91 | -1,41 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 7,81 | 10,31 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 9,66 | 12,16 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -4,87 | -2,37 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 7,72 | 10,22 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 9,89 | 12,39 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -4,97 | -2,47 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 7,71 | 10,21 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 9,66 | 12,16 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -4,89 | -2,39 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 7,6 | 10,1 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 10,56 | 13,06 |
|  | FR1 Mappying type B | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -4,1 | -1,6 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 7,89 | 10,39 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 9,74 | 12,24 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -3,77 | -1,27 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 7,91 | 10,41 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 9,66 | 12,16 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -4,79 | -2,29 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 7,71 | 10,21 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 9,89 | 12,39 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -4,92 | -2,42 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 7,7 | 10,2 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 9,37 | 11,87 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -4,82 | -2,32 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 7,61 | 10,11 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 10,56 | 13,06 |
|  | FR2 with PTRS | 60kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | 8,92 | 12,02 |
|  |  |  |  |  | DMRS 1+1 | 8,12 | 11,22 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 10,27 | 13,37 |
|  |  |  |  |  | DMRS 1+1 | 10,28 | 13,38 |
|  |  | 120kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | 7,8 | 10,9 |
|  |  |  |  |  | DMRS 1+1 | 7,43 | 10,53 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 10,28 | 13,38 |
|  |  |  |  |  | DMRS 1+1 | 10,24 | 13,34 |
|  |  | 120kHz, 200MHz | MCS16 | TDLA30-300 | DMRS 1 | 7,99 | 11,09 |
|  |  |  |  |  | DMRS 1+1 | 7,75 | 10,85 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 11,08 | 14,18 |
|  |  |  |  |  | DMRS 1+1 | 10,42 | 13,52 |
|  | FR2 without PTRS | 60kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -4,68 | -1,58 |
|  |  |  |  |  | DMRS 1+1 | -4,69 | -1,59 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 8,36 | 11,46 |
|  |  |  |  |  | DMRS 1+1 | 7,66 | 10,76 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 9,85 | 12,95 |
|  |  |  |  |  | DMRS 1+1 | 9,87 | 12,97 |
|  |  | 120kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -5,02 | -1,92 |
|  |  |  |  |  | DMRS 1+1 | -5,07 | -1,97 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 7,31 | 10,41 |
|  |  |  |  |  | DMRS 1+1 | 6,96 | 10,06 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 9,89 | 12,99 |
|  |  |  |  |  | DMRS 1+1 | 9,81 | 12,91 |
|  |  | 120kHz, 200MHz | MCS2 | TDLA30-300 | DMRS 1 | -5,33 | -2,23 |
|  |  |  |  |  | DMRS 1+1 | -5,37 | -2,27 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 7,49 | 10,59 |
|  |  |  |  |  | DMRS 1+1 | 7,25 | 10,35 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 9,97 | 13,07 |
|  |  |  |  |  | DMRS 1+1 | 9,98 | 13,08 |

Simulation results PUSCH CP-OFDM 1T4R

Simulation results for PUSCH CP-OFDM 1T4R and with the parameters defined are shown in Table 4.

Table 4: Simulation results for PUSCH CP 1T4R

| PUSCH 1T4R |  |  |  |  |  | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| CP-OFDM | FR1 Mappying type A | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -7,91 | -5,41 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 3,9 | 6,4 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 6,07 | 8,57 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -8,12 | -5,62 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 3,7 | 6,2 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 6,09 | 8,59 |
|  | FR1 Mappying type B | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -7,74 | -5,24 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 3,92 | 6,42 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 6,09 | 8,59 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -8,09 | -5,59 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 3,7 | 6,2 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 6,09 | 8,59 |
| CP-OFDM | FR1 Mappying type A | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -7,76 | -5,26 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 3,74 | 6,24 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 6,02 | 8,52 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -7,39 | -4,89 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 3,78 | 6,28 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 6,05 | 8,55 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -8,11 | -5,61 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 3,83 | 6,33 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 6,05 | 8,55 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -8,19 | -5,69 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 3,77 | 6,27 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 6,02 | 8,52 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -8,12 | -5,62 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 3,7 | 6,2 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 6,74 | 9,24 |
|  | FR1 Mappying type B | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -7,57 | -5,07 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 3,72 | 6,22 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 5,99 | 8,49 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -7,23 | -4,73 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 3,82 | 6,32 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 6,06 | 8,56 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -8,05 | -5,55 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 3,86 | 6,36 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 6,05 | 8,55 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -8,15 | -5,65 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 3,78 | 6,28 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 6,01 | 8,51 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -8,06 | -5,56 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 3,7 | 6,2 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 6,72 | 9,22 |

Simulation results PUSCH CP-OFDM 1T8R

Simulation results for PUSCH CP-OFDM 1T8R and with the parameters defined are shown in Table 5.

Table 5: Simulation results for PUSCH CP 1T8R

| PUSCH 1T8R |  |  |  |  |  | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| CP-OFDM | FR1 Mappying type A | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -10,77 | -8,27 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 0,46 | 2,96 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 3,01 | 5,51 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -10,91 | -8,41 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 0,33 | 2,83 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 2,89 | 5,39 |
|  | FR1 Mappying type B | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -10,61 | -8,11 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 0,46 | 2,96 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 3,01 | 5,51 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -10,9 | -8,4 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 0,34 | 2,84 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 2,89 | 5,39 |
| CP-OFDM | FR1 Mappying type A | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -10,73 | -8,23 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 0,33 | 2,83 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 2,93 | 5,43 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -10,42 | -7,92 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 0,32 | 2,82 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 3,01 | 5,51 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -10,97 | -8,47 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 0,51 | 3,01 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 3,01 | 5,51 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -11,04 | -8,54 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 0,43 | 2,93 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 2,91 | 5,41 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -10,9 | -8,4 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 0,36 | 2,86 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 3,46 | 5,96 |
|  | FR1 Mappying type B | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -10,52 | -8,02 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 0,36 | 2,86 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 2,92 | 5,42 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -10,28 | -7,78 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 0,35 | 2,85 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 2,88 | 5,38 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -10,93 | -8,43 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 0,52 | 3,02 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 3 | 5,5 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -11,01 | -8,51 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 0,44 | 2,94 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 2,92 | 5,42 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -10,85 | -8,35 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 0,38 | 2,88 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1+1 | 3,47 | 5,97 |

Simulation results PUSCH CP-OFDM 2T2R

Simulation results for PUSCH CP-OFDM 2T2R and with the parameters defined are shown in Table 6.

Table 6: Simulation results for PUSCH CP 2T2R

| PUSCH 2T2R |  |  |  |  |  | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| CP-OFDM | FR1 Mappying type A | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,09 | 2,41 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,94 | 19,44 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -1,04 | 1,46 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 15,76 | 18,26 |
|  | FR1 Mappying type B | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 0,22 | 2,72 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 17,74 | 20,24 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,97 | 1,53 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 15,89 | 18,39 |
|  | FR2 with PTRS | 60kHz, 100MHz | MCS16 | TDLA30-300 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | 16,68 | 19,78 |
|  |  | 120kHz, 100MHz | MCS16 | TDLA30-300 | DMRS 1 | 16,72 | 19,82 |
|  |  |  |  |  | DMRS 1+1 | 15,43 | 18,53 |
|  | FR2 without PTRS | 60kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -1,16 | 1,94 |
|  |  |  |  |  | DMRS 1+1 | -1,22 | 1,88 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | 15,8 | 18,9 |
|  |  | 120kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -1,56 | 1,54 |
|  |  |  |  |  | DMRS 1+1 | -1,62 | 1,48 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 15,8 | 18,9 |
|  |  |  |  |  | DMRS 1+1 | 14,75 | 17,85 |
| CP-OFDM | FR1 Mappying type A | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,73 | 1,77 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,27 | 18,77 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 0,3 | 2,8 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,79 | 19,29 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -1,33 | 1,17 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16 | 18,5 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,99 | 1,51 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 15,84 | 18,34 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,94 | 1,56 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,36 | 18,86 |
|  | FR1 Mappying type B | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,32 | 2,18 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,59 | 19,09 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 1,08 | 3,58 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 18,46 | 20,96 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -1,26 | 1,24 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,09 | 18,59 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,91 | 1,59 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 15,93 | 18,43 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,85 | 1,65 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,61 | 19,11 |
|  | FR2 with PTRS | 60kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | 16,55 | 19,65 |
|  |  | 120kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | 16,65 | 19,75 |
|  |  |  |  |  | DMRS 1+1 | 15,42 | 18,52 |
|  |  | 120kHz, 200MHz | MCS16 | TDLA30-300 | DMRS 1 | 16,83 | 19,93 |
|  |  |  |  |  | DMRS 1+1 | 15,47 | 18,57 |
|  | FR2 without PTRS | 60kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -1,05 | 2,05 |
|  |  |  |  |  | DMRS 1+1 | -1,16 | 1,94 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | 15,74 | 18,84 |
|  |  | 120kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -1,69 | 1,41 |
|  |  |  |  |  | DMRS 1+1 | -1,51 | 1,59 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 15,76 | 18,86 |
|  |  |  |  |  | DMRS 1+1 | 14,77 | 17,87 |
|  |  | 120kHz, 200MHz | MCS2 | TDLA30-300 | DMRS 1 | -1,6 | 1,5 |
|  |  |  |  |  | DMRS 1+1 | -1,57 | 1,53 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 15,83 | 18,93 |
|  |  |  |  |  | DMRS 1+1 | 14,76 | 17,86 |

Simulation results PUSCH DFT-s-OFDM 1T2R

Simulation results for PUSCH DFT-s-OFDM 1T2R and with the parameters defined are shown in Table 7.

Table 7: Simulation results for PUSCH DFT 1T2R

| PUSCH DFT-s-OFDM |  |  |  |  |  | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| DFT-s-OFDM 1T2R | FR1 Mappying type A | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1+1 | -4,72 | -2,22 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1+1 | -5,07 | -2,57 |
|  | FR1 Mappying type B | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1+1 | -4,42 | -1,92 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1+1 | -5,06 | -2,56 |
|  | FR2 without PTRS | 60kHz, 50MHz (30 PRB) | TDLA30-300 | MCS2 | DMRS 1 | -4,62 | -1,52 |
|  |  |  |  |  | DMRS 1+1 | -4,57 | -1,47 |
|  |  | 120kHz, 50MHz (30 PRB) | TDLA30-300 | MCS2 | DMRS 1 | -5,13 | -2,03 |
|  |  |  |  |  | DMRS 1+1 | -5,03 | -1,93 |

Conclusion

In this contribution we have provided updates to our simulation results for NR PUSCH test cases.

- References

- R4-1904717, WF on NR PUSCH demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#90b.

- R4-1904713, WF on requirement SNR derivation procedure, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, RAN4#90b.

- R4-1904714, Excel Script for Deriving NR Performance Requirements, ZTE, RAN4#90b.