3GPP TSG-RAN WG4 Meeting #91                 	R4-1906365

Reno, Nevada, US, 13th – 17th May 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR PUCCH simulation results

Agenda item:	6.12.2.3.1

Document for:	Discussion

Introduction

In the RAN4#90b meeting, the latest way forward detailing how to contribute the input for NR PUCCH demodulation performance requirements was agreed [1]. A finalized procedure to arrive at performance requirement SNR numbers from the contributions of all companies was furthermore agreed in [2] and the corresponding script is provided in [3].

In this contribution we deliver corrections to some of our previously delivered results for the agreed simulation cases.

Simulation Results

Simulation settings

The following simulation assumptions were agreed in [1] and the WFs referenced therein.

Table 1: Simulation parameters

| Parameters |  | PUCCH Format 0 | PUCCH Format 1 | PUCCH Format 2 | PUCCH Format 3 | PUCCH Format 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth/SCS |  | FR1: 15kHz: 5MHz, 10MHz, 20MHz; 30kHz: 10MHz, 20MHz, 40MHz, 100MHz FR2: 60kHz: 50MHz, 100MHz; 120kHz: 50MHz, 100MHz, 200MHz |  |  |  |  |
| Antenna |  | 1T2R, 1T4R and 1T8R for FR1; 1T2R for FR2 |  |  |  |  |
| Propagation condition |  | FR1: TDLC300-100; FR2: TDLA30-300 |  |  |  |  |
| Carrier frequency (GHz) |  | FR1: 4GHz; FR2: 30GHz |  |  |  |  |
| (#Bits, #Symbols, #PRBs) |  | (1, 1, 1) (1, 2, 1) | (2, 14,1) | (4, 1, 4) (22, 2, 9) | (16, 14, 1) (16, 4, 3) | (22, 14, 1) |
| DMRS pattern |  |  |  |  | FR1 and FR2: - Without additional DMRS for all cases - With additional DMRS for cases with the number of OFDM symbols more than 9 |  |
| Modulation |  |  |  | QPSK | QPSK | QPSK |
| initialCyclicShift |  | 0 | 0 |  |  |  |
| StartingSymbolIndex |  | 13 for 1 symbol; 12 for 2 symbols; | 0 | 13 for 1 symbol; 12 for 2 symbols; | 0 | 0 |
| Index of orthogonal sequence (time-domain-OCC) |  |  | 0 |  |  |  |
| occ-Length |  |  |  |  |  | n2 |
| occ-Index |  |  |  |  |  | n0 |
| Test metric | DTX to Ack probability <1% | √ | √ | √ for bits <=11 | × | × |
|  | Missed Ack probability < 1% | √ | √ | √ | × | × |
|  | NACK2ACK < 0.1% | × | √ | x | × | × |
|  | BLER < 1% | × | × | √ for bits >11 | √ | √ |

Simulation results PUCCH FR2 1T2R

Simulation results for PUCCH FR2 1T2R and with the parameters defined are shown in Table 2.

Table 2: Simulation results for PUCCH FR2 1T2R

| PUCCH FR2 1T2R, TDLA30-300 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Note | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| Format 0 | 60kHz, 100MHz | 1 | 1 | 1 |  | 6,40 | 9,50 |
|  |  | 1 | 2 | 1 |  | 1,72 | 4,82 |
|  | 120kHz, 100MHz | 1 | 1 | 1 |  | 6,68 | 9,78 |
|  |  | 1 | 2 | 1 |  | 0,74 | 3,84 |
| Format 1 | 60kHz, 100MHz | 2 | 14 | 1 | ACK miss | -7,10 | -4,00 |
|  |  |  |  |  | NACK to ACK | -5,90 | -2,80 |
|  | 120kHz, 100MHz |  |  |  | ACK miss | -7,00 | -3,90 |
|  |  |  |  |  | NACK to ACK | -5,90 | -2,80 |
| Format 2 | 60kHz, 100MHz | 4 | 1 | 4 | ACK miss | 4,70 | 7,80 |
|  |  | 22 | 2 | 9 | BLER@1% | -1,00 | 2,10 |
|  | 120kHz, 100MHz | 4 | 1 | 4 | ACK miss | 4,30 | 7,40 |
|  |  | 22 | 2 | 9 | BLER@1% | -1,70 | 1,40 |
| Format 3 | 60kHz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -1,90 | 1,20 |
|  |  |  |  |  | With additional DMRS | -1,70 | 1,40 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0,90 | 2,20 |
|  | 120kHz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -2,00 | 1,10 |
|  |  |  |  |  | With additional DMRS | -1,60 | 1,50 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -1,30 | 1,80 |
| Format 4 | 60kHz, 100MHz | 22 | 14 | 1 | Without additional DMRS | -0,50 | 2,60 |
|  |  |  |  |  | With additional DMRS | -0,10 | 3,00 |
|  | 120kHz, 100MHz |  |  |  | Without additional DMRS | -0,40 | 2,70 |
|  |  |  |  |  | With additional DMRS | 0,10 | 3,20 |
| Format 0 | 60kHz, 50MHz | 1 | 1 | 1 |  | 6,29 | 9,39 |
|  |  | 1 | 2 | 1 |  | 2,18 | 5,28 |
|  | 120kHz, 50MHz | 1 | 1 | 1 |  | 6,57 | 9,67 |
|  |  | 1 | 2 | 1 |  | 0,81 | 3,91 |
|  | 120kHz, 200MHz | 1 | 1 | 1 |  | 6,61 | 9,71 |
|  |  | 1 | 2 | 1 |  | 0,56 | 3,66 |
| Format 1 | 60kHz, 50MHz | 2 | 14 | 1 | ACK miss | -6,60 | -3,50 |
|  |  |  |  |  | NACK to ACK | -5,30 | -2,20 |
|  | 120kHz, 50MHz |  |  |  | ACK miss | -7,50 | -4,40 |
|  |  |  |  |  | NACK to ACK | -6,20 | -3,10 |
|  | 120kHz, 200MHz |  |  |  | ACK miss | -7,60 | -4,50 |
|  |  |  |  |  | NACK to ACK | -6,30 | -3,20 |
| Format 2 | 60kHz, 50MHz | 4 | 1 | 4 | ACK miss | 4,7 | 7,8 |
|  |  | 22 | 2 | 9 | BLER@1% | 0,3 | 3,4 |
|  | 120kHz, 50MHz | 4 | 1 | 4 | ACK miss | 4,1 | 7,2 |
|  |  | 22 | 2 | 9 | BLER@1% | -0,9 | 2,2 |
|  | 120kHz, 200MHz | 4 | 1 | 4 | ACK miss | 4,3 | 7,4 |
|  |  | 22 | 2 | 9 | BLER@1% | -1,7 | 1,4 |
| Format 3 | 60kHz, 50MHz | 16 | 14 | 1 | Without additional DMRS | -1,50 | 1,60 |
|  |  |  |  |  | With additional DMRS | -0,90 | 2,20 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0,30 | 2,80 |
|  | 120kHz, 50MHz | 16 | 14 | 1 | Without additional DMRS | -2,40 | 0,70 |
|  |  |  |  |  | With additional DMRS | -2,10 | 1,00 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -1,60 | 1,50 |
|  | 120kHz, 200MHz | 16 | 14 | 1 | Without additional DMRS | -2,70 | 0,40 |
|  |  |  |  |  | With additional DMRS | -2,30 | 0,80 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -1,70 | 1,40 |
| Format 4 | 60kHz, 50MHz | 22 | 14 | 1 | Without additional DMRS | 0,10 | 3,20 |
|  |  |  |  |  | With additional DMRS | 0,60 | 3,70 |
|  | 120kHz, 50MHz |  |  |  | Without additional DMRS | -0,90 | 2,20 |
|  |  |  |  |  | With additional DMRS | -0,40 | 2,70 |
|  | 120kHz, 200MHz |  |  |  | Without additional DMRS | -1,10 | 2,00 |
|  |  |  |  |  | With additional DMRS | -0,80 | 2,30 |

Conclusion

In this contribution we have provided updates to some of our simulation results for NR PUCCH test cases.

- References

- R4-1904718, Way forward on NR PUCCH demodulation performance in Rel-15, Huawei, RAN4#90b.

- R4-1904713, WF on requirement SNR derivation procedure, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, RAN4#90b.

- R4-1904714, Excel Script for Deriving NR Performance Requirements, ZTE, RAN4#90b.